3GPP TSG-RAN WG3 Meeting #128	R3-253580

Malta, MT, 19 – 23 May, 2025

Agenda item:	8.1

Source:	Nokia

Title:	Per-UE UE Performance metrics in Rel-18

Document for:	Decision

# 1	Introduction

RAN3 sent an LS to SA5 in R3-247775- S5-251324 in which RAN3 asked SA5 whether the Average UE Throughput DL and Average UE Throughput UL measurements formulas in fields c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558, refer to measurements that are specified with a per-UE granularity or with a per-DRB granularity as the measurement name in field e) of the same mentioned clauses starts with “DRB”. We provide those clauses from TS 28.558 for the reader’s reference:

##### 6.3.1.4.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9].

b)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI.

If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s]

If ThpTimeDl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one Data Radio Bearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form 
DRB.UEThpDl, or optionally 
DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

##### 6.3.1.4.2	Average UL UE throughput in gNB

a)	This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M5 in TS 37.320[9].

B)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI.

If ThpTimeUl>0, ThpVolUlThpTimeUl×1000 [kbit/s]

If ThpTimeUl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one Data Radio Bearer (DRB) is emptied. |
| --- | --- |
|  | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
|  | The point in time when transmission is started for the first data in data burst for a particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form 
DRB.UEThpUl, or optionally 
DRB.UEThpUl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpUl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	N/A

h)	One usage of this measurement is to support ML training and performance evaluation.

SA5 confirms that the formulas specified in fields (c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 calculate average UE throughput based on per DRB level measurement for a UE. The formulas aggregate data across DRBs for each UE, ensuring that the reported throughput measurement represents the overall throughput per UE rather than being limited to individual DRBs.

The rationale for using the "DRB." prefix in the measurement names, as listed in field (e), is to indicate that the measurements originate from data transmissions occurring at the DRB level. However, these measurements are aggregated to provide a per-UE throughput metric. Specifically:

- The formulas include a summation over DRBs, ensuring that the result represents the total throughput per UE rather than individual DRB-level values.

- The "DRB." prefix in field (e) indicates that the contributing measurements originate from DRBs, but they are aggregated to reflect per-UE throughput.

- The definitions in the tables in field (c) in clauses 6.3.1.4.1 and 6.3.1.4.2 are based on DRB level measurements, but the summation ensures that the final metric is at the UE level.

In summary, although the measurement inputs are taken per DRB, they are aggregated to provide throughput metrics at the UE level.

In this meeting, RAN3 received a reply LS from SA5 in R3-253023 -S5-252056 which SA5 stated the following:

In this paper, we discuss a way forward for Rel-18 UE Performance metric calculation.

# 2	Discussion

In TS 38.423 UE Performance IE is provided without semantics description in the tabular:

One reason behind that is that it has been unclear how to calculate UE Performance metrics per UE granularity.

However, from the reply LS from SA5, Average UE Throughput UL and Average UE Throughput DL measurements are specified in TS 28.558 per UE granularity.

Observation 1: In the reply LS from SA5, SA5 confirmed that the throughput measurements in TS 28.558 described in clauses 6.3.1.4.1 and 6.3.1.4.2 are measured per UE.

Based on the above observation, RAN3 can update the Stage 3 semantics tabular of the UE Performance IE in TS 38.423 to point to the corresponding clauses from TS 28.558.

Proposal 1: Update the Stage 3 semantics tabular of Average UE Throughput DL and Average UE Throughput UL in UE Performance IE in TS 38.423 with references to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 respectively.

For the other two metrics of Average Packet Delay and Average Packet Loss DL there do not exist specified measurements per UE in TS 28.558. For this reason, RAN3 captured a common understanding that they can be calculated as an average value of per DRB level measurements of the UE:

UE level performance is derived as the average value of per DRB level measurements of the UE. ****

In our view, SA5 is the group responsible to define measurements in order for those measurements to have a clear interpretation and support interoperability. We therefore think that the definition of new measurements should be left to SA5.

Proposal 2: Send an LS to SA5 asking SA5 to define Average Packet Delay UL and DL measurements per UE.

We provide a draft reply LS to SA5 in R3-253582.

For the Average Packet Loss, RAN3 sent an LS to SA5 in the previous meeting (R3-252492s) to seek for clarification whether Packet Loss Rate DL and UL is measured in PDCP layer in split and non-split architecture. Depending on SA5’s response to the LS, RAN3 may decide on a different way forward, including introducing new measurements at PDCP layer in the UL and DL to describe Packet Loss Rate or to delete the measurement. Therefore, RAN3 should wait for the response from SA5 on Rel-19 Packet Loss Rate measurement before updating the Rel-18 Packet Loss Rate.

Proposal 3: RAN3 should wait for the response from SA5 on Rel-19 Packet Loss Rate measurement before updating the Rel-18 Packet Loss Rate.

# 3	Conclusion

In this paper we make the following observations and proposals:

Observation 1: In the reply LS from SA5, SA5 confirmed that the throughput measurements in TS 28.558 described in clauses 6.3.1.4.1 and 6.3.1.4.2 are measured per UE.

Proposal 1: Update the Stage 3 semantics tabular of Average UE Throughput DL and Average UE Throughput UL in UE Performance IE in TS 38.423 with references to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 respectively.

Proposal 2: Send an LS to SA5 asking SA5 to define Average Packet Delay UL and DL measurements per UE.

Proposal 3: RAN3 should wait for the response from SA5 on Rel-19 Packet Loss Rate measurement before updating the Rel-18 Packet Loss Rate.