3GPP TSG-RAN WG3 Meeting #128	R3-253593

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	20.2

Source:	Nokia

Title:	(TP to TS 38.305, TP to TS 38.455) Model Training at gNB for case 3a

Document for:	Decision

# 1	Introduction

For case 3a, RAN3 has made the following agreements relevant to model training at the gNB:

The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

There exist several open aspects that we would like to address in this meeting.

FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

In this contribution, we provide our views with respect to case 3a, focusing on “opportunistic” data collection since for proactive we are waiting for a Reply LS from SA2.

# 2	Discussion

## 2.1	General

In RAN3#127, we discussed whether Part A is internal information to the gNB. According to current RAN1 agreements, Part A comprises channel measurement, a quality indicator of the channel measurement and timestamp information. Part A is used together with Part B which comprises a ground truth label or its approximation, a quality indicator of the label and a timestamp. Together, Part A and Part B are used to enable a gNB to train its own AI/ML Model. In our understanding, the channel measurements of Part A may, at least in part, be implementation specific and therefore cannot be specified over any network interfaces. Based on this understanding, we propose to confirm the WA, after making some clarification in the wording.

Proposal 1:	Confirm the following working assumption: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface.

## 2.2	User consent check

During the discussions of case 3a, user consent was brought up by several companies. User consent is part of user subscription information and is therefore available at the Unified Data Management (UDM). In Rel-19, SA2 introduced support for LMF to check user consent for direct AI/ML positioning methods.

In TS 23.273, SA2 has defined procedures for data collection at LMF for LMF-based AI/ML positioning based on UE measurements and based on NG-RAN measurements. If LMF determines that data collection from the NG-RAN is required, then before any data is received from the NG-RAN it needs to obtain user consent for data collection (including a specific purpose) from UDM (TS 23.502). This is done by the LMF by checking that user consent is provided for each SUPI for which data is collected. LMF additionally may subscribe to changes on subscription data related to user consent so that if user consent is revoked the data collection for this SUPI will be stopped. Figure 1 shows an example flow for data collection including user consent check from clause 6.22.3, TS 23.273 (steps 4, 5 and 9):

Figure 1 Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements (Figure 6.22.3-1, TS 23.273)

The SA2 user consent framework did not consider AI/ML assisted positioning with gNB-side AI/ML model, namely case 3a. However, the main assumption by SA2 is that user consent is checked before data is collected for AI/ML Model Training from the NG-RAN (or from a given UE). We therefore think that some aspects of the SA2 user consent framework can be reused for Opportunistic Data Collection.

In the BL CR to TS 38.305 the LMF starts positioning measurements for a given UE as shown in Figure 2:

Figure 2 Positioning Data Collection Procedure (Figure 7.x.2-1, BL CR to TS 38.305)

In step 2, a gNB determines that Data Collection is needed for the UE being measured and indicates to the LMF that it needs positioning data for this UE. User consent must be checked before step 4, when LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message that includes positioning data (e.g., UE location) to the gNB. In that sense, we think that the user consent framework from SA2 can be reused for case 3a.

Proposal 2:	User consent is checked by LMF before LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message, and SA2 framework can be reused.

Based on this proposal, we provide a Stage 2 TP to TS 38.305 in Annex A of our paper that introduces the step of checking user consent by the LMF.

## 2.3	Issues related to AI/ML flag

According to legacy positioning methods, an LMF requests measurements from a gNB and uses those measurements for the calculation of a UE positioning estimate. The LMF’s positioning algorithm does not depend on the method that a gNB used to determine those measurements and the reporting procedures are method agnostic. At the same time, it is up to gNB implementation whether and how to perform those measurements. If a gNB uses AI/ML methods to obtain those measurements one may question if those measurements meet certain accuracy requirements. Based on earlier RAN3 agreement, an NG-RAN node performs monitoring metric calculation for its own model. This means that a gNB is capable to perform Model Performance monitoring of its own AI/ML Model, by using implementation-specific, and dependent on its AI/ML Model, Model Performance monitoring metrics.

Observation 1: A gNB performs Model Performance monitoring of its own AI/ML Model.

This further implies that it can determine by implementation whether the performance of its own AI/ML Model is good or not and subsequently take corrective actions, such as retrain its AI/ML Model, use legacy methods to derive the measurements, etc. Therefore, a gNB by its own implementation, will use its best available method to determine the timing information.

Observation 2: For case 3a, the gNB, by its implementation, will use its best available method to determine the timing information measurements requested by the LMF.

Looking at the TRP Measurement Result IE in TS 38.455, we observe that the timing information that a gNB can provide through AI/ML corresponds to measurements that can be reported already by a gNB to the LMF. From LMF point of view, there is no difference between measurements that are derived by AI/ML Model Inference or by legacy measurements, measured by the gNB/TRP. Thus, it should be transparent to the LMF what methods (AI/ML or legacy) the gNB uses to provide the requested measurements to the LMF. This means that a gNB should respond to the LMF request by choosing the best measurement it has available by its implementation, either by deploying AI/ML or legacy procedures.

Observation 3: LMF behavior is no different if the measurements are derived by AI/ML Model Inference or by legacy measurements, measured by the gNB/TRP.

Therefore, in our view, there is no need for LMF to be aware that the reported measurements are AI/ML generated and similarly the gNB should not expose implementation specific information regarding the algorithms it has available and in use at a given time. Also, introducing LMF awareness about AI/ML generated measurements can be harmful, because it creates doubt about the trustworthiness of the measurements.

In addition, it creates potential interoperability issues if there is an expectation that the LMF takes precautionary actions when receiving AI/ML generated measurements. Such a flag creates ambiguity with respect to which entity is responsible for monitoring performance since a gNB may assume that by flagging measurements obtained through AI/ML to LMF it becomes LMF’s responsibility to determine the quality of the measurement. Thus, if such a flag is added, besides a stage 2 description of its functionality, we will need to also specify how it affects LMF behavior and to provide stage 3 description how this information is used from receiver perspective. As another consequence, it will create precedence in the AI/ML discussions to assume that whenever AI/ML is used to produce a measurement the receiver must become aware that AI/ML is involved. This is different from the assumptions so far in the AI/ML discussions where a reporting node indicates if an output is produced by AI/ML but only when this is indicated in the request.

Proposal 3:	The gNB decides whether the measurements reported to LMF are outcomes of legacy measurements or produced by AI/ML operations and no flag is needed to indicate measurements derived by AI/ML operations.

## 2.2	“Opportunistic” data collection

As agreed at RAN3#127bis, the gNB decides whether positioning data collection is needed for the UE being measured, and if so, it indicates this in the MEASUREMENT RESPONSE message. If the LMF receives an indication from the gNB that positioning data collection is needed for the UE being measured, it provides the positioning data (i.e., Part B information) to the gNB using a new non-UE associated Class 2 NRPPa procedure called Positioning Data Collection Report. The corresponding message is called the POSITIONING DATA COLLECTION REPORT.

Since all NRPPa procedures are mapped to NRPPa functions, a new NRPPa function should be introduced to support data collection from the LMF.

Proposal 4:	Introduce a new function in NRPPa called Positioning Data Collection Transfer, which allows the gNB to collect positioning data from the LMF.

According to RAN3 agreement, the POSITIONING DATA COLLECTION REPORT message includes information that enables the gNB to correlate the positioning data received from the LMF with information collected locally (i.e., Part A information). The most straightforward solution is for the POSITIONING DATA COLLECTION REPORT message to include the LMF Measurement ID and RAN Measurement ID that was included in the MEASUREMENT RESPONSE message that triggered the report.

Proposal 5:	The POSITIONING DATA COLLECTION REPORT includes an LMF Measurement ID IE and RAN Measurement ID IE to correlate the report with the MEASUREMENT RESPONSE that triggered the report.

If the user consent check fails, the LMF will not be able to provide the positioning data needed by the gNB. In such cases, it is still important for the LMF to send a message to the gNB to indicate that the positioning data is not being provided. There are several options for this:

Option A:	Introducing a failure message, e.g. POSITIONING DATA COLLECTION FAILURE.

Option B:	Make the positioning data optional in the POSITIONING DATA COLLECTION REPORT, so the data can be omitted when user consent check fails.

It seems unnecessary to introduce a separate procedure, while Option B seems very straightforward.

Proposal 6:	The POSITIONING DATA COLLECTION REPORT includes an optional Positioning Data List IE, which is omitted when the LMF cannot provide any data (e.g., user consent check fails).

In the TS 38.305 BL CR, there are Editor’s Notes to further check the details of the signalling flow. The following clarifications are needed:

-	It is more accurate and generalized to say “UE being measured” rather than “UE being positioning” (e.g., in case of PRU).

-	Since we are not yet sure about the details of Part B information, we should try to use general terminology such as “positioning data” rather than e.g. “information related to UE location”.

-	Similarly, since we are not yet sure about the details of Part A information (which could also comprise implementation-specific information), we should try to use general terminology such as “local data collected by the gNB”.

Proposal 7:	Agree to the TS 38.305 TP in Annex A which addresses the above clarifications.

# 4	Conclusion

In this paper we make the following observations and proposals:

Proposal 1:	Confirm the following working assumption: For case 3a data collection, Part A is known internally by the gNB and is not signalled  over any network interface.

Proposal 2:	User consent is checked by LMF before LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message, and SA2 framework can be reused.

Observation 1: A gNB performs Model Performance monitoring of its own AI/ML Model.

Observation 2: For case 3a, the gNB, by its implementation, will use its best available method to determine the timing information measurements requested by the LMF.

Observation 3: LMF behavior is no different if the measurements are derived by AI/ML Model Inference or by legacy measurements, measured by the gNB/TRP.

Proposal 3:	The gNB decides whether the measurements reported to LMF are outcomes of legacy measurements or produced by AI/ML operations and no flag is needed to indicate measurements derived by AI/ML operations.

Proposal 4:	Introduce a new function in NRPPa called Positioning Data Collection Transfer, which allows the gNB to collect positioning data from the LMF.

Proposal 5:	The POSITIONING DATA COLLECTION REPORT includes an LMF Measurement ID IE and RAN Measurement ID IE to correlate the report with the MEASUREMENT RESPONSE that triggered the report.

Proposal 6:	The POSITIONING DATA COLLECTION REPORT includes an optional Positioning Data List IE, which is omitted when the LMF cannot provide any data (e.g., user consent check fails).

Proposal 7:	Agree to the TS 38.305 TP in Annex A which addresses the above clarifications.

Also, a TS 38.455 TP is provided in Annex B to reflect proposal 4, 5 and 6 above.

# References

- R3-252306 Summary of Offline Discussion on AIPHY, CATT (moderator)

- R3-252500 Introduction of AIML air, CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT

- R3-252501 (BL CR to 38.455) Introduction of AI/ML air, Ericsson

# Annex A:	TP to TS 38.305 BL CR

## <<< start of changes >>>

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures.

2.	The gNB(s) determines that data collection is needed for the UE being measured.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that positioning data collection is needed for the UE being measured.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

4.	User Consent is checked as described in TS 23.273 [35].

NOTE 1:	Step 4 could be performed earlier.

5.	The LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being measured. The message includes the positioning data and information that enables the gNB to correlate the positioning data with local data collected by the gNB which pertains to the same UE.

Editor’s Note: The texts above could be further refined, if needed.

Editor’s Note: FFS on details of the signallings and IEs.

<<< end of changes >>>

# Annex B:	TP to TS 38.455 BL CR

## <<< start of changes >>>

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Positioning Data Collection Transfer. This function allows the gNB to collect positioning data from the LMF.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Positioning Data Collection Transfer | Positioning Data Collection Report |

<<< next change >>>

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

[…]

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Positioning Data Collection Report | POSITIONING DATA COLLECTION REPORT |

<<< next change >>>

#### 9.1.4.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| […] |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Data Collection Needed | O |  | 9.2.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<< next change >>>

### 9.1.x	Messages for Positioning Data Collection Transfer Procedures

#### 9.1.x.1	POSITIONING DATA COLLECTION REPORT

This message is sent by the LMF to report positioning data for a specific UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data List |  | 0..1 |  |  |  |  |
| >Positioning Data Item |  | 1..<maxnoDataItems> |  |  |  |  |
| >>Ground Truth Label [FFS] | M |  | [FFS] |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataItems | Maximum no. of data items that can be reported with one data collection report message. Value is [FFS]. |

<<< next change >>>

## 9.2	Information Element definitions

[…]

### 9.2.x1	Data Collection Needed

This IE indicates the positioning data that is needed by the NG-RAN node for a particular UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

<<< end of changes >>>