3GPP TSG-RAN WG3 Meeting #128	R3-253597

Malta, MT, 19 – 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1570 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of dynamic CLI Mitigation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In partially synchronized or unsynchronized TDD scenarios, cross-link interference (CLI) can significantly degrade uplink performance. The current specification (Release 15-18) primarily relies on static or semi-static TDD alignment or RIM-RS reporting. However, there is no standardized way for a gNB-DU to request immediate interference mitigation from neighboring gNBs in a fully dynamic manner. This CR introduces a mechanism for a victim DU to signal a “CLI Mitigation Request” to its CU, which can then coordinate with other gNBs to enact temporary DL muting, power reduction, or TDD reconfiguration. The new procedure will enhance real-time CLI management, improving UL performance in advanced TDD or SBFD deployments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extend the DU-CU Radio Information Transfer procedure (Clause 8.9.1) to include a “CLI Mitigation Request” indicator. - Allow the DU to signal the CU that cross-link interference is detected and that mitigation is requested. - CU, upon receiving this request, may initiate the new Xn-based procedure with the aggressor gNB. - After neighbor coordination, the CU can inform the DU of the outcome (accepted, partial, or rejected) via the existing CU-DU Radio Information Transfer (Clause 8.9.2). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 8.9.1, 8.9.2, 9.3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----Start Changes----

## 8.9	Radio Information Transfer procedures

### 8.9.1	DU-CU Radio Information Transfer

#### 8.9.1.1	General

The purpose of the DU-CU Radio Information Transfer procedure is to transfer radio-related information from the gNB-DU to the gNB-CU. The procedure uses non-UE-associated signalling.

The DU-CU Radio Information Transfer procedure may also be used by the gNB-DU to request interference mitigation for a serving cell experiencing cross-link interference. In this case, the DU includes a CLI Mitigation Request indication in the radio information message, identifying the interfering cell(s) and the desired mitigation assistance. The gNB-CU, upon receiving this, shall initiate appropriate inter-gNB signaling (e.g. over Xn) towards the identified aggressor gNB(s) to request interference reduction and shall subsequently inform the gNB-DU of the outcome. The DU-CU Radio Information Transfer procedure may also be used by the gNB-DU to request interference mitigation for a serving cell experiencing cross-link interference. In this case, the DU includes a CLI Mitigation Request indication in the radio information message, identifying the interfering cell(s) and the desired mitigation assistance. The gNB-CU, upon receiving this, shall initiate appropriate inter-gNB signaling (e.g. over Xn) towards the identified aggressor gNB(s) to request interference reduction and shall subsequently inform the gNB-DU of the outcome.

#### 8.9.1.2	Successful operation

Figure 8.9.1.2-1: DU-CU Radio Information Transfer procedure.

The gNB-DU initiates the procedure by sending the DU-CU RADIO INFORMATION TRANSFER message to the gNB-CU.

The gNB-CU considers that the RIM-RS Detection Status IE indicates the RIM-RS detection status of the cell identified by Aggressor Cell ID IE.

If the DU-CU Radio Information Transfer is initiated with a CLI Mitigation Request, the gNB-CU shall interpret the included information as a trigger to coordinate cross-link interference mitigation with neighboring gNBs via the CLI Mitigation Request procedure. If the DU-CU Radio Information Transfer is initiated with a CLI Mitigation Request, the gNB-CU shall interpret the included information as a trigger to coordinate cross-link interference mitigation with neighboring gNBs via the CLI Mitigation Request procedure.

#### 8.9.1.3	Abnormal Conditions

Not applicable.

### 8.9.2	CU-DU Radio Information Transfer

#### 8.9.2.1	General

The purpose of the CU-DU Radio Information Transfer procedure is to transfer radio-related information from the gNB-CU to the gNB-DU. The procedure uses non-UE-associated signalling.

The CU-DU Radio Information Transfer procedure may also be used by the gNB-CU to inform the gNB-DU of interference mitigation actions taken by a neighbor gNB. For example, the gNB-CU can signal that a particular RIM-RS has disappeared or that a certain interference will cease for a specified duration, enabling the DU to adjust its uplink scheduling accordingly. The CU-DU Radio Information Transfer procedure may also be used by the gNB-CU to inform the gNB-DU of interference mitigation actions taken by a neighbor gNB. For example, the gNB-CU can signal that a particular RIM-RS has disappeared or that a certain interference will cease for a specified duration, enabling the DU to adjust its uplink scheduling accordingly.

#### 8.9.2.2	Successful operation

Figure 8.9.2.2-1: CU-DU Radio Information Transfer procedure.

The gNB-CU initiates the procedure by sending the CU-DU RADIO INFORMATION TRANSFER message to the gNB-DU. The gNB-DU considers that the RIM-RS Detection Status IE indicates the detection status of RIM-RS associated with Victim gNB Set ID IE.

#### 8.9.2.3	Abnormal Conditions

Not applicable.

## 9.3	Information Element Definitions

#### 9.3.1.x DU-CU RIM Information

| IE / Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| DU-CU RIM Information | M | — | DUCURIMInformation (SEQUENCE) | This IE is used to convey Radio Information Management (RIM) status from the gNB-DU to the gNB-CU, and optionally to request Cross-Link Interference (CLI) mitigation. If a CLI Mitigation Request is included, the gNB-CU may coordinate over the Xn interface. |
| >victimGNBSetID | M | — | GNBSetID | Identifies the gNB Set ID of the victim (i.e., the DU’s gNB set). |
| >rIMRSDetectionStatus | M | — | RIMRSDetectionStatus | Indicates whether a RIM-RS is detected (e.g., “detected,” “notDetected,” or “disappeared”). |
| >aggressorCellList | O | 1..maxCell | AggressorCellList | Lists one or more neighboring cells identified as potential aggressors. |
| >>AggressorCellListItem | (repeated) | 1..maxCell | SEQUENCE | Each item identifies a specific aggressor cell and optionally includes interference severity. |
| >>>aggressorCellID | M | — | NR-CGI | Identifies the neighbor cell (NR-CGI) that is causing interference. |
| >>>interferenceLevel | O | 0..63 | INTEGER (0..63) | Indicates severity of the interference from this aggressor cell (e.g., CLI-RSSI level). A higher value corresponds to stronger interference. |
| >requestMitigation | O | N/A | NULL | If present, indicates that the DU is requesting CLI mitigation for one or more cells in aggressorCellList. The gNB-CU uses this to initiate CLI Mitigation Request(s) over Xn. |
| >iE-Extensions | O | — | ProtocolExtensionContainer {{DUCURIMInformation-ExtIEs}} | Container for future extensions. |