3GPP TSG-RAN WG3 Meeting #128	R3-253688

Malta, MT, May 19th – 23th, 2025

Title: 	(TP to BL CR to TS 36.300 and TS 36.413) Introduce indicator for S1 Suspend/Resume

Source: 	CMCC

Agenda item: 	15.2

Document Type:	Others

- Introduction

In RAN3#127bis meeting, it has been to introduce an indicator when RAN CONFIGURATION UPDATE over NGAP, for DL transmission suspend/ resume as in [1].	R3-253093 and [2].R3-253095.

To align the agreement for NR NTN, in IoT NTN, we propose to introduce it over S1AP.

- TP to BL CR to TS 36.300

<< Start of the change>>

23.21.5	Switchover

23.21.5.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption and or S1 link interruption may occur during the transition between the feeder links.

23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover between two eNBs is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs' context between the two eNBs at feeder link switchover is performed by means of either S1 based or X2 based handover or TNL functionality, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure) may be used.

For regenerative payload, an indication to suspend and resume data and signaling procedures over S1 interface may be sent to MME from eNB in the eNB Configuration Update procedure when the hard feeder link switch over occurs.

<<end of change>>

- TP to BL CR to TS 36.413

<< Start of the change>>

8.7.4	eNB Configuration Update

8.7.4.1	General

The purpose of the eNB Configuration Update procedure is to update application level configuration data needed for the eNB and the MME to interoperate correctly on the S1 interface. This procedure does not affect existing UE-related contexts, if any.

8.7.4.2	Successful Operation

Figure 8.7.4.2-1: ENB Configuration Update procedure: Successful Operation.

The eNB initiates the procedure by sending an ENB CONFIGURATION UPDATE message to the MME including an appropriate set of updated configuration data that it has just taken into operational use. The MME responds with ENB CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If information element(s) is/are not included in the ENB CONFIGURATION UPDATE message, the MME shall interpret that the corresponding configuration data is/are not changed and shall continue to operate the S1 with the existing related configuration data.

If the supported TA(s) is/are to be updated, the whole list of supported TAs, including those that are not to be updated, shall be included in the Supported TAs IE. The MME shall overwrite the whole list of TAs.

If the supported CSG ID(s) is/are to be updated, the whole list of supported CSG IDs, including those that are not to be updated, shall be included in the CSG Id List IE. The MME shall overwrite the whole list of CSG Ids.

If the ENB CONFIGURATION UPDATE message contains the eNB Name IE, the MME may use this IE as a human readable name of the eNB.

If the Default Paging DRX IE is included, the MME shall overwrite any previously stored default paging DRX value for the eNB.

If the NB-IoT Default Paging DRX IE is included in the ENB CONFIGURATION UPDATE message, the MME shall overwrite any previously stored NB-IoT default paging DRX value for the eNB.

If the Connected en-gNB to be Added List IE is included in the ENB CONFIGURATION UPDATE message, the MME shall replace, if applicable, any previously received information for the concerned en-gNBs and take it into account as specified in TS 36.300 [14].

If the Connected en-gNB to be Removed List IE is included in the ENB CONFIGURATION UPDATE message, the MME shall remove any stored information for the concerned en-gNBs.

The updated configuration data shall be stored in both the eNB and the MME and used for the duration of the TNL association or until any further update is triggered by the eNB.

The eNB may initiate a further eNB Configuration Update procedure only after a previous eNB Configuration Update procedure has been completed.

If the Downlink S1 Transmission Control IE is included in the ENB CONFIGURATION UPDATE message, the MME shall, if supported, suspend or resume the downlink S1 transmission accordingly.

<<next change>>

#### 9.1.8.7	ENB CONFIGURATION UPDATE

This message is sent by the eNB to transfer updated information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| eNB Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Supported TAs |  | 0..<maxnoofTACs> |  | Supported TAs in the eNB. | GLOBAL | reject |
| >TAC | M |  | 9.2.3.7 | Broadcast TAC. | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >RAT-Type | O |  | 9.2.1.117 | RAT Type associated with the TAC of the indicated PLMN(s). | YES | reject |
| CSG Id List |  | 0..1 |  |  | GLOBAL | reject |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Default Paging DRX | O |  | 9.2.1.16 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.2.1.114 |  | YES | ignore |
| Connected en-gNB To Be Added List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list of the added en-gNB. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 |  | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| Connected en-gNB To Be Removed List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list of the en-gNB to be removed. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 |  | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| Downlink S1 Transmission Control | O |  | ENUMERATED (suspend, resume, …) | Indicates the MME to suspend or resume the downlink S1 Transmission | YES | ignore |

- Reference

- R3-253093 (BL CR to TS 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN, Ericsson

- R3-253095 (BL CR to TS 38.413) Support for Regenerative Payload and MBS broadcast in NR NTN, CATT

- TS 36.413, S1AP stage3

- TS 36.300, E-UTRAN stage2

- R3-253133 Stage 2 BL CR for IoT NTN, ZTE

- RAN3_127bis_agenda_20250411_1400_EOM, chairnote

- RAN3_127_agenda_20250221_1300_EOM, chairnote