3GPP TSG-RAN WG3 Meeting #128                                           R3-253691

Malta, MT, 19th-23th May 2025

Agenda item:	10.3.2

Source:	CMCC

Title:	Discussion on SON/MDT for network slicing

Document for:	Discussion

- Introduction

In last meeting [1], RAN3 agreed to only focus on slice-based cell re-selection and the following open issues in R19 SON/MDT for network slicing.

1: Reporting of Rejected Slice to RAN Description: Reporting information about slices requested by the UE but rejected by the network to the RAN side. This could be valuable for the network to understand slice demand and for network planning. 2: Conditional Reporting of Immediate MDT (filter MDT data based on Slice usage level) Description: Whether there is a need to support conditional reporting of immediate MDT. Specific conditions might include being based on slice-level or node-level utilization, or based on the selected UE having one or more active PDU sessions on the given slice(s). Only focus on Slice-based Cell Reselection and the above open issue in R19 SONMDT Slicing.

This contribution will focus on open issue 1 (Reporting of Rejected Slice to RAN).

- Discussion

In last meeting, we raised a case that UE can report the rejected slice to the RAN in a certain area which is beneficial for the operator to have better observation and perform optimization in those areas to find mismatch between the coverage of the cells in which a slice is supported and the geographical areas where the users actually desired to access the services provided by a slice.

During the registration procedure, UE sends the requested NSSAIs to AMF which are usually all NSSAIs subscribed by UE, and then receives an allowed NSSAI list and a rejected NSSAI list from AMF. Both allowed list and rejected list are sub-set of the requested NSSAI list. In current specification, only the slices within the allowed NSSAI list are allowed to be initiated by the UE in the registration area.

However, it is possible that some rejected slices will be initiated in Application (i.e. APP layer) by the user. In this case, even if the specific slice is not allowed in the registration area, i.e., the specific slice is not within the allowed NSSAI list, the user may still try to launch the APP choosing this slice. Such request of APP layer will be rejected by UE NAS layer after checking the requested NSSAI is not within the allowed NSSAI list. Such kind of rejection is done inside the UE NAS layer and AS layer cannot be aware of the rejection, so the network side has no idea how often such rejection happens.

It is found that such rejection information is valuable for network side to assess if there is a strong demand to deploy a specific slice. If the operator observes a great number of UE trying to launch a slice which is not supported in this area, the operator will deploy this slice in the near future to meet the customers’ requirements.

Figure 1. Example for reporting the failure of slice initiation inside the UE

In last meeting, some companies proposed that CN has the rejected information and there is no need to send it to RAN. However, we don’t think they are the same thing between the rejected slices known by CN and within UE side. In general, the rejected slices within UE side will be a subset of which are known by CN. Furthermore, considering that the UE usually requests all subscribed slices to the CN during registration procedure, but it doesn't necessarily mean that UE will actually trigger these slicing services. If the operator re-plans slice deployment based on the rejected slices in CN, it may need to deploy all slices requested by the UE to meet the requirements. Therefore, the rejected slice inside the UE is more valuable than within the CN for the operator to assess the requirements and deploy slices in future.

Taking the Figure 1 as an example, the UE requests all subscribed slices {NSSAI#1, NSSAI#2, NSSAI#3, NSSAI#4} to the CN, and receives the allowed NSSAI list {NSSAI#1, NSSAI#2} and rejected NSSAI list {NSSAI#3, NSSAI#4}. Afterwards, the user launches an APP which is associated to NSSAI#3, and the NAS layer rejects the slice after checking the allowed NSSAI list. If the operator re-plans the slice deployment based on the rejected slices information in CN (i.e. NSSAI#3, NSSAI#4), both NSSAI#3 and NSSAI#4 will be deployed in this registration area. However, if a number of UE request the NSSAI#3 which rejected by the UE inside and report this information, the operator only need to deploy NSSAI#3 in this area, which is more effective and economical.

Observation 1: The failure of a slice initiation inside the UE (i.e. the requested slice is rejected by NAS) is not aware by network side, but it is valuable for the operator to assess the requirements from the mobile subscribers for the slice, which can be used by the operator for network planning of slice deployment.

Observation 2: It is not precise enough to use the rejected slices by CN for slice deployment, since the UE usually reports all subscribed slices to the CN during the registration procedure but it may not actually trigger these services.

Proposal 1: Support to report the failure of slice initiation inside the UE, when the higher layer initiates a slice but rejected by UE’s NAS layer due to NSSAI not within the allowed NSSAI list in current registration area.

# 3	Conclusion

Here are the observations and proposals for SON/MDT enhancements for network slicing.

Observation 1: The failure of a slice initiation inside the UE (i.e. the requested slice is rejected by NAS) is not aware by network side, but it is valuable for the operator to assess the requirements from the mobile subscribers for the slice, which can be used by the operator for network planning of slice deployment.

Observation 2: It is not precise enough to use the rejected slices by CN for slice deployment, since the UE usually reports all subscribed slices to the CN during the registration procedure but it may not actually trigger these services.

Proposal 1: Support to report the failure of slice initiation inside the UE, when the higher layer initiates a slice but rejected by UE’s NAS layer due to NSSAI not within the allowed NSSAI list in current registration area.

# 4	Reference

Draft_RAN3#127-bis Meeting Report