3GPP TSG-RAN WG3 Meeting #128	             R3-253693

Malta, MT, 19th – 23th May 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on Inventory Procedure and Signalling

Document for:	Discussion

# 1	Introduction

During last meeting, we agreed the following agreements on Inventory Procedure and Signalling.

About Inventory Procedures: - WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - The Inventory Request Transfer IE, also includes the following: - Device Identification for Paging (to enable paging for single device, a group of devices, all devices) - Requested Service Area Information (A-IoT Area list and/or Reader list) - Inventory Assistance Information (Approximates Number of Target A-IoT Devices). - It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

In this contribution, we will provide our views on signaling and procedure between A-IoT RAN and A-IoT CN to support inventory operation.

# 2	Discussion

After a series of discussion on the Correlation Identifier IE in last meeting, RAN3 reached the common understanding that this IE serves as a unique marker for the gNB to distinguish between different inventory sessions from the AIOTF. Next step is deciding the IE location for the Correlation Identifier IE, RAN3 made a work assumption that include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. The reason for putting it outside is to identify the same A-IoT session for the AMF among parallel A-IoT inventory operations in the indirect deployment, so we think the work assumption made in last meeting should be changed to an agreement, i.e., introducing a NGAP Class 1 Inventory Request procedure. Meanwhile, to make the AIOTF identify the same A-IoT session among parallel A-IoT inventory operations, the Correlation Identifier IE also needs to be included inside of the Inventory related Transfer IEs in all the Inventory related messages.

Proposal 1: Include the Correlation Identifier IE inside and outside of the Inventory related Transfer IEs in all the Inventory related messages.

Based on the progress of the last meeting, RAN3 already determined some sub-IEs need to be included in the Inventory Request Transfer IE. Inside the Inventory Request Transfer IE, one of the sub-IEs is the A-IoT Device Identification Requested IE representing the Requested Identity of the A-IoT device to enable paging for single device, a group of devices, and all devices. This IE can be designed as a CHOICE type with the choice to indicate whether the inventory applies to a single device, a group of devices, or all devices. The A-IoT device can check and determine whether the inventory points to itself. Assuming that AIOTF decides to inventory a single A-IoT device, the A-IoT Device Identification Requested IE would be the single A-IoT device ID. Assuming that AIOTF decides to inventory a group of devices, the A-IoT Device Identification Requested IE would be a Mask ID or a Group ID. Assuming that AIOTF decides to inventory all devices in an area, the A-IoT Device Identification Requested IE is not included in Inventory Request Transfer IE, instead using NULL codepoint which signifies the inventory operation applies to all A-IoT devices.

Proposal 2: The A-IoT Device Identification Requested IE can be designed as a CHOICE type with the choice to indicate whether the inventory applies to a single device, a group of devices, or all devices.

Additionally, RAN3 agreed the Request Service Area Information IE needs to be included in the Inventory Request Transfer IE from A-IoT CN which is encoded as A-IoT area and/or reader ID list. For the case that a gNB simultaneously receives the A-IoT area and the reader ID list within the Inventory Request Transfer IE, as stated in R3-253692, we think the case does not exist since it is impossible for A-IoT CN to provide both information at the same time in Inventory Request message which will make the gNB confused if the readers within indicated area and the reader list provided by CN is not consistent.

Proposal 3: The case where a gNB simultaneously receives the A-IoT area and the reader ID list within the Inventory Request Transfer IE does not exist.

There is a remaining FFS in last meeting to be discussed is whether the Inventory Assistance Information IE within the Inventory Request Transfer IE includes the “Estimate of Expected D2R Message Size”. The estimate of expected D2R message size can make the gNB aware the length of device ID and assign the UL resource more efficiently. Before the meeting, RAN3 also received an LS from RAN2 to check their conclusions on the inventory response message size. The content of LS is shown as follows:

RAN2 discussed segmentation of D2R messages and made the following assumption on inventory response message size. Agreements on segmentation - For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. RAN2 would like to inform SA2 and RAN3 that RAN2 assumes that the inventory response size can always be provided by the core network to the reader.

It is noted that RAN2 always assumes that the inventory response size can always be provided by the core network to avoid the A-IoT device ID segmentation received by the reader, so this message size information is mandatory included within Inventory Assistance Information IE in the Inventory Request Transfer IE.

In addition, the length of AIoT Device Permanent Identifier is already decided and reflected in SA2 CR.

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits. NOTE 4: The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

SA2 already supported the length for the Identification Information in an AIoT Device Permanent Identifier as 96bit or 128bit, so the D2R message size can be determine and currently with two choices. Thus, the IE needs to be renamed as "Expected D2R message size" designed to ENUMERATE type temporarily with 96bit and 128bit.

Proposal 4: The "Expected D2R Message Size" is designed to ENUMERATE type temporarily with 96bit and 128bit as a mandatory IE within Inventory Assistance Information IE in the Inventory Request Transfer IE.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on the inventory procedure and signaling:

Proposal 1: Include the Correlation Identifier IE inside and outside of the Inventory related Transfer IEs in all the Inventory related messages.

Proposal 2: The A-IoT Device Identification Requested IE can be designed as a CHOICE type with the choice to indicate whether the inventory applies to a single device, a group of devices, or all devices.

Proposal 3: The case where a gNB simultaneously receives the A-IoT area and the reader ID list within the Inventory Request Transfer IE does not exist.

Proposal 4: The "Expected D2R Message Size" is designed to ENUMERATE type temporarily with 96bit and 128bit as a mandatory IE within Inventory Assistance Information IE in the Inventory Request Transfer IE.

# 4	Reference

[1] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TS 23.369 v0.3.0, Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19);