# 3GPP TSG-RAN WG3 Meeting #128	R3-253728

# Malta, MT, 19 - 23 May, 2025

# Agenda Item:	21.3

# Source:	ZTE Corporation

# Title:	Further discussion on XR uplink rate control with LS and TP to BLCR for 38473

# Document for:	Discussions & Decision

- Introduction

In previous meeting, RAN3 has made following progress on uplink rate control:

Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

Introduce the same indication in XnAP to inform target gNB.

FFS on per QoS flow indication or per DRB indication over F1AP?

FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

Continue the discussion on UL rate control.

In this meeting, we prefer to further discuss the remaining issues on XR marked as blue and provide our views. TP may be used to capture RAN3 progress is also provided in our other contributions.

# 2 Discussion

2.1 Previously On

The discussion on uplink rate control began when RAN3 received RAN2's LS(R3-250012) at RAN3#127. By the end of this meeting, RAN3 reached a consensus that transmitting per-QoS-flow indications to the UE via either MAC CE or RRC would impact F1AP and responded to RAN2 in their reply LS(R3-250805).

Subsequently, both RAN2 and RAN3 further discussed this topic in the XR TU. However, RAN3 has not reached any further agreement on this matter. Meanwhile, based on feedback from RAN2, the following agreement was made in RAN2#129bis:

==== agreement in RAN2#129bis ====

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

==== agreement in RAN2#129bis ====

It is observed that RAN2 has chosen MAC CE solution and is making further progress on the MAC CE design.If RAN3 has no significant concerns regarding the F1 design, RAN2 will continue to proceed in this direction.

Observation 1: RAN2 has selected to transmit the per QoS flow information for uplink rate control via MAC CE and is making progress accordingly.

2.2 For non-split architecture

In case of non-split architecture, the gNB terminates all protocol layers including SDAP and hence the gNB can monitor the bit rate per QoS flow and generate the MAC CE for recommended bit rate on a per QoS flow basis. Thus for this case, the solution agreed in RAN2 is feasible and has no impact to RAN3.

Proposal 1: RAN3 should inform RAN2 that in case of non-split architecture, the solution agreed by RAN2 is feasible and has no impact to RAN3 specs.

2.3 For split architecture

Based on our discussion so far, whether DU has the capacity to be aware of the uplink bit rate information for a specific QoS flow can be divided into two parts: GBR QoS flow and non-GBR QoS flow.

- For GBR QoS flow, since one QoS flow maps to one DRB, there is a general consensus that the DU can be aware of the uplink bit rate.

- For non-GBR QoS flow, considering the QoS flow and DRB are many to one mapping relationship, some companies have some concern above whether DU can be aware of the uplink bitrate for a specific QoS flow in this scenario.

To make some progress, we prefer to have the following agreement based on our previous consensus and will further discuss the controversial part based on our updated progress in next following sections.

Proposal 2: For GBR QoS flow, DU is aware of the bit rate state of this QoS flow and can send MAC CE to UE for recommended uplink bit rate.

2.3.1 DU based solution

In this part, we propose to discuss why/how DU is aware of the uplink bit rate for non-GBR QoS flow as well as the potential F1AP impact.

Given the current mechanism, QoS flow parameters for each QoS flow are forwarded from CU to DU. With the detailed uplink bit rate state for a specific DRB and the QoS flow parameters for QoS flows mapped to this DRB, DU can perform uplink bit rate control for each QoS flow if needed. More specifically, DU can setup the recommended bit rate for each QoS flow based on the received QoS parameters via F1AP messages, in combination with its own resource situation, or based on actual traffic statistics

Observation 2: DU can perform uplink bit rate control for non-GBR QoS flow by using QoS parameters and DRB traffic state.

Moreover, as pointed out by contribution(R3-251856) in previous meeting, the SDAP header is not ciphered. Hence, for non-GBR QoS flow, DU can read the QFI in SDAP header for the UP packets transmitted via a DRB and hence can estimate a non-GBR QoS flow’s uplink bit rate state.

It is observed that, based on the current mechanism defined in TS 38.323, if the SDAP header compression is enabled, some UP packets may not carry the SDAP header. In other words, whether all UP packets have an SDAP header depends on the configuration. But considering the uplink rate control for a specific QoS flow is also an optional function, it is not difficult for the NW to require all UP packets carry the SDAP header for the QoS flows that need to perform uplink bit rate control.

Observation 3: DU can read the QFI in SDAP header to estimate the uplink bit rate of an non-GBR QoS flow.

Therefore, based on the above description, the DU can perform recommended uplink bit rate for non-GBR QoS flow to the UE by using the following alternatives:

- Received QoS flow parameters for this QoS flow and the related DRB traffic state

- QFI in the SDAP header

Observation 4: DU can independently be aware of non-GBR QoS flow bit rate by using the SDAP header and/or received QoS flow parameters for this QoS flow along with the DRB traffic state.

Moreover, we also prefer to discuss the potential F1AP impact when DU independently provides recommended uplink bit rate. The general RAN3 related procedure is shown below:

Figure 2.3.1-1

It is noted that the NGAP part has been agreed in previous RAN3 meeting. Given that DU is responsible for the uplink rate control, after CU receives the requirement from CN, CU shall inform DU the QoS flow information(e.g. QFI) and the uplink rate control indication. Then when available, DU may send the recommended uplink bit rate information for a specific QoS flow to UE via MAC CE.

2.3.2 CU based solution

It is clear that some companies have concerns on whether DU can independently be aware of the non-GBR QoS flow bit rate, and whether MAC CE can be used for the uplink bit rate control. In this part, we will further explain whether/how DU transmit the uplink bit rate info to UE via MAC CE if DU can not independently be aware of the non-GBR QoS flow bit rate.

In this case, as explained in previous contribution(R3-251955), it is clear that at least CU is aware of the bit rate of a QoS flow. CU may be responsible for the uplink bit rate monitoring and provide the necessary information(e.g. recommended uplink bit rate for a QoS flow) to DU. When DU receives such information, it can forward this information to UE via MAC CE. A figure illustrating the general procedures for this alternative is shown below:

Figure 2.3.2-1

In this alternative, though CU is the entity responsible for UL rate control, CU may need to inform DU which QoS flow need the uplink bit rate modification after it receives the information from CN. When CU determines that the uplink bit rate for a specific QoS flow needs to be modified, CU can send the QoS flow information(e.g. QFI) and the recommended uplink bit rate information to DU. Then, DU passes the information to UE via MAC CE.

Observation 5: CU can be aware of non-GBR QoS flow bit rate and provide the recommended uplink bit rate information for specific QoS flow to DU.

2.3.3 CU assisted solution

As explained in above sections, no matter whether DU can independently be aware of the bit rate for non-GBR QoS flow, DU can send the recommended uplink bit rate information to UE via MAC CE with F1AP enhancement. Therefore, regardless of which solutions is applied, the following agreement can be made:

Proposal 3: DU can provide the non-GBR QoS flow recommended uplink bit rate to UE via MAC CE. This bit rate information may be either received from CU or determined by DU.

In addition, we proposed to further check companies view on final solution may be used for this uplink rate control function. It is clear that one function with two different parallel solutions is not ideal for the deployment. We also suggest discussing whether the above two solutions can be combined. Specifically:

Figure 2.3.3-1

The figure above shows the combined solution which combined by solution 1 and solution 2 with the following procedures.

- CU sends the uplink bit rate indication for specific QoS flow to DU with/without recommended uplink bit rate information for this QoS flow.

- CU can send the recommended uplink bit rate information for a QoS flow to DU. Meanwhile,

  - If DU is aware of the uplink bit rate state for this QoS flow, DU may take the received uplink bit rate info into account and determine a recommended uplink bit rate info for this QoS flow. Then DU forwards this info to UE via MAC CE.

  - If DU is not aware of the uplink bit rate state for this QoS flow, and can not provide the recommended uplink bit rate info on its own. DU may directly forward the received uplink rate information for this QoS flow to UE via MAC CE.

By using this solution, RAN3 can avoid the debating whether DU is able to know the non-GBR QoS flow uplink bit rate.  For gNB-DU tha have such capacity, the DU itself can handle the uplink rate control function and may take the received assistance info from CU into account. For gNB-DU that do not have such capacity, gNB-CU can provide the required uplink bit rate info for specific QoS flow, which this gNB-DU can directly forward to UE via MAC CE.

Compared with the previous solutions, the 3rd solution offers a flexible space for product implementation. The procedures for solution 2 and solution 3 looks similar. However, the significant difference between these solutions is that:

For CU based solution, CU is the only entity responsible for the UL rate control decision. DU merely passes the CU determined recommended uplink bit rate info to UE.

- For CU assisted solution, depending on the DU capacity, either DU or CU may be the entity that make the final decision on the recommended uplink rate information.

In short, it is proposed for RAN3 to make decision on which of the following 3 solutions shall be used for uplink rate control function.

- DU based solution: DU is fully responsible for uplink rate control for a specific QoS flow and sends the recommended uplink bit rate information to UE via MAC CE.This is the solution with minimum RAN3 impact.

- CU based solution: CU is responsible for uplink rate control for a specific QoS flow and sends the recommended uplink bit rate information to DU. Then DU passes the received  information to UE via MAC CE.

- CU assisted solution: CU can sends the recommended uplink bit rate information for a specific QoS flow to DU. Then DU sends the recommended uplink bit rate information to UE via MAC CE. The info send from DU may either be determined by DU based on its local state and/or received recommended uplink bit rate info from CU, or directly passed from CU.

Proposal 4: RAN3 shall discuss the following solutions for uplink rate control:DU based solution, CU based solution, CU assisted solution.

Proposal 5: Introduce the Indication of Bit Rate Adaptation IE and/or Recommended Bit Rate IE in F1AP UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message.

Proposal 6: Capture the proposals using TP to BLCR for TS 38.473 considering the TPs in annex part as baseline.

2.4 Received LS from SA2

Before the start of this meeting, SA2 sends LS to RAN2 and cc RAN3 with the following content:

==== content in SA2 LS(R3-253014) ====

SA2 Feedback:

- SA2 confirms that the CN can indicate to the gNB that a QoS flow is subject to uplink rate control.

- SA2 would like to inform RAN WGs that SA2 has not decided yet on whether the existing or a new mechanism can be reused.

- SA2 would like RAN2 to confirm that the RAN2 LS on UL rate control applies to both GBR and non-GBR.

==== content in SA2 LS(R3-253014) ====

For the first bullet, RAN3 has already confirmed this assumption in our previously sent LS.

For the second and third bullets, it is benefit for RAN3 to sync up our latest progress on uplink rate control with SA2 and RAN2to assist them in the further discussion. Therefore, a draft reply LS is prepared in annex and can be considered as baseline.

Proposal 7: RAN3 may reply the SA2 LS on uplink rate control and consider the annex reply LS as baseline.

# 3	Conclusion

Based on above analysis, we provide the following proposals and observations

Observation 1: RAN2 has selected to transmit the per QoS flow information for uplink rate control via MAC CE and is making progress accordingly.

Proposal 1: RAN3 should inform RAN2 that in case of non-split architecture, the solution agreed by RAN2 is feasible and has no impact to RAN3 specs.

Proposal 2: For GBR QoS flow, DU is aware of the bit rate state of this QoS flow and can send MAC CE to UE for recommended uplink bit rate.

Observation 2: DU can perform uplink bit rate control for non-GBR QoS flow by using QoS parameters and DRB traffic state.

Observation 3: DU can read the QFI in SDAP header to estimate the uplink bit rate of an non-GBR QoS flow.

Observation 4: DU can independently be aware of non-GBR QoS flow bit rate by using the SDAP header and/or received QoS flow parameters for this QoS flow along with the DRB traffic state.

Observation 5: CU can be aware of non-GBR QoS flow bit rate and provide the recommended uplink bit rate information for specific QoS flow to DU.

Proposal 3: DU can provide the non-GBR QoS flow recommended uplink bit rate to UE via MAC CE. This bit rate information may be either received from CU or determined by DU.

Proposal 4: RAN3 shall discuss the following solutions for uplink rate control:DU based solution, CU based solution, CU assisted solution.

Proposal 5: Introduce the Indication of Bit Rate Adaptation IE and/or Recommended Bit Rate IE in F1AP UE CONTEXT SETUP REQUEST message and UE CONTEXT MODIFICATION REQUEST message.

Proposal 6: Capture the proposals using TP to BLCR for TS 38.473 and considering the TP in annex part as baseline.

Proposal 7: RAN3 may reply the SA2 LS on uplink rate control and consider the annex reply LS as baseline.

# Annex Draft Reply LS to SA2 on uplink rate control

Title:	[Draft] Reply LS on uplink rate control

Response to:	R3-253014/S2-2504252

Release:	Rel-19

Work Item:	XRM_Ph2, NR_XR_Ph3-Core

Source:	ZTE Corporation [to be RAN3]

To:	SA2,RAN2

Cc: 	SA4

# Contact Person:

# Name:	Yansheng Liu

# Email:		liu.yansheng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	[TBD]

# 1. Overall Description:

RAN3 thanks SA2 informing the progress on uplink rate control discussion. RAN3 has made the following agreement on uplink rate control function:

- From RAN3 point of view, the uplink rate control can apply to both GBR and non-GBR QoS flow.

- CU can sends the recommended uplink bit rate information for a specific QoS flow to DU. Then DU sends the recommended uplink bit rate information to UE via MAC CE. The info send from DU may either be determined by DU based on its local state and/or received recommended uplink bit rate info from CU, or directly passed from CU.

RAN3 agreed the corresponding Stage 3 updates.

# 2. Actions:

# To SA2 and RAN2:

ACTION: RAN3 kindly asks SA2 and RAN2 to take the above RAN3 progress into account for further work on XR uplink rate control and provide feedback if any.

# 3. Date of Next TSG RAN WG3 Meetings:

RAN3#129	25th Aug – 29th Aug 2025					India, IN

RAN3#129bis	13th Oct – 17th Oct 2025					Prague, CZ

Annex TP to BL CR for TS 38.473 UL rate control

CHANGES START

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to provide suggestion on whether to activate or deactivate DL PSI based SDU discarding for the indicated DRB.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

------ Unchanged part skipped ------

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for the UE’s Ranging and Sidelink Positioning service.

For each GBR QoS flow whose DRB has been successfully established and the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and perform available bitrate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established, if the MMSID IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [21] and TS 38.300 [6].

[To be removed]Editor Note: The following description is only used for DU based solution:

For each QoS flow whose DRB to be established, if the Indication of Bit Rate Adaptation IE was contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and perform available bit rate monitoring for this QoS flow, as specified in TS 23.501 [21].

[To be removed]Editor Note: The following description is only used for CU based solution:

For each QoS flow whose DRB to be established, if the Indication of Bit Rate Adaptation IE was contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and consider this QoS flow needs available bit rate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established, if the Recommended Bit Rate IE was contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported,forward the information as specified in TS 23.501 [21].

[To be removed]Editor Note: The following description is only used for CU assisted solution:

For each QoS flow whose DRB to be established, if the Indication of Bit Rate Adaptation IE was contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and consider this QoS flow needs available bit rate monitoring, or perform available bit rate monitoring for this QoS flow, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established, if the Recommended Bit Rate IE was contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported,forward the information, or take it into account for available bit rate monitoring for this QoS flow as specified in TS 23.501 [21].

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

NEXT CHANGE

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE, the gNB-DU shall, if supported, take it into account to provide suggestion on whether to activate or deactivate DL PSI based SDU for the indicated DRB.

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

------ Unchanged part skipped ------

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

For each GBR QoS flow whose DRB has been successfully established or modified and the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and, perform available bitrate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established or modified, if the MMSID IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [21] and TS 38.300 [6].

[To be removed]Editor Note: The following description is only used for DU based solution:

For each QoS flow whose DRB to be established or modified, if the Indication of Bit Rate Adaptation IE was contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and perform available bit rate monitoring for this QoS flow, as specified in TS 23.501 [21].

[To be removed]Editor Note: The following description is only used for CU based solution:

For each QoS flow whose DRB to be established or modified, if the Indication of Bit Rate Adaptation IE was contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and consider this QoS flow needs available bit rate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established or modified, if the Recommended Bit Rate IE was contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported,forward the information as specified in TS 23.501 [21].

[To be removed]Editor Note: The following description is only used for CU assisted solution:

For each QoS flow whose DRB to be established or modified, if the Indication of Bit Rate Adaptation IE was contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and consider this QoS flow needs available bit rate monitoring, or perform available bit rate monitoring for this QoS flow, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established or modified, if the Recommended Bit Rate IE was contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported,forward the information, or take it into account for available bit rate monitoring for this QoS flow as specified in TS 23.501 [21].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

NEXT CHANGE

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>>> Indication of Bit Rate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the QoS Flow allows rate adaptation in the indicated direction. | YES | ignore |
| >>>>>> Recommended Bit Rate | O |  | Bit Rate 9.3.1.22 |  | YES | ignore |
| [To be removed] Editor Note: The Recommended Bit Rate is only needed in CU decided or CU assisted solution. |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>>> Indication of Bitrate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the QoS Flow allows rate adaptation in the indicated direction. | YES | ignore |
| >>>>>> Recommended Bit Rate | O |  | Bit Rate 9.3.1.22 |  | YES | ignore |
| [To be removed] Editor Note: The Recommended Bit Rate is only needed in CU decided or CU assisted solution. |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

#### 9.2.2.13	NOTIFY

This message is sent by the gNB-DU to notify the gNB-CU that the QoS for already established DRBs associated with notification control is not fulfilled any longer or it is fulfilled again.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Notify List |  | 1 |  |  | YES | reject |
| >DRB Notify Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Notification Cause | M |  | ENUMERATED(Fulfilled, Not-Fulfilled, ..., not fulfilled DL, not fulfilled UL) |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters set Notify Index 9.3.1.124 | Index to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameter set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |

NEXT CHANGE

#### 9.3.1.45	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow, or to a DRB, or to a BH RLC channel, or to a Uu Relay RLC channel, or to a PC5 Relay RLC channel, or to a MRB.

NOTE:	For a BH RLC channel, the listed mandatory IEs and the GBR QoS Flow Information IE are applicable, where GBR QoS Flow Information IE may be present if BH RLC channel conveys the traffic belonging to a GBR QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.49 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.47 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.48 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.46 | This IE shall be present for GBR QoS Flows only and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Details in TS 23.501 [21]. This IE applies to non-GBR flows only and is ignored otherwise. | - |  |
| PDU Session ID | O |  | INTEGER (0 ..255) | As specified in TS 23.501 [21]. | YES | ignore |
| UL PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.22 | The PDU session Aggregate Maximum Bit Rate Uplink which is associated with the involved PDU session. | YES | ignore |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| PDCP Terminating Node DL Transport Layer Address | O |  | Transport Layer Address 9.3.2.3 | DL Transport Layer Address of node terminating PDCP. Included for MN-terminated SCG bearers and SN-terminated MCG bearers. | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.319 |  | - |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.319 |  | - |  |
| MMSID | O |  | OCTET STRING (SIZE (FFS)) | Multi-modal service ID from the application, used to indicate QoS flows are related to a multi-modal service, as specified in TS 23.501 [21]. | YES | ignore |

NEXT CHANGE

#### 9.3.1.46	GBR QoS Flow Information

This IE indicates QoS parameters for a GBR QoS flow or GBR bearer for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate in DL. Details in TS 23.501 [21]. | - |  |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate in UL. Details in TS 23.501 [21]. | - |  |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate (provided there is data to deliver) in DL. Details in TS 23.501 [21]. | - |  |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate (provided there is data to deliver). Details in TS 23.501 [21]. | - |  |
| Maximum Packet Loss Rate Downlink | O |  | Maximum Packet Loss Rate 9.3.1.50 | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Details in TS 23.501 [21]. | - |  |
| Maximum Packet Loss Rate Uplink | O |  | Maximum Packet Loss Rate 9.3.1.50 | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Details in TS 23.501 [21]. | - |  |
| Alternative QoS Parameters Set List | O |  | 9.3.1.125 | Indicates alternative sets of QoS Parameters for the QoS flow. | YES | ignore |
| Monitoring Request on Available Data Rate |  | 0..1 |  |  | YES | ignore |
| >Monitoring Request | M |  | ENUMERATED (ul, dl, both, stop, …) | Indicates to monitor and report UL, DL, or both UL/DL available data rate for the associated QoS flow as specified in TS 23.501 [21], or stop the corresponding monitoring. | - |  |
| >DL Available Data Rate Report Thesholds | C-ifReportDL |  | Available Data Rate Reporting Threshold List 9.3.1.x | Indicates the DL report thresholds for available data rate exposure, as specified in TS 23.501 [21]. | - |  |
| >UL Available Data Rate Report Thesholds | C-ifReportUL |  | Available Data Rate Reporting Threshold List 9.3.1.x | Indicates the UL report thresholds for available data rate exposure, as specified in TS 23.501 [21]. | - |  |

| Condition | Explanation |
| --- | --- |
| ifReportDL | This IE shall be present if the Monitoring Request IE is set to the value “dl” or “both”. |
| ifReportUL | This IE shall be present if the Monitoring Request IE is set to the value “ul” or “both”. |

NEXT CHANGE

#### 9.3.1.125	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the gNB can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  | - |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.123 |  | - |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.22 |  | - |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 |  | - |  |
| >Packet Delay Budget | O |  | 9.3.1.51 |  | - |  |
| >Packet Error Rate | O |  | 9.3.1.52 |  | - |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.54 | Maximum Data Burst Volume is specified in TS 23.501 [21]. This IE is included if the Delay Critical IE is set to “delay critical” and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.145 |  | YES | ignore |
| >PDU Set Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.145 |  | YES | ignore |
| >PDU Set Error Rate Downlink | O |  | Packet Error Rate 9.3.1.52 |  | YES | ignore |
| >PDU Set Error Rate Uplink | O |  | Packet Error Rate 9.3.1.52 |  | YES | ignore |

NEXT CHANGE

#### 9.3.1.x	Available Data Rate Reporting Thresholds

This IE contains a list of available data rate report thresholds. It is used for available data rate report for UL and/or DL as specified in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available Data Rate Report Threshold Item |  | 1..<maxnoofThresholds> |  |  |
| >Reporting Threshold | M |  | INTEGER (0..FFS, …) | Unit: Mbps |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of reporting thresholds allowed for one QoS flow on one direction. Value is FFS. |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

------ Unchanged part skipped ------

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-PSIbasedSDUdiscardDL,

id-PduSetDelayBudgetDownlink,

id-PduSetDelayBudgetUplink,

id-PduSetErrorRateDownlink,

id-PduSetErrorRateUplink,

id-MonitoringRequestonAvailableDataRate,

id-MMSID,

id-IndicationOfBitRateAdaptation,

id-RecommendedBitRate,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

------ Unchanged part skipped ------

maxnoHopsMinusOne,

maxnoAggCombinations,

maxnoAggregatedPosSRSCombinations,

maxnoofThresholds

NEXT CHANGE

#### -- A

------ Unchanged part skipped ------

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			QoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MaxDataBurstVolume 	CRITICALITY ignore	EXTENSION MaxDataBurstVolume		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetDownlink 	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetErrorRateDownlink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		}|

{ ID id-PduSetErrorRateUplink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		},

...

}

NEXT CHANGE

ARPLocationInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ARPLocationType ::= CHOICE {

aRPPositionRelativeGeodetic			RelativeGeodeticLocation,

aRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-SingleContainer { { ARPLocationType-ExtIEs } }

}

ARPLocationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

AvailableDataRateReportThresholdList	::= SEQUENCE (SIZE(1..maxnoofThresholds)) OF AvailableDataRateReportThresholdItem

AvailableDataRateReportThresholdItem	::= SEQUENCE {

reportingThreshold				ReportingThreshold,

iE-Extensions					ProtocolExtensionContainer { { AvailableDataRateReportThresholdItem-ExtIEs} }	OPTIONAL,

...

}

AvailableDataRateReportThresholdItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NEXT CHANGE

#### -- D

------ Unchanged part skipped ------

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardUL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardUL		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardDL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardDL		PRESENCE optional	},

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

NEXT CHANGE

#### -- F

------ Unchanged part skipped ------

Flows-Mapped-To-DRB-List	::=	SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF Flows-Mapped-To-DRB-Item

Flows-Mapped-To-DRB-Item 	::= SEQUENCE {

qoSFlowIdentifier							QoSFlowIdentifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

iE-Extensions							ProtocolExtensionContainer { { Flows-Mapped-To-DRB-ItemExtIEs} } OPTIONAL

}

Flows-Mapped-To-DRB-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoSFlowMappingIndication			PRESENCE optional}|

{ID id-TSCTrafficCharacteristics	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics			PRESENCE optional}|

{ID id-IndicationOfBitRateAdaptation	CRITICALITY ignore	EXTENSION IndicationOfBitRateAdaptation	PRESENCE optional}|

{ID id-RecommendedBitRate			CRITICALITY ignore	EXTENSION BitRate							PRESENCE optional},

...

}

[To be removed] Editor Note: RecommendedBitRate is only needed for CU based or CU assisted solution.

NEXT CHANGE

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink			BitRate,

maxFlowBitRateUplink			BitRate,

guaranteedFlowBitRateDownlink	BitRate,

guaranteedFlowBitRateUplink		BitRate,

maxPacketLossRateDownlink		MaxPacketLossRate		OPTIONAL,

maxPacketLossRateUplink			MaxPacketLossRate		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ 	ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	}|

{ 	ID id-MonitoringRequestonAvailableDataRate	CRITICALITY ignore	EXTENSION MonitoringRequestonAvailableDataRate 	PRESENCE optional	},

...

}

NEXT CHANGE

#### -- I

------ Unchanged part skipped ------

IndicationOfBitRateAdaptation::= ENUMERATED {

uplink,

...

}

NEXT CHANGE

#### -- M

MappingInformationIndex	::= BIT STRING (SIZE (26))

MappingInformationtoRemove	::= SEQUENCE (SIZE(1..maxnoofMappingEntries)) OF MappingInformationIndex

MaskedIMEISV ::= 	BIT STRING (SIZE (64))

------ Unchanged part skipped ------

MUSIM-GapConfig ::= OCTET STRING

MobileIAB-Barred	::=	ENUMERATED {barred, not-barred, ...}

MeasBasedOnAggregatedResources ::= ENUMERATED { true, ... }

MMSID ::= OCTET STRING (SIZE (FFS))

MonitoringRequestonAvailableDataRate	::= SEQUENCE{

monitoringRequest						MonitoringRequest,

dlAvailableDataRateReportThresholds		AvailableDataRateReportThresholdList		OPTIONAL,

-- The above IE shall be present if the Monitoring Request IE is set to the value “dl” or “both”

ulAvailableDataRateReportThresholds		AvailableDataRateReportThresholdList		OPTIONAL,

-- The above IE shall be present if the Monitoring Request IE is set to the value “ul” or “both”

iE-Extensions							ProtocolExtensionContainer { { MonitoringRequestonAvailableDataRate-ExtIEs} }	OPTIONAL,

...

}

MonitoringRequestonAvailableDataRate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MonitoringRequest ::= ENUMERATED {ul, dl, both, stop,...}

NEXT CHANGE

#### -- N

------ Unchanged part skipped ------

NonUPTrafficType ::=	ENUMERATED {ue-associated, non-ue-associated, non-f1, bap-control-pdu,...}

NoofDownlinkSymbols	::= INTEGER (0..14)

NoofUplinkSymbols	::= INTEGER (0..14)

Notification-Cause ::= ENUMERATED {fulfilled, not-fulfilled, ..., not-fulfilled-dl, not-fulfilled-ul}

NotificationControl ::= ENUMERATED {active, not-active, ...}

NotificationInformation ::= SEQUENCE {

message-Identifier	MessageIdentifier,

serialNumber		SerialNumber,

iE-Extensions	ProtocolExtensionContainer { { NotificationInformationExtIEs} } OPTIONAL,

...

}

NotificationInformationExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

NEXT CHANGE

#### -- P

------ Unchanged part skipped ------

PDUSetQoSInformation	::= SEQUENCE {

pduSetDelayBudget						ExtendedPacketDelayBudget		OPTIONAL,

pduSetErrorRate							PacketErrorRate					OPTIONAL,

pduSetIntegratedHandlingInformation		ENUMERATED {true, false, ...}	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { PDUSetQoSInformation-ExtIEs } }	OPTIONAL

}

PDUSetQoSInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PSIbasedSDUdiscardUL ::= ENUMERATED {start, stop, ...}

PointA  ::= INTEGER (0..3279165)

PSIbasedSDUdiscardDL ::= ENUMERATED {configured, not-configured, ...}

NEXT CHANGE

#### -- Q

------ Unchanged part skipped ------

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation				OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionID								CRITICALITY ignore	EXTENSION PDUSessionID				PRESENCE optional}|

{ ID id-ULPDUSessionAggregateMaximumBitRate			CRITICALITY ignore	EXTENSION BitRate						PRESENCE optional}|

{ ID id-QosMonitoringRequest						CRITICALITY ignore	EXTENSION QosMonitoringRequest		PRESENCE optional}|

{ ID id-PDCPTerminatingNodeDLTNLAddrInfo			CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional	}|

{ ID id-PDUSetQoSParameters							CRITICALITY	ignore	EXTENSION PDUSetQoSParameters		PRESENCE optional}|

{ ID id-MMSID										CRITICALITY ignore	EXTENSION MMSID							PRESENCE optional},

...

}

NEXT CHANGE

#### -- R

RACH-Config-Common	::= OCTET STRING

RACH-Config-Common-IAB	::= OCTET STRING

------ Unchanged part skipped ------

ResourceTypeAperiodicPos ::= SEQUENCE {

slotOffset			INTEGER (0..32),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodicPos-ExtIEs} }	OPTIONAL

}

ResourceTypeAperiodicPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ReportingThreshold ::= INTEGER(0..FFS, ...)

RLCDuplicationInformation ::= SEQUENCE {

rLCDuplicationStateList 			RLCDuplicationStateList,

primaryPathIndication			PrimaryPathIndication	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RLCDuplicationInformation-ExtIEs} }	OPTIONAL

}

NEXT CHANGE

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

------ Unchanged part skipped ------

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

------ Unchanged part skipped ------

maxnoAggCombinations					INTEGER ::= 2

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

maxnoofThresholds						INTEGER ::= FFS

NEXT CHANGE

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

------ Unchanged part skipped ------

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-PSIbasedSDUdiscardDL								ProtocolIE-ID ::= xxx

id-PduSetDelayBudgetDownlink						ProtocolIE-ID ::= xxx1

id-PduSetDelayBudgetUplink							ProtocolIE-ID ::= xxx2

id-PduSetErrorRateDownlink							ProtocolIE-ID ::= xxx3

id-PduSetErrorRateUplink							ProtocolIE-ID ::= xxx4

id-MonitoringRequestonAvailableDataRate 			ProtocolIE-ID ::= xxx5

id-MMSID											ProtocolIE-ID ::= xxx6

id-IndicationOfBitRateAdaptation					ProtocolIE-ID ::= xxxx

id-RecommendedBitRate								ProtocolIE-ID ::= xxxy

END

-- ASN1STOP

CHANGES END