3GPP TSG-RAN3 Meeting #129bis																R3-256550

Prague, CZ, 13-17 October, 2025

Agenda Item:	12.2.2

Source:	ZTE Corporation

Title:	(TP to TR 38.745) Consideration on other handover enhancements

Document for:	other

- Introduction

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

- handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

The Rel-20 study will focus on evaluating AI/ML-based mobility use cases, including multi-hop UE trajectory across gNBs, lower-layer triggered mobility (LTM), and other handover enhancements. The objective is to improve handover reliability, reduce latency, and optimize mobility-related resource allocation through predictive and data-driven approaches.

In this contribution, we focus on the topic of other handover enhancements and provide our proposed solution.

- Discussion

Among the objectives of the Rel-20 study, inter-CU LTM handover is listed under other handover enhancements. From the perspective of LTM discussions, the general order is to first study intra-CU LTM before moving on to inter-CU LTM. Since the 2nd objective of RAN3 is AI/ML-assisted intra-CU LTM, it is more appropriate to first conduct a thorough study on the benefits and standardization impacts of applying AI/ML to intra-CU LTM. Only after these aspects have been fully understood, RAN3 can, if time permits, extend the discussion to inter-CU LTM.

- It is suggested that RAN3 prioritize the study of AI/ML-assisted intra-CU LTM and, depending on available time and progress, subsequently address inter-CU LTM within the Rel-20 study phase.

For the other handover enhancements, it is noted that during Rel-18 and Rel-19, the standardization work on mobility optimization mainly focused on predicted UE trajectory and measured UE trajectory. In Rel-18, the predicted UE trajectory was defined at the cell level. This allowed the network to anticipate the next serving cell, but with limited granularity. If further enhancements are considered in Rel-20, it may be beneficial to extend the predicted UE trajectory to a finer granularity. For example: Beam-level trajectory prediction, enabling more precise handover preparation and beam management.

- Extend the predicted UE trajectory inforamtion to a finer granularity, e.g., beam-level trajectory prediction.

During the Rel-19 study item phase, there was discussion on whether to introduce the predicted UE trajectory, e.g., predicted PSCell(s), in the DC procedure (e.g., SN Addition, MN-initiated SN change) from the MN to the SN. However, a few companies remain unconvinced of the benefits of this type of assistance information.

- During Rel-19 SI phase, there is no consensus to introduce the predicted UE trajectory over NR-DC.

It is important to emphasize that, based on the conclusions of Rel-18, the gNB has the capability to perform AI-based predictions. The gNB can also utilize the UE’s historical trajectory and predicted UE trajectory to further generate recommended PSCell ID(s) for SN.

In Release 18, we introduced the concept of the predicted UE trajectory in the handover request message. The predicted UE trajectory represents a list of cells where the UE is expected to connect, in chronological order. The first predicted cell, placed at the top of this list, is the cell to which the UE will move after leaving the serving cell at the source node. Before the source NG-RAN node initiates the HO Request procedure, it can refer to the predicted UE trajectory to select a suitable target cell for the UE and convey this information to the target NG-RAN node.

9.2.3.180	Cell Based UE Trajectory Prediction

The Cell Based UE Trajectory Prediction IE contains information related to NG-RAN cells where the UE is predicted to connect.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Cell Based UE Trajectory Prediction |  | 1 |  | List of cells where the UE is predicted to connect, in chronological order. The first predicted cell added to the top of this list, is the cell in the target node where the UE will move to after the serving cell at the source node. |
| >Predicted UE Trajectory Item |  | 1..<maxnoofCellsTrajectoryPredict> |  |  |
| >>Predicted Trajectory Cell Information | M |  | 9.2.3.181 |  |

Upon reviewing the current description of the Xn interface, "The first predicted cell added to the top of this list is the cell in the target node where the UE will move to after the serving cell at the source node." This implies that the source node has already predicted a cell ID before the handover to assist with the handover request. Similar principle also applies to the DC procedure.

In the current DC procedure (e.g., SN Addition procedure), there is no information related to the PSCell ID. In the legacy process, the MN does not explicitly instruct the SN on which cell to select as the PScell. However, MN could provide the assistance information to SN to choose and configure SCG cells. Now MN as NG-RAN node can predict UE trajectory, and then MN can use this information to select the appropriate target SN node. As a result, the MN could provide a predicted (suggested) PSCell ID to the SN for reference.

Therefore, it is proposed to introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information.

- MN, as a NG-RAN node, is able to leverage the predicted information to generate the assistance information to SN.

- Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information.

- Conclusion

Following is the observations and proposals:

- It is suggested that RAN3 prioritize the study of AI/ML-assisted intra-CU LTM and, depending on available time and progress, subsequently address inter-CU LTM within the Rel-20 study phase.

- Extend the predicted UE trajectory inforamtion to a finer granularity, e.g., beam-level trajectory prediction.

Observation 1 	During Rel-19 SI phase, there is no consensus to introduce the predicted UE trajectory over NR-DC.

- MN, as a NG-RAN node, is able to leverage the predicted information to generate the assistance information to SN.

- Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information.

- Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

4.3	Other handover enhancements

4.3.1 Finer granularity UE trajectory information

In Rel-18, the predicted UE trajectory was defined at the cell level. This allowed the network to anticipate the next serving cell, but with limited granularity. Further enhancements are considered in Rel-20 that it is beneficial to extend the predicted UE trajectory to a finer granularity.

4.3.2 Mobility Optimization over NR-DC

In the current DC procedure (e.g., SN Addition procedure), there is no information related to the PSCell ID. In the legacy process, the MN does not explicitly instruct the SN on which cell to select as the PScell. However, MN could provide the assistance information to SN to choose and configure SCG cells. Now MN as NG-RAN node can predict UE trajectory, and then MN can use this information to select the appropriate target SN node. As a result, the MN could provide a predicted (suggested) PSCell ID to the SN for reference.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>