3GPP TSG RAN WG3 Meeting #129-bis														R3-256560

Prague, CZ, 13 - 17 October, 2025

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Discussion on per-UE UE performance metrics

Document for:	other

1	Introduction

RAN3 asked SA5 to check the feasibility of the average packet delay per DRB per UE and average packet loss in UL and DL at PDCP level, and to specify the solution to align the assumptions in RAN3. The reply LS on per-UE UE performance metrics was received in this meeting.

In this contribution, we put forward the solution to handle the left issues about Rel-18 UE performance metrics and provide the corresponding CRs.

2	Discussion

For Average packet delay measurements:

RAN3 would like to ask SA5 whether the measurements are aggregated per-DRB per-UE to reflect corresponding per-UE metrics. If such per UE aggregation is not supported and if such UE aggregation is feasible, RAN3 would like to ask SA5 to specify these measurements.

SA5’s answer: SA5 confirms that RAN3’s assumption is correct and the measurements defined in TS 28.558 clause 6.3.1.1 are aggregated per-DRB per-UE to reflect corresponding per-UE metrics.

For Average packet loss measurements:

RAN3 would like to further ask whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which may also be aggregated to reflect a per-UE Packet Loss Rate metric.

SA5’s answer: SA5 confirms that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity. SA5 has specified the corresponding packet loss and drop rate measurements in TS 28.558.

SA5 replied the LS [1] to RAN3 that they specified the average packet delay and average packet loss per DRB per UE.

The packet delay related measurements have already defined in clause 6.3.1.1, the UL PDCP SDU Loss Rate measurement in split gNB deployment have been already defined in clause 6.3.1.3. And SA5 updates their specification with the addition of DL packet drop rate measurement definition for split and non-split architecture, and UL PDCP SDU Loss Rate measurement in non-split gNB in TS28.558.

- UL PDCP SDU Loss Rate in split architecture and non-split architecture are defined in TS 28.558.

- DL PDCP SDU Drop Rate in split architecture and non-split architecture are defined in TS 28.558.

- Introduce Average Packet Loss Rate UL in the UE performance IE, and add semantic description to refer to UL PDCP SDU Loss Rate in TS 28.558.

Additionally, SA5 confirms that Average Packet Delay IE that the corresponding per-UE measurement is derived from per-DRB measurements across all configured DRBs for the UE. Since SA5 supports measuring and calculating average UL/DL packet delay at the per-UE level through the aggregation of measurements at the DRB level, we propose to directly adopt this definition in the RAN3 specification.

- Refer to the definition in TS 28.558 for Average UL/DL UE packet delay.

Another issue is the wrong reference for packet loss DL IE in the current specification.

9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.1. |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.2. |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

9.2.3.11	Packet Loss Rate

This IE indicates the Packet Loss Rate for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Loss Rate | M |  | INTEGER (0..1000, ...) | Ratio of lost packets per number of packets sent, expressed in tenth of percent. |

The issue was initially discussed in the last meeting, and following is the solution proposed to correct the mistake:

-	Option 1: Remove the IE, i.e., removing the packet loss rate in Rel-18.

-	Option 2: Introduce new IE for packet loss metric in UE performance to replace the old IE. (NBC change)

-	Option 3: Ignore the IE in R18, and introduce the new IE for packet loss Rate. (BC change)

-	Option 4: Do nothing.

Based on the observations and proposals above, Option 1 and Option 4 can be excluded since the UL PDCP SDU Loss Rate and DL PDCP SDU Drop Rate are supported in SA5. Regarding Option 2 and Option 3, the difference lies in the fact that one is an NBC approach, while the other is a BC approach. Given that R18 is the first release to specify AI/ML for NG-RAN, and in the interest of achieving clean standard specifications, an NBC approach to resolve this wrong reference issue is considered acceptable.

And more straightforward way is to remove the reference for Average packet loss DL IE. And add the range of packet loss DL IE with the update of semantic description to refer to DL PDCP SDU Drop Rate in TS28.558.

- Add the range of Average packet loss DL IE to replace the old reference, and update its semantic description to refer to DL PDCP SDU Drop Rate in TS 28.558.

In Rel-19, the same corrections should be applied to UE performance per S-NSSAI and UE performance over E1 interface.

- Same corrections should be applied to UE performance per S-NSSAI and UE performance per DRB over E1 interface.

3	Conclusion

Following is our observations and proposals:

- UL PDCP SDU Loss Rate in split architecture and non-split architecture are defined in TS 28.558.

- DL PDCP SDU Drop Rate in split architecture and non-split architecture are defined in TS 28.558.

- Introduce Average Packet Loss Rate UL in the UE performance IE, and add semantic description to refer to UL PDCP SDU Loss Rate in TS 28.558.

Proposal 2	Refer to the definition in TS 28.558 for Average UL/DL UE packet delay.

Proposal 3	Add the range of Average packet loss DL IE to replace the old reference, and update its semantic description to refer to DL PDCP SDU Drop Rate in TS28.558.

Proposal 4	Same corrections should be applied to UE performance per S-NSSAI and UE performance per DRB over E1 interface.

4	Reference

[1] R3-256518, Reply LS on per-UE UE performance metrics, SA5