3GPP TSG-RAN WG3 Meeting #129bis       	   				    R3-256582

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:		14.2

Source:			China Telecom

Title:	Support of Ambient IoT Topology 2

Document for:	 	Discussion & Decision

- Introduction

The Rel-20 Ambient IoT WID contains the following objective[1]:

RAN3

- Specify signaling for Topology 2

- The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection.

  - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized.

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

In this contribution, we will focus on the support of Topology 2 and provide some initial considerations.

- Discussion

2.1 Signalling for Inventory and Command

Based on SA2 and RAN Plenary progress, the support of Topology 2 in Rel-20 is based on the RRC-based option, and only indirect connectivity is applied. Candidate procedures for A-IoT Command and Inventory can refer to the procedures in TR 38.769 [2]. Figure 1 depicts the basic communication among AIoT devices, UE Reader, gNB, and A-IoT CN node for the Inventory procedure.

Figure 1: Inventory procedure in Topology 2

Figure 2 depicts the basic communication among AIoT devices, UE Reader, gNB, and A-IoT CN node for the Command procedure.

Figure 2: Command procedure in Topology 2

Proposal 1: Capture the above A-IoT Command and Inventory procedures for Topology 2.

The Rel-19 introduced NGAP procedures can be reused to support Topology 2 RRC-based solution, and may also require UE reader related information:

- Add a cell ID list, TAI list, and/or UE Reader list to Requested Service Area Information IE to support UE reader-based inventory.

- Inventory Report Transfer IE may include a UE Reader ID.

Proposal 2: Reuse existing NGAP signalings to support Inventory and Command in Topology 2. Potential enhancements may include introducing UE reader ID for inventory purposes.

2.2 UE reader authorization

Only if the UE is authorized to perform the A-IoT service, the UE can communicate with the A-IoT device and be configured with AIoT radio resources controlled by the gNB.

Similar to SL Relay and other operations, we can include the AIoT UE Reader Authorized IE (Indicates whether the UE is authorized forAIoT services) in the F1AP/XnAP/NGAP messages:

- NGAP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE

- XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE

- F1AP: UE CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST

Proposal 3: Include the AIoT UE Reader Authorized IE in the F1AP/XnAP/NGAP messages.

- NGAP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST. PATH SWITCH REQUEST ACKNOWLEDGE

- XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE

- F1AP: UE CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST

2.3 Inter-gNB coordination

For following objectives:

- Inter-gNB resource coordination is not specified.

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

The choice of UE reader depends on the gNB. As inter-gNB resource coordination is not specified, once the UE preform handover, A-IoT service provided by the UE under the source gNB should stop. After handover successfully, the target gNB can reuse it as a UE reader. Therefore, except AIoT UE Reader Authorized IE, there is no need to transmit other A-IoT related signalling exchanging between gNBs.

Proposal 4: Except UE Reader authentication information, there is no need to transmit other A-IoT related signalling exchanging between gNBs.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Capture the above A-IoT Command and Inventory procedures for Topology 2.

Proposal 2: Reuse existing NGAP signalings to support Inventory and Command in Topology 2. Potential enhancements may include introducing UE reader ID for inventory purposes.

Proposal 3: Include the AIoT UE Reader Authorized IE in the F1AP/XnAP/NGAP messages.

- NGAP: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST. PATH SWITCH REQUEST ACKNOWLEDGE

- XnAP: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE

- F1AP: UE CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST

Proposal 4: Except UE Reader authentication information, there is no need to transmit other A-IoT related signalling exchanging between gNBs.

- References

[1] 	RP-252894, Solutions for Ambient IoT (Internet of Things) in NR Phase 2.

[2] 	3GPP TR38.769, Study on solutions for ambient IoT (Internet of Things).