3GPP TSG-RAN WG3 Meeting #129-bis	R3-256637

# Prague, CZ, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Positioning activation and deactivation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Samsung, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, Area-specific SRS Configuration Activation Procedure is specified in 7.14.4. However, on activation of semi-persistent UL-SRS, it is not clear which node could decide the SRS resource set to be activated. The legacy way is that LMF decides the SRS resource sets to be activated and triggers Positioning Activation Request procedure towards the serving gNB. Thus, following the same logic, the activation of semi-persistent UL-SRS could be decided and triggered by the LMF via the Positioning Activation Request procedure. Similary, it’s not clear how the semi-persistent Positioning SRS is deactivated for the UE in RRC_INACTIVATE. The legacy way could also be reused, i.e. LMF triggers Positioning Deactivation procedure towards the last serving gNB with the SP SRS resource set(s) to be deactivated. It is beneficial to clearly specify the overall procedure of the SRS deactivation procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Area-specific SRS Configuration Activation Procedure, clarify how the semi-persistent UL-SRS is activated for the UE. Introduce Semi-persistent Positioning SRS Deactivation Procedure in section 7.14.x. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact from protocol and functional point of view. The impact can be considered isolated because it only impacts the activation/deactivation of the SP SRS transmission for positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functionality of Area-specific SRS configuration activation and deactivation is not complete.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.4, 7.14.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.455 CR0198 TS 38.423 CR1528 TS 38.473 CR1592 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

7.14.4	Area-specific SRS Configuration Activation Procedure

Figure 7.14.4-1 shows the Area-specific SRS Configuration Activation procedure.

Figure 7.14.4-1: Area-specific SRS Configuration Activation Procedure

0.	The UE in RRC_INACTIVE state is (pre-)configured with one or more area-specific SRS for positioning configurations as specified in clause 7.14.2 and receives a request from upper layers to activate an area-specific (pre-)configured SRS for positioning when the UE is camped on one of the cells indicated in the validity area(s) (e.g., when a configured periodic or triggered event is detected).

1.	The UE sends a RRC Resume Request message with the resume cause "srs-PosConfigOrActivationReq" to request activation of the area-specific (pre-)configured SRS.

2.	The receiving gNB which receives the request from the UE triggers the Retrieve UE Context procedure towards the last serving gNB and includes the area-specific (pre-)configured SRS activation request in the Retrieve UE Context Request message.

3.	The last serving gNB sends a NRPPa Positioning Information Update message to the LMF including the Cell ID of the receiving gNB where the UE resumes from, together with an indication to notify the LMF that the area-specific (pre-)configured SRS is activated in the UE.

4. 	In case of the semi-persistent UL SRS, the LMF initiates Positioning Activation Request procedure towards the gNB, which includes the UL semi-perstent SRS resource set to be activated and may also include the spatial relation for the semi-persistent UL SRS resource to be activated.

5.	The last serving gNB relocates the full UE context to the receiving gNB and includes the (pre-)configured SRS for positioning configuration(s), the Routing ID and the NRPPa Transaction ID in the Retrieve UE Context Response message. The message may also include the SRS Resource Set ID(s) to be activated as received in the step 4.

NOTE:	If the last serving gNB decides not to relocate the full UE context, the last serving gNB provides a Retrieve UE Context Failure message to the receiving gNB, including a RRC Release message with the area-specific (pre-)configured SRS for positioning configuration and steps 5, 6, and 8 below are not performed. For semi-persistent UL SRS, the last serving gNB provides the SRS Resource Set ID(s) to be activated in the Retrieve UE Context Failure message, as received in the NRPPa Positioning Activation Request message.

6. The last serving gNB sends the NRPPa Positioning Activation Response message to the LMF.

NOTE: 	Step 4 and 6 are only applicable for activation of semi-persistent UL-SRS.

7.	The receiving gNB triggers the Path Switch Request procedure towards the AMF.

8.	The AMF responds with the Path Switch Request Acknowledge.

9.	The receiving gNB activates the area-specific (pre-)configured SRS for positioning and releases the UE to RRC_INACTIVE state.

10.	The receiving gNB indicates the last serving gNB to release the UE context.

Next Change

7.14.x	Semi-persistent Positioning SRS Deactivation Procedure

Figure 7.14.x-1 shows the Semi-persistent Positioning SRS Deactivation Procedure.

Figure 7.14.x-1: Semi-persistent Positioning SRS Deactivation Procedure

0.	The semi-persistent Positioning SRS is activated for a UE in RRC_INACTIVE state.

1.	LMF sends a Positioning Deactivation message to the last serving gNB, which indicates the UL SRS resource set(s) to be deactivated or release all the related resources.

2. The last serving gNB pages the UE in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s).

3. The Receiving gNB sends a Paging message to the UE in NR Uu.

4. The UE sends a RRC Resume Request message towards the Receiving gNB.

5.	The receiving gNB triggers the Retrieve UE Context procedure towards the last serving gNB.

6.	The last serving gNB responds with Retrieve UE Context Response/Failure message, which includes the UL SRS resource set(s) to be deactivated.

7. The receiving gNB deactivates the semi-persistent SRS for positioning and releases the UE to RRC_INACTIVE state.

END OF CHANGES