3GPP TSG-RAN WG3 #129bis                                                 R3-256657

Prague, CZ, 13th – 17th Oct, 2025

Agenda Item:	11.2.2

Source:	CATT

Title:	Discussion on SON enhancement for conditional intra-CU LTM

Document for:	Discussion

Introduction

According to the WI [1], the following objective is captured:

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

Here we provide some analysis on MRO enhancement for conditional intra-CU LTM.

Discussion

In release 19, MRO enhancement for intra-CU LTM is specified to support following objectives:

- Failure type detection and optimization;

- Beam optimization;

- TA optimization;

For the first bullet, it is based on RLF Report to optimize handover triggering point and candidate cell selection. As a legacy solution, in release 20, MRO enhancement for conditional intra-CU LTM should take it as baseline.

For the other two bullets, RAN3 introduce network solution to perform MRO on beam and TA for intra-CU LTM because suitable beam and valid TA can improve LTM performance. For the same reason, optimization on beam and TA should also be discussed in case of conditional intra-CU LTM.

Proposal 1: It is proposed to take R19 MRO for intra-CU LTM as the baseline for R20 MRO for conditional intra-CU LTM. The work may include:

- Legacy MRO solution based on RLF Report;

- Beam optimization and TA optimization, network based or UE based solution.

For the differences between conditional intra-CU LTM and legacy intra-CU LTM, it is the same as CHO and legacy handover. In R17, MRO for CHO is supported and following information is recorded in RLF report based on MRO for legacy handover.

- CHO configuration, for example, candidate cell list and execution condition;

- The fulfilled condition;

- Time info. For example, Time elapsed between CHO execution until the first HOF/RLF:

In case of conditional intra-CU LTM, the following information should also be recorded similar as the solution of MRO for CHO:

- LTM configuration, for example, LTM candidate cell list and execution condition;

- The fulfilled LTM condition;

- Time info. For example, Time elapsed between LTM execution until the first HOF/RLF:

For bullet 1, in case of LTM, network based solution is used to record LTM configuration, and the following agreement has been achieved in RAN2.

Agreements - We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases.

As for conditional intra-CU LTM, LTM configuration should also be saved in network.

Proposal 2: It is proposed to use network based solution to save conditional LTM configuration. For example, LTM candidate cell list and execution condition.

For the other two bullets, only UE is aware of the information, so UE based solution is necessary.

Proposal 3: It is proposed to include the fulfilled LTM condition and time information in RLF Report.

In R19 mobility, L3 triggered LTM is supported and l3-Conditions/l1-Conditions IE is included ltm-ExecutionCondition to indicate UE how to trigger LTM. The information is necessary for MRO to decide whether to optimize L3 conditional or L1 conditional configuration. It can be also recorded by network as a kind of LTM configuration, no need to report by UE.

Proposal 4: It is proposed to use network based solution to identify conditional L1 or conditional L3 LTM.

In R19 mobility, CSI-RS configuration is supported. CSI-RS configuration should be considered in R20 MRO. For example, RAN3 considers CSI-RS level target selection.

Proposal 5: It is proposed to consider CSI-RS level target selection for intra-CU conditional LTM.

For conditional intra-CU LTM，new TAT is introduced in EarlyUL-SyncConfig IE for UE to decide whether performing RACH-less access. If TAT is un-optimal, RACH-less access will fail. Therefore, For TA optimization in case of conditional intra-CU LTM, optimization on TAT is necessary.

Proposal 6: It is proposed to optimize TAT to increase success rate of RACH-less LTM.

As for sub-optimal successful LTM event, SHR may be enhanced for analysis of near failure handover. In release 19, SHR is not discussed in RAN3. For the time reason, we propose to discuss successful LTM after the real LTM failure.

Proposal 7: It is proposed to prioritize MRO for conditional intra-CU LTM failure over successful case.

In release 19, Ping-Pong issue in case of intra-CU LTM has been discussed. The solution is CU detects LTM Ping-Pong occurs and then inform DU to solve the issue. In release 20, Ping-Pong issue should be considered in conditional intra-CU scenarios. For conditional LTM, CU is also aware of the UE stay cell and stay time, so, the solution for R19 intra-CU LTM could be reused.

Proposal 8: It is proposed to reuse the solution on Ping-Pong issue of R19 intra-CU LTM for that of R20 conditional intra-CU LTM.

Conclusions

Proposal 1: It is proposed to take R19 MRO for intra-CU LTM as the baseline for R20 MRO for conditional intra-CU LTM. The work may include:

- Legacy MRO solution based on RLF Report;

- Beam optimization and TA optimization, network based or UE based solution.

Proposal 2: It is proposed to use network based solution to save conditional LTM configuration. For example, LTM candidate cell list and execution condition.

Proposal 3: It is proposed to include the fulfilled LTM condition and time information in RLF Report.

Proposal 4: It is proposed to use network based solution to identify conditional L1 or conditional L3 LTM.

Proposal 5: It is proposed to consider CSI-RS level target selection for intra-CU conditional LTM.

Proposal 6: It is proposed to optimize TAT to increase success rate of RACH-less LTM.

Proposal 7: It is proposed to prioritize MRO for conditional intra-CU LTM failure over successful case.

Proposal 8: It is proposed to reuse the solution on Ping-Pong issue of R19 intra-CU LTM for that of R20 conditional intra-CU LTM.

Reference

[1] RP-251869 “Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR Phase 5” 3GPP TSG RAN Meeting #108