3GPP TSG-RAN WG3 Meeting #129-bis	R3-256682

Prague, Czech Republic, 13th - 17th Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1288 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Specifying procedure texts for the Core Network Assistance Information for RRC INACTIVE IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedure texts for several optional IEs (e.g., the Extended UE Identity Index Value IE) contained in the Core Network Assistance Information for RRC INACTIVE IE are missing though some other optional IEs are specified. Note that the Core Network Assistance Information for RRC INACTIVE IE could be used also by the ng-eNB for RRC inactive state as specified in TS 36.300. However, only TS 38.300 is referred in general while the E-UTRA Paging eDRX Information IE should refer to TS 36.300. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the procedure texts for several optional IEs in the Core Network Assistance Information for RRC INACTIVE IE. - Add reference to TS 36.300 for the E-UTRA Paging eDRX Information IE when the NG-RAN node is an ng-eNB. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it specifies the procedure texts for the included optional IEs contained in the Core Network Assistance Information for RRC INACTIVE IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing NG-RAN behaivors for optional IEs contained in the Core Network Assistance Information for RRC INACTIVE IE. No reference to the E-UTRA Paging eDRX Information IE for the ng-eNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.4.2, 8.4.2.2, 8.4.4.2, 9.3.1.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version: R3-255232 Rev1: R3-256682 Update to Rel-19 CR. Update against the latest specification. |  |  |  |  |  |  |  |  |

Change Begins

8.3.1	Initial Context Setup

8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE, and RAN paging, if any, for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. In particular:

-	if the UE Specific DRX IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for RAN paging.

-	if the MICO Mode Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, consider that the UE is configured with MICO mode by the AMF as specified in TS 23.501 [9].

-	if the Expected UE Behaviour IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it to assist the UE RRC state transition decision.

-	if the E-UTRA Paging eDRX Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, take it into account to configure the RAN Paging when the UE is in RRC_INACTIVE as specified in TS 36.300 [17].

-	if the Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for RAN Paging.

-	if the UE Radio Capability for Paging IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, check and update it with all the RAT(s) the UE supports in the serving PLMN, and use it for subsequent RAN paging.

-	if the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE.

-	if the NR Paging eDRX Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, take it into account to configure the RAN Paging when the UE is in RRC_INACTIVE as described in TS 38.300 [8].

-	if the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8].

-	if the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the Hashed UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it to determine the start point of PTW.

-	if the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10].

-	if the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the Further Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it as specified in TS 38.304 [12].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.3.4	UE Context Modification

8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE context. The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, replace the previously provided Core Network Assistance Information for RRC INACTIVE and use it for the RRC_INACTIVE state decision and RNA configuration for the UE, and RAN paging, if any, for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. In particular:

-	if the UE Specific DRX IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for RAN paging.

-	if the MICO Mode Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, consider that the UE is configured with MICO mode by the AMF as specified in TS 23.501 [9].

-	if the Expected UE Behaviour IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it to assist the UE RRC state transition decision.

-	if the E-UTRA Paging eDRX Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, take it into account to configure the RAN Paging when the UE is in RRC_INACTIVE as specified in TS 36.300 [17].

-	if the Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for RAN Paging.

-	if the UE Radio Capability for Paging IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for subsequent RAN paging.

-	if the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE.

-	if the NR Paging eDRX Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, take it into account to configure the RAN Paging when the UE is in RRC_INACTIVE as described in TS 38.300 [8].

-	if the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8].

-	if the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the Hashed UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it to determine the start point of PTW.

-	if the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10].

-	if the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the Further Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it as specified in TS 38.304 [12].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE, and RAN paging, if any, for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. In particular:

-	if the UE Specific DRX IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for RAN paging.

-	if the MICO Mode Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, consider that the UE is configured with MICO mode by the AMF as specified in TS 23.501 [9].

-	if the Expected UE Behaviour IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it to assist the UE RRC state transition decision.

-	if the E-UTRA Paging eDRX Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, take it into account to configure the RAN Paging when the UE is in RRC_INACTIVE as specified in TS 36.300 [17].

-	if the Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for RAN Paging.

-	if the UE Radio Capability for Paging IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for subsequent RAN paging.

-	if the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE.

-	if the NR Paging eDRX Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, take it into account to configure the RAN Paging when the UE is in RRC_INACTIVE as described in TS 38.300 [8].

-	if the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8].

-	if the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the Hashed UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it to determine the start point of PTW.

-	if the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10].

-	if the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the Further Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it as specified in TS 38.304 [12].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

8.4.4	Path Switch Request

8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE, and RAN paging, if any, for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. In particular:

-	if the UE Specific DRX IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for RAN paging.

-	if the MICO Mode Indication IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, consider that the UE is configured with MICO mode by the AMF as specified in TS 23.501 [9].

-	if the Expected UE Behaviour IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it to assist the UE RRC state transition decision.

-	if the E-UTRA Paging eDRX Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, take it into account to configure the RAN Paging when the UE is in RRC_INACTIVE as specified in TS 36.300 [17].

-	if the Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for RAN Paging.

-	if the UE Radio Capability for Paging IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it for subsequent RAN paging.

-	if the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE.

-	if the NR Paging eDRX Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, take it into account to configure the RAN Paging when the UE is in RRC_INACTIVE as described in TS 38.300 [8].

-	if the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the Hashed UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, use it to determine the start point of PTW.

-	if the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10].

-	if the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

-	if the Further Extended UE Identity Index Value IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it as specified in TS 38.304 [12].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.1.15	Core Network Assistance Information for RRC INACTIVE

This IE provides assistance information for RRC_INACTIVE configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| UE Specific DRX | O |  | Paging DRX 9.3.1.90 |  | - |  |
| Periodic Registration Update Timer | M |  | 9.3.3.24 |  | - |  |
| MICO Mode Indication | O |  | 9.3.1.23 |  | - |  |
| TAI List for RRC Inactive |  | 1 |  |  | - |  |
| >TAI List for RRC Inactive Item |  | 1..<maxnoofTAIforInactive> |  |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| Expected UE Behaviour | O |  | 9.3.1.93 |  | - |  |
| E-UTRA Paging eDRX Information | O |  | 9.3.1.154 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.3.52 | This IE is ignored if the Further Extended UE Identity Index Value IE is present. | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| MICO All PLMN | O |  | 9.3.1.194 |  | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.227 |  | YES | ignore |
| Paging Cause Indication for Voice Service | O |  | ENUMERATED (supported, …) | This IE indicates that the UE supports the feature of indication of paging cause for voice service. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.232 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.3.3.62 |  | YES | ignore |
| CN MT Communication Handling | O |  | ENUMERATED (supported, …) | This IE indicates the CN support of CN based MT communication handling. | YES | ignore |
| LP-WUSPS Assistance Information | O |  | 9.3.1.279 |  | YES | ignore |
| Further Extended UE Identity Index Value | O |  | 9.3.3.77 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforInactive | Maximum no. of TAIs for RRC Inactive. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Change Ends