3GPP TSG-RAN WG3 Meeting #129bis	R3-256695

Prague, CZ, 13th – 17th October 2025

Agenda item:	12.2.2

Source: 	Samsung

Title: 	Discussion on AI/ML-based intra-CU LTM

Document for:	Discussion and decision

Introduction

In Rel-19, AI/ML-based mobility optimization was standardized, which enabled the prediction of cell-based UE trajectory for L3 handover procedure. This paper provides discussion on AI/ML-based intra-CU LTM mechanism, with the spirit of further enhancing the mobility mechanisms by using the assistance of AI.

Discussion

When generally considering Handover or UE mobility in Connected state, RACH or UL synchronization would be a non-negligible process for the UE to successfully connect to the target cell, which is also an explicit step that pulls the UE back from a seamless handover without interruption. In LTM, even if RACH procedure is skipped, the early synchronization is needed for the UE to be synchronized to the selected target cell, during which the TA acquisition is an important step. If there is a method of omitting the RACH procedure or TA acquisition procedure for the UE with the assistance of AI/ML, we believe the handover procedures of UE, including LTM, could be simplified, and the interruption of services could be reduced accordingly.

If we focus on the intra-CU LTM in this sub-agenda, there are a few procedures defined in Rel-18 to allow the early TA acquisition for the purpose of early UL synchronization. Before the cell switch command is executed, UE may perform UL synchronization with LTM candidate cell(s), by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the gNB [1]. No matter it is in early sync or RACH procedure, the TA acquisition is a main reason which brings the interruption and raises the latency during handover.

Observation 1: The TA acquisition is one of the main factors that leads to the interruption during handover procedures.

In this paper, we would like to propose a third way for the UE to get the TA value —— by receiving the predicted TA value of candidate cells from the NG-RAN node. If the NG-RAN node, whether it’s a gNB-CU or source gNB-DU or candidate/target gNB-DU, as long as the the network node is able to provide the TA value(s) of candidate cell(s) to the UE, the TA acquisition procedure could be omitted. With this understanding, since the UE doesn’t need to send preamble to the target RAN node, there would be no need for the candidate cells to reserve early sync resources, and this would simplify the implementation of early synchronization. Last but not least, the prediction of TA value is not only applicable to intra-CU LTM, other handover procedures, generally speaking, would be optimized by using the predicted TA value to replace early TA acquisition or RACH process, so as to minimize the interruption during handover.

Based on the above discussion, the predicted TA has the following benefits:

-	Early TA acquisition procedure could be saved, by using the predicted TA value from RAN node to UE.

-	There is no need for the candidate DU to reserve early sync resources, since the UE does not need to send the preamble to acquire TA.

-	The predicted TA value is generally applicable to handover procedures, not only LTM.

Proposal 1: It is proposed to study the prediction of TA value for the use case of AI/ML-based intra-CU LTM.

Regarding how to predict the TA value for UE, the time difference between DL and UL is what matters to measure the TA, since TA is used to adjust the uplink frame timing relative to the downlink frame timing. Hence, it is reasonable that the information related to UE location or trajectory could be useful for the RAN node to derive the TA value. For example, if the UE is relatively far from the RAN node, the time difference between UL frame and DL frame would be relatively larger, because it takes more time for the UL signal to arrive at the RAN node. Similarly, the radio measurements would also reflect the distance between the UE and the RAN node, as it is understandable that a UE at the cell edge would suffer from bad radio quality, the TA of which would also be larger than those close to the centre of the cell or RAN node. With the same logic, we believe the frequency of a cell would also impact the TA value, e.g., the frequency of the source cell and candidate cell(s) can all be taken into account. Aside from the frequency of a cell, the UE mobility history supported in current specifications could be reused as the input for predicting TA values, which can be collected from the RAN node. To sum up, the following information can be collected as the input information for predicting TA value(s).

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available.

-	Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR.

From the RAN node:

-	Frequency information of the source cell and candidate cell(s)

-	UE mobility history

-	UE trajectory prediction

Proposal 2: The following information can be taken as the input information for predicting TA value(s):
From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available.

-	Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR.

From the RAN node:

-	Frequency information of the source cell and candidate cell(s)

-	UE mobility history

-	UE trajectory prediction

Regarding the output of AI/ML training and inference, aside from the TA value itself, we think it is also necessary to predict a validity time of the TA value. UE is not always static, and when UE moves to another location or another cell, the predicted TA value would not be valid any more. So, along with the UE mobility information and the UE trajectory prediction within a certain time, it is feasible to generate the validity time of the predicted TA value.

Proposal 3: The predicted TA value of candidate cell(s) and the corresponding validity time can be taken as the output of the AI/ML model training/inference.

For the intra-CU LTM, it shall also be discussed which node shall generate the prediction of TA value. Technically speaking, either the gNB-CU, source gNB-DU or candidate gNB-DU can generate the prediction of the TA value, if the input information is provided to the corresponding node with AI/ML model. However, with the guideline in the WID and the agenda, we should study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, which means the following principle is supposed to be taken as the baseline:

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

Under the above principle, we would agree that gNB-CU inference of predicted TA value(s) can be taken as a start point when we discuss the prediction of TA value, but this does not mean inference at the source gNB-DU or candidate gNB-DU shall be excluded at this stage.

For a gNB-CU to predict the TA value, the input information can be collected from the UE, source DU and candidate DU via direction connections, and the output shall be provided to the source gNB-DU sent to the UE via cell switch command.

There is also some value for the source gNB-DU or candidate gNB-DU to predict TA value(s).

If the source gNB-DU predicts the TA value(s), it can directly send the TA value of the selected target gNB-DU to the UE via the cell switch command, while some part of the input information may need to be transferred via F1AP to collect essential information from the gNB-CU or candidate gNB-DU(s).

Besides, for a candidate DU, there is also justification for the candidate DU(s) to become the node who generates prediction. First of all, in the legacy mechanism for early TA acquisition, it is the candidate/target DU to measure the TA value and provides it to the UE (other than the UE itself is configured to measure the TA value), then it should be natural for the candidate DU to generate the predicted TA value using the assistance of AI/ML. Last but not least, the candidate DU or target DU itself knows the most about the cell information of itself, and could perform an accurate model inference to the most extent. In the case of candidate/target gNB-DU makes the prediction, some information about the UE may need to be transferred from the gNB-CU to the gNB-DU, which can be further studied.

It should be noted that, this paper mainly focuses on the case of intra-CU inter-DU LTM, because for intra-DU LTM, it can be understood that the source DU and the candidate DU is the same one.

Proposal 4: The gNB-CU inference of the predicted value(s) could be taken as a start point, while it can also be further studied how the source gNB-DU or candidate/target DU generate the inference of TA value(s), which means the gNB-DU model inference can also be considered.

Based on the discussion above, the potential standard impact from network perspective is mainly on F1AP, depending on which node makes the inference about TA values. We summarize the potential F1AP impact for the input and output of AI/ML model training and inference in the following bullets based on our analysis above:

-	gNB-CU inference: the predicted TA value(s) from the gNB-CU to the source gNB-DU.

-	source gNB-DU inference: UE location information, measured/predicted UE trajectory, frequency information of the candidate DU(s) from the source CU to the source DU.

-	candidate gNB-DU inference: UE location information, measured/predicted UE trajectory from the gNB-CU to the candidate DU, predicted TA information to the source gNB-DU via the gNB-CU.

As for AI/ML-based UE performance feedback, generally speaking, it should be the target DU to provide how the UE performs using the predicted TA value to the target DU, and which information can be considered as UE performance feedback can be further discussed.

Regarding the specific F1 signalling to support the above information transmission to fulfil the AI/ML-based data collection initiation and reporting, we think it can be further considered in the WI phase, whether to reuse the UE Context management procedures and CU-DU/DU-CU TA Information Transfer procedures or to define new F1AP procedures for the data collection initiation and reporting for AI/ML-based intra-CU LTM.

Based on the above discussion, we have the following proposal for the standard impact to be captured in the TR:

Proposal 5: The potential F1AP standard impact is proposed to the captured as:

-	New signalling procedure or existing procedure to provide the predicted TA value(s) to the source gNB-CU, if the AI/ML model inference is located at the gNB-CU.

-	New signalling procedure or existing procedure to retrieve the input information from the gNB-CU to the gNB-DU, if the AI/ML model inference is located the source or candidate gNB-DU.

-	New signalling procedure or existing procedure to retrieve feedback information via F1AP.

Conclusion

In this paper, the following proposals are provided:

Observation 1: The TA acquisition is one of the main factors that leads to the interruption during handover procedures.

Proposal 1: It is proposed to study the prediction of TA value for the use case of AI/ML-based intra-CU LTM.

Proposal 2: The following information can be taken as the input information for predicting TA value(s):
From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available.

-	Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR.

From the RAN node:

-	Frequency information of the source cell and candidate cell(s)

-	UE mobility history

-	UE trajectory prediction

Proposal 3: The predicted TA value of candidate cell(s) and the corresponding validity time can be taken as the output of the AI/ML model training/inference.

Proposal 4: The gNB-CU inference of the predicted value(s) could be taken as a start point, while it can also be further studied how the source gNB-DU or candidate/target DU generate the inference of TA value(s), which means the gNB-DU model inference can also be considered.

Proposal 5: The potential F1AP standard impact is proposed to the captured as:

-	New signalling procedure or existing procedure to provide the predicted TA value(s) to the source gNB-CU, if the AI/ML model inference is located at the gNB-CU.

-	New signalling procedure or existing procedure to retrieve the input information from the gNB-CU to the gNB-DU, if the AI/ML model inference is located the source or candidate gNB-DU.

-	New signalling procedure or existing procedure to retrieve feedback information via F1AP.

References

[1] TS 38.300 v18.6.0

TP to the TR 38.745

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

4.2	AI/ML assisted Intra-CU LTM

4.2.1	Use case description

LTM is a procedure in which a gNB receives L1 or L3 measurement report(s) from a UE, and the gNB may change UE serving cell by a cell switch command signalled via a MAC CE on the basis of the received measurement report(s), as specified in TS38.300 [x1]. Intra-CU LTM is the case when the UE moves within the same gNB-DU (intra-gNB-DU) or from one gNB-DU to another gNB-DU within the same gNB-CU (inter-gNB-DU) during NR operation for LTM, as described in TS 38.401 [x2].

During intra-CU LTM, TA acquisition is an important step for the UE to perform UL synchronization with the gNB, whether it is via a RACH procedure or an early synchronization procedure without RACH. Before the cell switch command is executed, UE may perform UL synchronization with LTM candidate cell(s), by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the gNB. No matter it is in early sync or RACH procedure, the TA acquisition is a main reason which brings the interruption and raises the latency during handover.

With the assistance of the AI/ML, the gNB itself could generate predicted TA value(s) of the candidate cell(s), without the need for the UE to send the preamble for the acquisition of TA value(s). The prediction of TA value(s) of the candidate cell(s) could enable the network provide TA value of the target cell in the cell switch command, without the procedures for the UE to trigger TA acquisition in advance. The interruption and latency during the intra-CU LTM could be reduced by using AI/ML.

Editor Note: Capture the description of use case

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>