3GPP TSG-RAN WG3 Meeting #129Bis                              R3-256747 Prague, Czech, Oct 13 – 17, 2025

Agenda Item:	8.1

Source:	ZTE corporation

Title:	Temporary suspension of trace production

Document for:	Discussion

Introduction

In the incoming LS[R3-256519/S5-253909], SA5 ask RAN3 to evaluate the feasibility of adding administrativeState and operationalState attributes to the TraceJob IOC.This contribution provide our views on this aspect.

Discussion

The main intention of the LS from SA5 is asking RAN3 to evaluate the impact of introduce pause and resume of Trace procedure.The trace procedure in RAN3 involved MDT and Trace only function.

Impact to MDT

Based on the Minimization of Drive Tests (MDT) procedures, Pause/Resume operations are feasible for Immediate MDT but infeasible for Logged MDT. This distinction entirely on the mobility state of the User Equipment (UE) during the measurement activity and the corresponding signaling capabilities of the core and access networks.

1. Feasibility of Pause and Resume for Immediate MDT

Immediate MDT measurements are performed by the UE when it is in the CONNECTED state (RRC_CONNECTED). Since the UE maintains an active signaling connection with the network (eNB/gNB), the network retains continuous control over the MDT configuration.

The ability to dynamically pause and resume corresponds directly to the established network procedures for activation and deactivation of trace sessions:

Network Control in Connected Mode: Immediate MDT configuration relies on extensions to existing Radio Resource Control (RRC) measurement procedures. In the case of Immediate MDT, the network continuously evaluates the selection conditions for activating and deactivating the MDT measurements.

Activation/Resume Mechanism: The network (Core Network, CN, or Access Network, AN) initiates the measurement configuration. For example, in 5GC, the AMF initiates the process by sending a TRACE START message to the NG-RAN node (gNB/eNB), which subsequently configures the corresponding MDT RRC measurements at the UE.

Deactivation/Pause Mechanism: The deactivation mechanism, which functionally acts as a pause or stop command, is explicitly defined for connected UEs.

2. Infeasibility of Pause and Resume for Logged MDT

Logged MDT measurements are characterized by the UE performing measurement logging primarily when it is in IDLE mode or INACTIVE state. This fundamental state transition makes dynamic signaling control (like Pause/Resume) from the network infeasible.

When the UE is in RRC_IDLE or RRC_INACTIVE, it does not maintain an active dedicated signaling connection (RRC context) with the RAN.

Observation 1: Pause and Resume function is feasible for immediate MDT while not feasible for logged MDT.

Impact to Trace function

Implementing Pause and Resume functionality is feasible for General Trace functionality,it is because general tracing relies entirely on Network Element (NE) control via active signaling connections.

Trace functionality aims to provide detailed information at the call level on one or more specific mobiles by capturing events and signaling messages across various Network Elements (NEs). This process primarily involves logging signaling information at the core network (CN) and the Radio Access Network (RAN). Therefore traditional "Trace Only" (Job Type TRACE_ONLY) involves logging internal network activities, contrasting with MDT which often relies on the UE executing and storing measurements

The feasibility of introducing explicit Pause and Resume states for a general Trace job stems from the fact that the entire life cycle of a Trace session is driven by explicit signaling activation and signaling deactivation commands transmitted between network nodes.Due to the network components (both CN and RAN) are mandated to stop (Deactivate) and start (Activate) trace sessions based on direct signaling commands associated with a UE context, implementing an administrative Pause (using deactivation without deletion) and Resume (using activation with preserved configuration) is technically feasible for General Trace functionality.

Observation 2: Pause and Resume function is feasible for traditional Trace only.

Impact to interfaces

The introduction of Pause and Resume functionality for the TraceJob Information Object Class (IOC), driven by the need for administrativeState and operationalState attributes (as outlined in the premise), requires specific, yet generally manageable, technical adjustments across 5G RAN control plane protocols: NGAP (38.413), XnAP (38.423), F1AP (38.473), and E1AP (37.483).

The primary focus of these changes is to ensure that when a trace session is "paused," the network elements (gNB, gNB-CU, gNB-DU) preserve the complete configuration parameters instead of discarding them, allowing for instantaneous "resume" (re-activation) using the existing activation signaling.

NGAP is the critical interface responsible for signaling Trace and MDT activation/deactivation from the Core Network (CN) to the NG-RAN node (gNB-CU-CP). RAN3 may need to Refine Deactivation Semantics or introduce new procedure for trace pause and resume.

Same impact apply to other specifications.

Observation 3: The introduction of Trace Pause and Resume functionality is not a minor adjustment, as it necessitates affecting NGAP (TS 38.413), XnAP (TS 38.423), and F1AP (TS 38.473).

Conclusion

In this contribution, we provide the following observations and proposal:

Observation 1: Pause and Resume function is feasible for immediate MDT while not feasible for logged MDT.

Observation 2: Pause and Resume function is feasible for traditional Trace only.

Observation 3: The introduction of Trace Pause and Resume functionality is not a minor adjustment, as it necessitates affecting NGAP (TS 38.413), XnAP (TS 38.423), and F1AP (TS 38.473).

Proposal: RAN3 to send reply LS in [R3-256748].

Reference

- RP-251881

- RP-252870

- RP-252909