3GPP TSG-RAN WG3 Meeting # 129bis	R3-256763

# Prague, Czech, 13 – 17, October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1333 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on GW Context Release Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NGAP UE Context Release Request message introduces a GW Context Release Indication, which is used for the NR Femto GW to release a UE’s context when the UE’s Xn handover is performed successfully from the NR Femto sending the context release indication to it. 1. The description of the context release indication IE states that this IE is used to indicate the AMF to release the UE context, which is not aligned with the agreement. 2. Criticality for the context release indication IE in tabular is “reject”, but it is “ignore” is asn.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change the description of the context release indication IE to avoid mention AMF. 2. Change the criticality of the context release indication IE from “reject” to “ignore” in the tabular. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because it affects only the description of the GW Context Release Indication IE and the criticality in tabular). The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of IE is misleading, or the tabular is not aligned with asn.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.4, 9.3.1.274 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.4	UE CONTEXT RELEASE REQUEST

This message is sent by the NG-RAN node to request the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| PDU Session Resource List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| GW Context Release Indication | O |  | 9.3.1.274 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

9.3.1.274	GW Context Release Indication

This information element is set by the gNB to provide an indication that any resources related to the signalled NG UE context can be released(see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GW Context Release Indication | M |  | ENUMERATED (true, …) | This IE indicates that the gNB has successfully performed an Xn HO for the UE to a target gNB. |

<<<<<<<<<<<<<<<<<<<< End of changes >>>>>>>>>>>>>>>>>>>>