3GPP TSG-RAN WG3 Meeting #129bis	R3-256826

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	13.2

Source: 	Qualcomm Incorporated

Title: 	Network Architecture and Protocol Aspects for NR Sensing Support

Document for:		Discussion and Decision

# 1.	Introduction

At RAN#108, a new (RAN1-led) Study Item has been approved to study "Integrated Sensing And Communication (ISAC) for NR", which has been slightly revised at RAN#109 [1]. The objectives for this study are shown below.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signalling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

For the RAN3 objectives, the study is expected to be completed at RAN#112 (June 2026; which gives five RAN3 meetings until RAN3#132). A parallel study is ongoing in SA2 [2], which is expected to be completed at SA#110 (December 2025). The RAN1 aspects are expected to be completed at RAN1#124 (February 2026 (RAN#111)).

In this contribution, we discuss the network architecture to support gNB sensing and the general protocol aspects.

# 2.	SA2 Studies of the System Architecture

The SA2 study [2] include various Work Tasks, where WT-1 (architecture and function enhancements to support sensing) is (at least initially) most relevant to this RAN3 study:

- WT-1: Architecture and function enhancements to support sensing. - WT-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer. - WT-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities. - WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation. - WT-5: Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting. - WT-6: Configuration parameters/policy provisioning for the support of ISAC services.

To support the sensing operation, SA2 consider the introduction of a new "Sensing Function (SF)" to support the sensing service [3]. A "Sensing Entity (SE)" in SA2 refers to a sensing transmitter or to a sensing receiver [3], which is in our case a gNB. The exact functionalities of this "Sensing Function (SF)" are not yet all defined, but this function may control the overall sensing operation and may also calculate the sensing results (e.g., similar to a "Location Management Function (LMF)" in positioning and location services (LCS) [4]). E.g., the "Sensing Function (SF)" may select the sensing entities (gNBs) and trigger necessary procedures to obtain the "sensing results" (see also [5]).

On a high-level, SA2 consider two architecture options to provide support for sensing, as illustrated in Figure 1:

- There is a "direct connection/interface" between the SF and the gNB. This "direct connection" may be "N2-like" (1a) or SBI (1b).

- There is an "indirect connection/interface" between the SF and the gNB, where the communication between SF and gNB is via an AMF.

Figure 1: Illustration of Architecture Options.

Independent of the architecture options, a RAN3 application protocol (referred to as "Sensing Protocol" in the following) would be required to support the various sensing operations. This protocol should be end-to-end between the SF and the gNB, independent on whether it is routed via an AMF or not. If the transport is via an AMF (Option (2) above), the protocol stack could be the NG interface stack with SBI between SF and AMF, as illustrated in Figure 2. However, since the signalling between the SF and gNB is non-UE associated, an AMF involvement would not strictly be needed if the SF can be aware of available gNBs with sensing capability (e.g., NG-RAN node IDs/addresses, etc.).

Figure 2: Possible Protocol Layering for SF to gNB Signalling for Option 2.

For direct connectivity between gNB and SF (Option (1)), the transport layers for the "Sensing Protocol" would need to be defined, however, the actual "Sensing Protocol" could be independent of the transport. If the "direct interface" is a Service-Base-Interface (SBI), a possible protocol stack is illustrated in Figure 3.

Figure 3: Possible Protocol Layering for SF to gNB Signalling for Option 1b (SBI).

If the "direct interface" is "NG-like", a possible protocol stack is illustrated in Figure 4. If NGAP is used for the Nx interface, only a subset of NGAP would be required at the SF side (e.g., setup procedures and non-UE associated UL/DL transport procedures). However, the NG Application Protocol (NGAP) currently supports the functions of the NG interface [6] and therefore, the scope of NGAP would require clarification if re-used over Nx interface or a new NxAP need to be defined with the required procedures.

Figure 4: Possible Protocol Layering for SF to gNB Signalling for Option 1a.

It can be observed from Figures 2 and 3, that the protocol layering at the SF side for Option 1b and 2 would be the same as for an LMF to support location services [4]. Therefore, a co-location of the SF and LMF would be easily possible to support combined positioning and sensing. For example, if mobile TRPs can be used as sensors, their location may need to be determined first or in parallel.

In SA2 there are also considerations on separating "sensing control signalling" and "sensing measurement/data reporting signalling" [3]. In any case, a "Sensing Protocol" could be studied independent on the architecture options under consideration in SA2.

Observation 1:	It is assumed that SA2 will define the system architecture to support sensing.

Observation 2:	An end-to-end "Sensing Protocol" to support the various sensing operations could be defined independent on the architecture options under consideration in SA2.

Observation 3:	The lower/transport layers for the "Sensing Protocol" depend on the architecture option selected by SA2.

Observation 4:	The protocol layering at the Sensing Function (SF) for Option 1b and Option 2 could be the same as for an LMF to support location services [4]. Therefore, a co-location of the SF and LMF would be easily possible to support combined positioning and sensing.

Proposal 1:	RAN3 study an end-to-end (SF–gNB) "Sensing Protocol", independent on the lower transport layers and architecture option.

# 3.	Sensing Data/Results

The Stage 1 service requirements for Integrated Sensing and Communication TS 22.137 [7] defines various performance metrics for the sensing capabilities in 5G networks:

| Metric | Description |
| --- | --- |
| Positioning Accuracy | Horizontal and vertical accuracy in meters. |
| Velocity Accuracy | Horizontal and vertical velocity estimation accuracy in m/s. |
| Confidence Level | The probability that the sensing estimate (e.g., position or velocity) falls within the specified limits, e.g., 95%, 99%. |
| Sensing Resolution | Includes range resolution and velocity resolution. |
| Missed detection/false alarm probability | Percentage of targets not detected when they should be/the percentage of detections that are incorrect (i.e., no actual target present). |
| Latency ("Response Time") | Time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result. |
| Refresh Rate ("reporting interval") | Inverse of the time elapsed between two successive sensing results. |

Performance requirements for various use cases using the metrics above are defined in [7]. Apart from the missed detection/false alarm probability, the sensing performance indicators are the same as for positioning (incl. sidelink positioning) and location services [4].

Observation 5:	The key performance metrics defining the service requirements for Integrated Sensing and Communication are similar to positioning/location services (e.g., positioning/velocity accuracy, etc.).

One aspect which may affect the selection of the architecture and "Sensing Protocol" transport is the type of "data" (data volume) which need to be exchanged between SF and gNB (i.e., the "Sensing Data/Measurement Report"). This is currently being studied in RAN1, and there appears the following general options:

(1)	"Sensing Measurements", i.e., (raw) physical layer measurements representing range, angle, doppler (RAD), etc. measurements. These measurements may be similar or the same as for positioning (incl. sidelink positioning) such as RTT, AoA, range and direction, etc.

(2)		Position/velocity of detected object(s), incl. a "point cloud" (i.e., a collection of points in a three-dimensional coordinate system used to represent the shape and surface of the detected object).

Analogous to positioning [4], the above general options could be mapped to "gNB-assisted" and "gNB-based", referring to the node that is responsible for determining the position/velocity of the object(s) (SF or gNB).

The specific measurement details reported by a gNB are subject to ongoing developments and conclusions within RAN1. For both "sensing measurement types" above, it is assumed that a final processing step is needed (either at SF or gNB). This processing step would interpret the "sensing data" and extract the meaningful sensing information, such as identifying and classifying the target object (e.g., distinguishing a specific UAV from other detected objects). This information is referred to as "Sensing Result" in TS 22.137 [7] and defined as: "Processed 3GPP sensing data requested by a service consumer". It is assumed that the details of the "Sensing Service Request/Result" will be defined by SA2.

Observation 6:	Two general "sensing measurement types" are under consideration in RAN1:

(1)	"Sensing Measurements", i.e., (raw) physical layer measurements representing range, angle, Doppler (RAD), etc. measurements.

(2)	Position/velocity of detected object(s), incl. a "point cloud" (i.e., a collection of points in a three-dimensional coordinate system used to represent the shape and surface of the detected object).

Observation 7:	Precise information on the "data volume/size" to be exchanged between gNB and SF for both "sensing measurement types" in Observation 6 require further RAN1 input.

Observation 8:	Regardless of the general "sensing measurement type", it is assumed that a final processing step is needed, which would interpret the results and extract the meaningful sensing information, such as identifying and classifying the target object (e.g., distinguishing a specific UAV, etc.). This final processing step could be at the gNB or SF.

The "sensing measurement type" may impose distinct requirements on several system aspects. In particular, the two types may differ in terms of the volume and size of data exchanged between the gNB and SF. Additionally, they may impact the service request information required at the gNB, such as object classification parameters, Quality of Service (QoS) indicators, etc.

Proposal 2:	RAN3 should await further input from RAN1 regarding the expected data volume for the sensing measurement data. This information may impact e.g., the selection of appropriate transport for the "Sensing Protocol".

# 4.	RAN Architecture

The gNB architecture to support sensing could be the same as for positioning [4] by defining Transmission-Reception Points (TRPs) hosted by the gNB-DUs as illustrated in Figure 5. The TRPs may support Transmission Point (TP) functionality and/or Reception Point (RP) functionality. For bi-static sensing for example, the TP and RP functionality may not be collocated.

Figure 5: gNB Architecture.

The Nx or Nran or NG interface (dependent on the architecture option discussed in section 2) would terminate at the gNB-CU and the F1 interface supports the sensing operation between gNB-CU and gNB-DU/TRP, similar to positioning [4]. Note however, that the F1 interface may not need to replicate all the procedures to be defined over the Nx or Nran or NG interface like in F1AP for positioning [8], [9], but may just forward OCTET STRINGs with embedded "Sensing Protocol" PDUs to the appropriate gNB-DU/TRP, which is discussed in greater detail in [5]. A corresponding protocol layering is illustrated in Figure 6, where lower layers at gNB-CU and SF side depends on the architecture options discussed in section 2.

Figure 6: Possible Protocol Layering for SF to gNB-DU Signalling (see also [5]). The empty boxes depend on the architecture option discussed in section 2.

Observation 9:	The gNB architecture to support sensing could be the same as for positioning support by defining Transmission-Reception Points (TRPs) in the gNB-DUs to support sensing signal transmission and/or reception.

The RP component of the TRP within a gNB-DU is responsible for signal acquisition and the extraction of sensing data, as outlined in Observation 6. Regardless of the sensing measurement type – whether raw physical layer measurements (e.g., range, angle, Doppler, etc.) or higher-level object information (e.g., position, velocity, point cloud) – the generation of sensing data can be performed locally at the gNB-DU. The gNB-DU processes the reflected signals to derive essential parameters such as range, angle, and Doppler shift, and translates these into location/velocity of the object(s). This enables real-time object detection directly at the gNB-DU. The gNB-CU serves primarily as a routing entity, forwarding the Sensing Protocol PDUs to the appropriate gNB-DU, as further discussed in [5], possibily reducing the processing latency.

Proposal 3:	The gNB includes "Transmission-Reception Point (TRP)" functionality to support sensing signal transmission (TP) and/or reception (RP).

Proposal 4:	In case of a split-gNB, the gNB-DU determines the "sensing data" (raw sensing measurements or position/velocity of the object(s) (e.g. "point-cloud")).

Proposal 5:	The gNB-CU can be transparent to the "sensing signalling" and would only need to route the "sensing signalling" PDU to the desired gNB-DU/TRP.

# 5.	Summary

In this contribution, we discussed the network architecture to support gNB sensing and the general protocol aspects which need to be studied to enable the gNB sensing capability in 5G. The following Observations and Proposals were made.

System Architecture and General Protocol Aspects

Observation 1:	It is assumed that SA2 will define the system architecture to support sensing.

Observation 2:	An end-to-end "Sensing Protocol" to support the various sensing operations could be defined independent on the architecture options under consideration in SA2.

Observation 3:	The lower/transport layers for the "Sensing Protocol" depend on the architecture option selected by SA2.

Observation 4:	The protocol layering at the Sensing Function (SF) for Option 1b and Option 2 could be the same as for an LMF to support location services [4]. Therefore, a co-location of the SF and LMF would be easily possible to support combined positioning and sensing.

Proposal 1:	RAN3 study an end-to-end (SF–gNB) "Sensing Protocol", independent on the lower transport layers and architecture option.

Sensing Data and Results

Observation 5:	The key performance metrics defining the service requirements for Integrated Sensing and Communication are similar to positioning/location services (e.g., positioning/velocity accuracy, etc.).

Observation 6:	Two general "sensing measurement types" are under consideration in RAN1:

(1)	"Sensing Measurements", i.e., (raw) physical layer measurements representing range, angle, Doppler (RAD), etc. measurements.

(2)	Position/velocity of detected object(s), incl. a "point cloud" (i.e., a collection of points in a three-dimensional coordinate system used to represent the shape and surface of the detected object).

Observation 7:	Precise information on the "data volume/size" to be exchanged between gNB and SF for both "sensing measurement types" in Observation 6 require further RAN1 input.

Observation 8:	Regardless of the general "sensing measurement type", it is assumed that a final processing step is needed, which would interpret the results and extract the meaningful sensing information, such as identifying and classifying the target object (e.g., distinguishing a specific UAV, etc.). This final processing step could be at the gNB or SF.

Proposal 2:	RAN3 should await further input from RAN1 regarding the expected data volume for the sensing measurement data. This information may impact e.g., the selection of appropriate transport for the "Sensing Protocol".

RAN Architecture

Observation 9:	The gNB architecture to support sensing could be the same as for positioning support by defining Transmission-Reception Points (TRPs) in the gNB-DUs to support sensing signal transmission and/or reception.

Proposal 3:	The gNB includes "Transmission-Reception Point (TRP)" functionality to support sensing signal transmission (TP) and/or reception (RP).

Proposal 4:	In case of a split-gNB, the gNB-DU determines the "sensing data" (raw sensing measurements or position/velocity of the object(s) (e.g. "point-cloud")).

Proposal 5:	The gNB-CU can be transparent to the "sensing signalling" and would only need to route the "sensing signalling" PDU to the desired gNB-DU/TRP.

# References

[1]	RP-252819, "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[2]	SP-250401, "New SID on Study on Stage 2 for Integrated Sensing and Communication".

[3]	3GPP TR 23.700-14: "Study on Integrated Sensing and Communication; Stage 2".

[4]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[5] R3-256827, "Signalling and Procedures for NR Sensing Support", Qualcomm Incorporated.

[6]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[7]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[8]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[9]	3GPP TS 38.473: "F1 application protocol (F1AP)".