3GPP TSG-RAN WG3 Meeting #129bis	             R3-256847

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	10.2.2

Source:	CMCC

Title:	Initial consideration on RAN functions and logical architecture

Document for:	Discussion

# Introduction

A new study item [1] was agreed at RAN#108 that aims to develop a one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases, and requirements as defined during the RAN requirements study in TR38.914[2].

One of the objectives of the study is:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

One of the items in the above objective is the study of the overall RAN side architecture to support of various services and functionalities (e.g. AI/ML, sensing, etc). In this paper, we provide our initial consideration on RAN functions and logical architecture.

# Background

## Functions supported by 5G

Following are the functions supported in 5G RAN, specified in TS 38.300[3]:

The gNB and ng-eNB host the following functions: - Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling); - IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data; - Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE; - Routing of User Plane data towards UPF(s); - Routing of Control Plane information towards AMF; - Connection setup and release; - Scheduling and transmission of paging messages; - Scheduling and transmission of system broadcast information (originated from the AMF or OAM); - Measurement and measurement reporting configuration for mobility and scheduling; - Transport level packet marking in the uplink; - Session Management; - Support of Network Slicing; - QoS Flow management and mapping to data radio bearers; - Support of UEs in RRC_INACTIVE state; - Distribution function for NAS messages; - Radio access network sharing; - Dual Connectivity; - Tight interworking between NR and E-UTRA; - Maintain security and radio configuration for User Plane CIoT 5GS Optimisation, as defined in TS 23.501 [3] (ng-eNB only). NOTE 1: BL UE or UE in enhanced coverage is only supported by ng-eNB, see TS 36.300 [2]. NOTE 2: NB-IoT UE is only supported by ng-eNB, see TS 36.300 [2].

The functions supported in 5G RAN can be classified to the following categories:

- Radio Resource Management

- Transfer of user data

- Mobility control

- Connection setup and release

- Support of Network Slicing

- Radio access network sharing

- Migration and interworking with other RAT(s)

Observation 1: 5G RAN support the following function categories:

- Radio Resource Management

- Transfer of user data

- Mobility control

- Connection setup and release

- Support of Network Slicing

- Radio access network sharing

- Migration and interworking with other RAT(s)

## Lessons from 5G commercial deployment

When first discussing the new 6GR architecture, we need to review the 5G architecture and identify what functions are already supported in NG-RAN architecture. Some functions listed above are supported in commercial network deployment, while some are not supported. From the deployment and operation on NR, several important experiences and lessons have emerged:

- Network management and maintenance: In 5G, dual connectivity has been deployed and tested in limited regions, and based on the feedback of operators, dual connectivity costs a great effort for network management and maintenance in RAN. Considering the deployment and maintenance costs, dual connectivity is not expected in 6G.

Observation 2: Dual connectivity between different RATs or intra-RAT results in complexity in RAN and is limited deployed in 5G.

- Service assurance for specific traffic/scenario: In current commercial network, it’s difficult to achieve a finer service assurance mechanism since only flow-level service requirements can be acquired in the real network, while the acquisition of finer granularity service requirements, such as packet-level is not feasible to acquire more fine-grained service requirements in a highly cost-efficient manner. In some complex scenario, the user experience cannot be assured, and this mainly due to the following issues:

- Independent service requirements in RAN and CN: Currently, both RAN and CN have the separate requirement for service awareness and support its own service awareness capability/function. However, there’s limited interaction between RAN and CN to share the service awareness results to each other, which results in the duplicated deployment of service awareness capability/function, and leads to the low cost-effectiveness.

- Heavy load/cell edge scenario: In some networks with heavy loads and poor coverage scenarios, due to limited resource and/or limited coverage, it is impossible to guarantee the deterministic experience of users.

Observation 3: It’s difficult to assure user experience in a cost-effective way due to the independent service requirements in RAN and CN, especially in heavy load/ poor coverage scenario.

## 6G Requirements of architecture and migration

At RAN#109 meeting, the requirements for architecture for 6G has been identified and captured in TR 38.914 Clause 5.2.

## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

The agreements made in RAN#109 meeting can be considered as baseline when designing the 6G RAN functions and logical architecture.

# Discussion

From the agreements captured in [2], the requirements for 6G RAN architecture design share some similar common requirements for 5G RAN architecture in TR 38.913[4]. The general legacy design framework for 6G RAN architecture is referred to 5G design. In the light of that, 6G RAN architecture should inherit some functions of 5G to allow for a smooth transition from NR to 6GR. The following key logical functions for 5G RAN architecture design are worth to be considered in 6G design:

- Radio Resource Management

- Transfer of user data

- Mobility control

- Connection setup and release

- Support of Network Slicing

- Radio access network sharing

- Migration and interworking with other RAT(s)

As to migration and interworking with other RAT(s), inter-RAT handover between NR and 6GR should be supported in 6GR.

Proposal 1: 6G RAN architecture should inherit the following functions of 5G to allow for a smooth transition from NR to 6GR：

- Radio Resource Management

- Transfer of user data

- Mobility control

- Connection setup and release

- Support of Network Slicing

- Radio access network sharing

- Migration and interworking with other RAT(s), e.g., inter-RAT handover between NR and 6GR

Different from the functions defined in previous radio access technologies, 6GR is not required to maintain backward compatibility. Instead, it should introduce a new logical architecture that supports diverse services and functionalities, with a focus on achieving significant improvements in capability, efficiency, and simplicity.

In defining the appropriate 6G RAN architecture, it is essential to first identify the key characteristics of each logical function relevant to each service, so that the function can be placed in the most suitable architecture.

From our view, at least the following logical functions should be firstly included in 6GR:

- AI/ML Function: It can be observed that AI/ML is a crucial component of 6G and RAN for AI is assumed as new service based on 6G SID. The potential RAN for AI new services can include such as AI agent (including multi-agent collaboration), XR/Immersive communication and token communication. Furthermore, to satisfy the requirements on high computing demand, low-latency communication and high security, a centralized AI node within RAN is introduced to support 6G RAN for AI new service. For instance, the computing of AI applications/services can be offloaded to RAN to fulfill the real-time requirements of AI services. The centralized AI node, which serves multiple base stations, will support various AI functions, including AI computing resource management, online model training, model inference and service exposure.

Figure.1 Overall architecture of centralized AI node in 6G RAN

Furthermore, the centralized AI node can also be used to better support AI for RAN use cases by the flexible and efficient AI computing resource management. In addition, the duplicated AI data collection and reporting will be avoided, since the centralized AI node can coordinate the duplicated requests for the same data, e.g. the L1 measurement results used for different use cases.

On the other hand, a RAN data plane should be considered to support the large volumes of AI data collection, especially for model training, and AI/ML model transfer/delivery.

- Sensing Function: Sensing is identified as an objective of 6GR. Considering the enormous deployment areas, 6G sensing should take data privacy, service latency and deployment flexibility into consideration. To achieve these requirements, Sensing Function Node (SF Node) deployed in RAN (illustrated in Figure.2) should be considered in 6G network architecture. To further improve sensing performance, the impact on RAN architecture of multiple Base Stations cooperation (including integrated AI and sensing) should be considered.

Figure.2 Proposed Radio Access Network architecture for sensing in 6G RAN

- Network Digital Twin Function (NDT): NDT enables proactive network state prediction and policy pre-verification, thereby helping to achieve a higher level of network autonomy. It needs to be introduced into 6G RAN due to the following reasons:

  - NDT has been introduced in SA5 to support autonomous network management and operation. However, the NDT deployed in MnS (Management Service) cannot meet the near real-time predicting and pre-validation requirements of RAN.

  - Self-organizing network (SON) cannot pre-validate configurations, and therefore cannot guarantee that configuration, optimization, or fault-related parameters will not have negative impacts.

Therefore, NDT in 6G RAN needs to be supported from 6G Day 1, as illustrated in Figure.5. Based on dynamic network mapping and modeling, NDT enables near real-time network prediction and policy pre-validation (e.g., AI-driven policies) in RAN, thereby reducing fault risks, and lowering costs. Its performance elevation has been proven in several RAN scenarios such as proactive link adaptation and AI-based resource allocation scenarios. To fully unleash the capabilities of NDT, 6G RAN should standardize bi-directional interfaces between NDT and RAN network elements to support near real-time or real-time data collection and controls. It should also standardize the functions and invocation interfaces of various twin models from multi-vendor devices.

Figure.3 Proposed Radio Access Network architecture for NDT in 6G RAN

From the operator’s perspective, the most cost-effective approach is to establish a unified 6G RAN architecture capable of supporting various services. This can be achieved by identifying commonalities across different service requirements, thereby minimizing the need for multiple architectural solutions and reducing upgrade costs for operators.

Figure 4: Unified 6G RAN architecture capable of supporting various services

Proposal 2: It is suggested to establish a unified 6G RAN architecture capable of supporting various services, i.e., AI, sensing and NDT.

- Finer-grained service assurance：Based on the discussion on the service assurance for specific traffic/scenario in Section 2.2, it is proposed to support a finer-grained service assurance mechanism in 6G RAN, enabling the RAN and CN, as well as RAN nodes, to exchange the service-related information. This mechanism leverages AI capabilities in prediction and decision-making to achieve precise wireless resource allocation and optimal multi-cell collaboration. With such a mechanism, service awareness could be performed more cost-effective, while the network can support a larger number of UEs with guaranteed UE’s performance.

Proposal 3: It is suggested to support a cost-effective service assurance mechanism in RAN to achieve finer-grained service awareness.

- Conclusion

In this contribution, we provides our view on the functions supported by 6G RAN, based on 5G RAN functions, 5G lessons and latest agreed requirements of 6G architecture and migration. Following are the observations and proposals:

Observations:

Observation 1: 5G RAN support the following function categories:

- Radio Resource Management

- Transfer of user data

- Mobility control

- Connection setup and release

- Support of Network Slicing

- Radio access network sharing

- Migration and interworking with other RAT(s)

Observation 2: Dual connectivity between different RATs or intra-RAT results in complexity in RAN and is limited deployed in 5G.

Observation 3: It’s difficult to assure user experience in a cost-effective way due to the independent service requirements in RAN and CN, especially in heavy load/ poor coverage scenario.

Proposals:

Proposal 1: 6G RAN architecture should inherit the following functions of 5G to allow for a smooth transition from NR to 6GR：

- Radio Resource Management

- Transfer of user data

- Mobility control

- Connection setup and release

- Support of Network Slicing

- Radio access network sharing

- Migration and interworking with other RAT(s), e.g., inter-RAT handover between NR and 6GR

Proposal 2: It is suggested to establish a unified 6G RAN architecture capable of supporting various services, i.e., AI, sensing and NDT.

Proposal 3: It is suggested to support a cost-effective service assurance mechanism in RAN to achieve finer-grained service awareness.

- References

- RP-251881 New SID: Study on 6G Radio;

- TR 38.914, Study on 6G Scenarios and Requirements; (Release 20);

- TS 38.300, NR and NG-RAN Overall Description; Stage 2 (Release 18);

- 3GPP TR 38.913 Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 18).