3GPP TSG-RAN WG3 Meeting #129-bis	R3-256945

Prague – Czech Republic, 13th – 17th October 2025

Title: 	(TP to 6G TR) General Principles and Requirements for the 6G RAN architecture

Source: 	Ericsson

Agenda item:	10.2.1

Document Type:	Discussion and agreement

# 1. Introduction

In [1] a new Study Item was approved to study aspects of the 6G RAN. One of the objectives of the SID is to derive a 6G RAN architecture that fulfils the objectives of the study. Some of the objectives that are relevant for RAN3 in its early phases of 6G RAN architecture design are reported below:

The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance. […] The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. […] Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. […] - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] […] - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] […] - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

At the same time, RAN plenary agreed the following requirements for 6G RAN (see [2]):

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

In this paper we take the SID in [1] and the requirements in [2] as a starting point to formulate the main problems to be solved with a 6G RAN architecture as well as key fundamental principles of the 6G RAN architecture.

# 2. Discussion

## 2.1 Overall Requirements for the 6G RAN

The 6G RAN architecture shall enable efficient support of RAN-UE communication as well as flexible and effective communication within the RAN and between RAN entities.

One of the fundamental principles established both by the SID in [1] and by the requirements agreed in [2] is that the 6G RAN shall be based on a “stand-alone” architecture. “Stand-alone” is a term that has been derived during the course of 5G specifications and that, in the context of 5G, indicates a network that does not interwork with any other RAT.

In the context of 6G, “stand-alone” shall be interpreted in a similar way as for 5G, with the addition of important design principles that would enable to greatly enhance the 6G RAN performance. Such principles are the following. One of the main design blocks we think a standalone 6G RAN should support is the ability to maintain functionalities for the configuration and handling of a UE in a single logical entity. We have seen times and times again in 5G how complexity increases when the configuration of a UE has to be negotiated between multiple logical nodes. Each logical node would select configuration options suited to its own capabilities and to the capabilities available at the UE. The result is often a sub-optimal configuration, which impacts the overall performance with which the UE can be served.

Additionally, coordination of UE configuration and handling procedures between different logical entities implies delays in exchanging information, processing, and converging to a viable configuration option. Such delays greatly impact performance, as they slow down the activation of optimised data transfer options. In a communication world where data is generated in high bursts for very short time durations this is a very limiting factor.

We would therefore like to propose the following principle:

Proposal 1a: Capture in the “General Principle” section of the RAN3 TR for 6G that “handling of a UE context is under the responsibility of a single logical entity”.

And as a consequence

Proposal 1b: Capture in the “General Principle” section of the RAN3 TR for 6G that “handling of UE-associated functionalities such as UE configurations, UE state management, UE capability handling etc. shall be controlled by a single logical RAN entity”.

A second aspect we would like to focus on is “simplicity”. Simplicity is a key design requirement we see reflected in the 6G SID in [1]. As an example, the following is mentioned in [1]:

The detailed objectives of the study are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

While it should be discussed during the 6G study what are essential functionalities and options for the 6G RAN, it is certain that the work in RAN3 shall focus on the design and definition of logical entities and interfaces that are essential for inter-vendor interoperability. It is widely acknowledged that the standard should provide design guidelines to enable interoperability between different parts of the 6G system, in the most efficient way. 
Any standardisation effort towards processes that are not required to be multi-vendor interoperable would simply come as a limitation to the fostering of a competitive market based on innovation and execution speed.

For this reason, we would like to propose the following principle:

Proposal 2: Capture in the “General Principle” section of the RAN3 TR for 6G that RAN3 should “focus standardisation effort on the design of interfaces and logical entities that are truly needed for intervendor-interoperability”.

One important assumption to take at the very start of our study on the 6G RAN concerns legacy features already supported in 5G and to be supported in 6G and how these would be supported between the 5G RAN and the Core Network for the 6G system. 
We assume that such legacy features to be supported in 6G will evolve from features supported in 5G. Undoubtedly, enhancements to such features would be appropriate, but to ensure prompt feature parity between 5G and 6G, we believe that the well-established feature framework available in 5G should be maintained.

It would indeed be expensive and time consuming to aim for a major redesign of features already available in 5G, while such approach would not be guaranteed to deliver feature improvements.

For this reason, we would like to propose the following principle:

Proposal 3: Capture in the “General Principle” section of the RAN3 TR for 6G that “for 5G legacy features already supported in 5G and to be supported in 6G, base the feature design on the RAN-CN functional split and interactions established for the 5G RAN”.

Finally, as an overall principle, we believe that a modular design of RAN functions, protocols and interfaces is key to achieve virtualisation. Virtualization is an increasingly used tool to create additional instances of a function (or module) in a data centre to distribute load across additional CPUs or servers. 
The possibility to re-start individual processes or to execute different software versions in parallel is vital to minimize the impact on end users and to avoid that, e.g. failures affect all cells in a geographic area.

A logical node (e.g. AMF, neighbour 6G RAN node) that communicates with a 6G RAN node should not be required to know the internals of the 6G RAN node. It should be able to reach the 6G RAN node at its known IP address(s) and port number(s) and to have a guarantee that the AP message is successfully delivered at the instance of the function where its content is meant to be used. 
Therefore, the standard should be based on a design of functions/protocols and interfaces that is modular, so that such “modules” can be virtualised, instantiated and distributed.

For this reason, we would like to propose the following principle:

Proposal 4: Base the 6G RAN design on the concept of modularity at function, protocol and interface procedure level, with the objective to support flexible distribution of 6G RAN processes.

# 3. Conclusions

In this paper overall principles for the design and standardisation work needed to derive a sound 6G RAN specification have been laid down.

These principles are based on lessons learned from standardisation work on previous Radio Access Networks generations.

The following proposals have been derived and are reported below:

Proposal 1a: Capture in the “General Principle” section of the RAN3 TR for 6G that “handling of a UE context is under the responsibility of a single logical entity”.

Proposal 1b: Capture in the “General Principle” section of the RAN3 TR for 6G that “handling of UE-associated functionalities such as UE configurations, UE state management, UE capability handling etc. shall be controlled by a single logical RAN entity”.

Proposal 2: Capture in the “General Principle” section of the RAN3 TR for 6G that RAN3 should “focus standardisation effort on the design of interfaces and logical entities that are truly needed for intervendor-interoperability”.

Proposal 3: Capture in the “General Principle” section of the RAN3 TR for 6G that “for 5G legacy features already supported in 5G and to be supported in 6G, base the feature design on the RAN-CN functional split and interactions established for the 5G RAN”.

Proposal 4: Base the 6G RAN design on the concept of modularity at function, protocol and interface procedure level, with the objective to support flexible distribution of 6G RAN processes.

It is proposed to agree to the principles above and to capture them in the RAN3 6G TR, as proposed in the Annex.

# 4. References

[1] RP-251881, New SID: Study on 6G Radio

[2] RP-252870, pCR for 38.914 on Requirements for architecture and migration for 6G, NTT DOCOMO (moderator)

# 5. Annex: TP to 6G TR

# 5	Objectives and requirements

Editor’s note: The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

## 5.1	General Principles

The following principles should be fulfilled to achieve standardisation of the 6G RAN:

- Focus standardisation effort on the design of interfaces and logical entities that are truly needed for intervendor-interoperability

- Handling of UE-associated functionalities such as UE configurations, UE state management, UE capability handling etc. shall be controlled by a single logical RAN entity.

- Handling of UE-associated functionalities such as UE configurations, UE state management, UE capability handling etc. shall be controlled by a single logical RAN entity

- For 5G legacy features already supported in 5G and to be supported in 6G, base the feature design on the RAN-CN functional split and interactions established for the 5G RAN

- Base the 6G RAN design on the concept of modularity at function, protocol and interface procedure level, with the objective to support flexible distribution of 6G RAN processes.