3GPP TSG-RAN WG3 Meeting #129bis	R3- 256983

# Prague, Czech Republic, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing support for Continuous management-based MDT (C-MDT) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Jio Platforms, Ofinno |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-19 “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” RAN3 discussed the Continuous MDT (C-MDT) feature, and tasked SA5 to introduce OAM support for this feature. SA5 provided feedback on the feasibility of specifying such feature in Rel-19 (refer to the LS in R3-256523 / S5-254110) as well as Rel-19 CRs to relevant SA5 specifications. Based on the specification work conducted by SA5, there is the need to also enhance the Stage 2 description of C-MDT. This CR introduces the concept of C-MDT and provides a description of the feature at Stage 2 level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the concept of C-MDT and provides a description of the feature at Stage 2 level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The C-MDT functionality is not properly supported in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This draftCR is based on the latest Rel-18 version (V18.4.0) since the Rel-19 TS 37.320 is not available yet. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<< Start of Changes >>>>>>>>>>

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] apply.

Continuous management-based MDT: as specified in TS 32.422 [6].

Immediate MDT: MDT functionality involving measurements performed by the UE in CONNECTED state and reporting of the measurements to RAN available at the time of reporting condition as well as measurements by the network for MDT purposes.

*** skip unmodified text ***

<<<<<<<<<< Next Change >>>>>>>>>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACK	Acknowledgement

AICH	Acquisition Indicator CHannel

BLER	Block Error Rate

BSSID	Basic Service Set Identifier

CA	Carrier Aggregation

CDMA	Code Division Multiple Access

C-MDT	Continuous management-based MDT

CHO	Conditional Handover

*** skip unmodified text ***

<<<<<<<<<< Next Change >>>>>>>>>>

# 4	Main concept and requirements

## 4.1	General

The general principles and requirements guiding the definition of functions for Minimization of drive tests are the following:

- MDT mode
There are two modes for the MDT measurements: Logged MDT and Immediate MDT. There are also cases of measurement collection not specified as either immediate or logged MDT, such as Accessibility measurements or C-MDT.

*** skip unmodified text ***

<<<<<<<<<< Next Change >>>>>>>>>>

### 5.4.x	Continuous management-based MDT

Continuous management-based MDT (C-MDT) is a management-based MDT functionality that enables continuous collection of MDT data within a certain area, including both Immediate MDT and Logged MDT as specified in TS 32.422 [6]. This continuous data collection is supported during both UE’s RRC state transition from idle/inactive to connected, as well as during connected mode mobility.

C-MDT is activated by OAM towards each participating gNB involved in the same C-MDT session within a certain area, as specified in TS 32.422 [6].

The gNB shall configure the UE selected for C-MDT with Immediate MDT and Logged MDT by using the configuration provided by the OAM; a unique Trace Recording Session Reference is assigned to the UE by the gNB valid for both Immediate MDT and Logged MDT, allocated based on the received OAM configuration.

Upon UE’s RRC state transition from idle/inactive to connected, the UE may report Logged MDT measurements to the new serving gNB, including Trace Reference and Trace Recording Session Reference. If the reported Trace Reference is same as provided by OAM configuration, the new serving gNB shall identify the UE as part of the same C-MDT session. If the area-based selection and user consent conditions are satisfied, the new serving gNB shall re-configure the UE with Immediate MDT and Logged MDT.

At connected mode mobility, the source gNB provides the target gNB with Trace Reference and Trace Recording Session Reference via Xn signalling. If the reported Trace Reference is same as provided by OAM configuration, the target gNB shall identify the UE as part of the same C-MDT session. If the area-based selection and user consent conditions are satisfied, the gNB shall re-configure the UE with Immediate MDT and Logged MDT.

In any case, if a new Trace Recording Session Reference is assigned by the current serving gNB, the correlation between the old Trace Recording Session Reference and the new Trace Recording Session Reference shall be reported by gNB to the TCE to enable correlation of Immediate MDT and Logged MDT reports gathered from the same UE.

<<<<<<<<<< End of Changes >>>>>>>>>>