3GPP TSG-RAN WG3 Meeting #129bis	R3-256987

# Prague, Czech Republic, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1617 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AI/ML-based CCO use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-19 “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” RAN3 discussed the AI/ML-based Coverage and Capacity Optimization (CCO) use case, and enhanced the gNB-CU Configuration Update as well as the gNB-DU Configuration Update procedures to support such use case. Among the enhancements, a cancellation mechanism was specified to allow for cancelling previously sent future coverage modifications associated to predicted CCO issues for the same list of cells/SSBs predicted to be affected by such issues. The codepoint ‘cancel’ was introduced in the Predicted CCO Issue IE of the Predicted CCO Assistance Information IE included in the GNB-CU CONFIGURATION UPDATE message. This codepoint indicates that a previously sent predicted CCO issue is cancelled. Furthermore, clause 8.2.5.4 introduces an abnormal condition to clarify that the Predicted Affected Cells and Beams IE must be included for the cancellation mechanism to properly work. However, it remains unclear what is the gNB-DU behaviour in case the Time for Predicted CCO Issue IE is also included by the gNB-CU when sending the cancellation indication. This CR proposes that, in case such IE is sent by the gNB-CU together with the cancellation indication, it will be ignored by the gNB-DU. Similarly, the codepoint ‘cancel’ was also introduced in the Future Coverage Modification Cause IE of the Future Coverage Modification Notification List IE included in the GNB-DU CONFIGURATION UPDATE message. This codepoint, signalled together with cells and beams listed in the Future Coverage Modification Notification IE, is interpreted at the gNB-CU side as a notification that the gNB-DU has cancelled all the future coverage modifications associated to those cells and beams. However, it remains unclear what is the gNB-CU behaviour when receiving also the Future Cell Coverage State IE and the Future SSB Coverage State IE (which are mandatorily sent) but the cancel codepoint is also received. This CR proposes to allow the gNB-CU to ignore the Future Cell Coverage State IE and the Future SSB Coverage State IE when received in the cancellation notification sent by the gNB-DU. Moreover, the Neighbour Future Coverage Modification Notification List IE was introduced in the gNB-CU CONFIGURATION UPDATE message to provide the gNB-DU with information about the future coverage modifications of cells and beams controlled by the neighbour gNB-CU which predicted a CCO issue, so that the gNB-DU could react timely by deriving a matching future CCO state for its own cells and beams. The procedural text for this IE was also specified for the case it is sent by the gNB-CU together with the Predicted CCO Issue IE (included in the Predicted CCO Assistance Information IE) set to “cancel”, to clarify that the gNB-DU needs to consider it as a cancellation notification of all the future coverage modifications for cells and beams of the neighbour gNB-DU. However, it remains unclear what is the gNB-DU behaviour when receiving also the Neighbour Future Cell Coverage State IE and the Neighbour Future SSB Coverage State IE (which are mandatorily sent by the gNB-CU) but the cancel codepoint is also received. This CR proposes to allow the gNB-DU to ignore the Neighbour Future Cell Coverage State IE and the Neighbour Future SSB Coverage State IE when received in the cancellation notification of neighbor future coverage modifications sent by the gNB-CU. Similar considerations can be done for the gNB-DU behaviour in case it also receives the Time for Neighbour Future Coverage Modification IE together with the Predicted CCO Issue IE (included in the Predicted CCO Assistance Information IE) set to “cancel”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR: - enhances the sematics description of the Time for Predicted CCO Issue IE to clarify that it will be ignored by the gNB-DU if it is included by the gNB-CU when sending the cancellation indication (i.e., when the Predicted CCO Issue IE is set to ‘cancel’); - enhances the sematics descriptions of the Future Cell Coverage State IE and the Future SSB Coverage State IE to clarify that they will be ignored by the gNB-CU after they are received within the cancellation notification sent by the gNB-DU; - enhances the sematics descriptions of the Neighbour Future Cell Coverage State IE and the Neighbour Future SSB Coverage State IE to clarify that they will be ignored by the gNB-DU after they are received within the cancellation notification of neighbor future coverage modifications sent by the gNB-CU; - enhances the sematics description of the Time for Neighbour Future Coverage Modification IE to clarify that it will be ignored by the gNB-DU if it is included by the gNB-CU when sending the Neighbour Future Coverage Modification Notification List IE together with the Predicted CCO Issue IE (included in the Predicted CCO Assistance Information IE) set to “cancel”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification support for the AI/ML-based CCO use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.367, 9.3.1.368, 9.3.1.369 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.367	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO Issue | O |  | ENUMERATED (coverage, cell edge capacity, cancel, ...) | Indicates the type of predicted CCO issue or that the previously sent predicted CCO issue is cancelled. |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. The IE is ignored if the Predicted CCO Issue is set to “cancel”. |

<<<<<<<<<< Next Change >>>>>>>>>>

#### 9.3.1.368	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. The IE is ignored if the Future Coverage Modification Cause is set to “cancel” |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. The IE is ignored if the Future Coverage Modification Cause is set to “cancel” |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. The IE is ignored if the Future Coverage Modification Cause is set to “cancel” |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, cancel, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<< Next Change >>>>>>>>>>

#### 9.3.1.369	Neighbour Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by one or more neighbour node(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Future Coverage Modification Notification List |  | 1 |  |  |
| >Neighbour Future Coverage Modification Notification Item |  | 1..< maxNeighbourCellReport > |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Neighbour Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. The IE is ignored if the Predicted CCO Issue IE (included in the Predicted CCO Assistance Information IE) is set to “cancel” |
| >>Neighbour Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Neighbour Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Neighbour Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. The IE is ignored if the Predicted CCO Issue IE (included in the Predicted CCO Assistance Information IE) is set to “cancel” |
| >>Time for Neighbour Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. The IE is ignored if the Predicted CCO Issue IE (included in the Predicted CCO Assistance Information IE) is set to “cancel” |

<<<<<<<<<< End of Changes >>>>>>>>>>