3GPP TSG-RAN WG3 Meeting #129bis	R3-257064

# Prague, Czech Republic, 13th - 17th October 2025

Agenda Item:	10.3.1

Source:	Ericsson

Title:	[potentialTP for TR 38.xxx] On Principles and Requirements for the 6G RAN-CN interface

Document for:	Discussions & Approval

# 1	Introduction

This TP follows discussions in R3-257063.

The TP assumes a TR structure basically following the meeting Agenda.

Requirements deemed to be applicable for the 6G RAN-RAN interface could be considered to be captured in respective chapter(s) as well.

# 2	Text Proposal

<<<<<<<<<<<<<<<<<<<< Begin of Changes (all new text) >>>>>>>>>>>>>>>>>>>>

## X.Y	6G RAN-CN Interface

### X.Y.1	General Principles and Requirements

Studies on aspects of the 6G RAN-CN Interface and related specification work shall be performed along the following principles:

-	The 6G RAN-CN interface design shall enable realisation of RAN node and CN node modular functions in a cloud environment, while neither affecting inter-vendor operability nor requiring modular functions to be visible outside logical nodes.

-	How 6G RAN-CN interface traffic is distributed to node-internal modular functionalities shall be transparent to the peer node.

-	For functionalities also supported in 6G,6G RAN-CN application protocol functions assume the same RAN-CN functional split as for 5G and are terminated at a single logical node in the 6G RAN and the 6G CN.

-	6G RAN-CN application protocol design shall consider expected 6G logical node realisations applying modularisation by defining dedicated procedures for different protocol functions, minimising interactions between the node-internal functions and avoiding serving multiple functionalities with a signle procedure as much as possible.

-	6G RAN-CN application protocol design shall be decoupled from the signalling transport, "signalling TNL agnostic", i.e. it shall not expect any other function from the signalling transport than delivery of SDUs.

-	Introduce protocol means to enable efficient routing of AP messages supporting 6G logical node realisations in a modularised cloud environment.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>