3GPP TSG-RAN WG3 Meeting #129-bis	R3-257078

Prague, Czech Republic, 13 – 17 October, 2025

Agenda Item:	9.2.2

Source:	Nokia

Title:	(CR to TS 38.423, TS 37.483, TS 38.300, TS 38.401) Corrections of UE performance and CCO

Document for:	Discussion and CR

# 1	Introduction

We here provide corrections for UE performance (stage 3) and CCO (stage 2 and stage 3).

Corresponding CRs to TS 38.423, TS 37.483, TS 38.300 and TS 38.401 are provided in annexes of this paper.

# 2	UE performance corrections

## 2.1	Overview

The main point requiring XnAP correction relates to exit conditions for the UE performance reporting. This is discused in section 2.2 of this paper.

Furthermore, in line with proposals submitted to AI 8.1 (R3-257093), we also propose to remove the Average Packet Loss DL IE from UE Performance IE due to absence of support of this measurement at the PDCP layer in SA5’s specification. The semantics description of Slice Average Packet Loss UL IE is updated to refer to TS 28.558.

An XnAP CR is provided in annex A of this paper.

For E1AP we correct a misalignment in Data Collection Reporting Initiation procedure regarding whether partial reporting is allowed, we add the UE Performance Collection Configuration IE to DATA COLLECTION REQUEST, currently missing,  and we propose to remove the Average Packet Loss DL IE from E1AP UE performance reporting similarly to the XnAP correction proposed above.

## 2.2	Handling of exit conditions for UE performance feedback over Xn

After a UE is handed over, the target RAN node can measure different UE performance metrics (e.g Packet Delay) per-DRB and aggregate/consolidate it per-UE to send it to the source node for UE performance feedback evaluation. UE Performance measurements depend on UE configuration including its bearer configuration and radio configuration.

In RAN3#129, it was agreed to introduce Data Collection ID IE in the E1AP BEARER CONTEXT MODIFICATION REQUEST message and allow collection of UE Performance Delay for new bearers to be setup in UE CONTEXT MODIFICATION REQUEST message to allow a RAN Node to collect UE performance even for newly added DRB(s). Therefore, the UE bearer configuration (i.e., number of bearers) can change (e.g., new bearer can be added) while data collection is ongoing. When this happens, the aggregated/consolidated per-UE metric reported over XnAP to the source node also changes but the target node will continue to report. The changes will typically impact the UE throughput, but the source node will not be aware of the reasons for this change and can draw wrong conclusions. Also other events at the target side can create similar issues, e.g. if the target node sets up or deactivates carrier aggregation while the UE performance reporting is going on.

This will create an issue of incomparability of the reported UE Performance at the source node.

We think this issue needs to be corrected, e.g. according to one of these options:

- Option 1:  New exit condition(s). Similarly to the other exit conditions that we have introduced, the target NG-RAN node can monitor if there is any change e.g., in the number of bearers while data collection is ongoing and terminatethe data collection in that case. This option is simple in that it does not require any special handling at the source node.

- Option 2: Continue to report the data with an indication at the source. Another alternative is that the target continues the data collection after the configuration changes mentioned above, but in that case it would need to notify the source, otherwise the UE Performance received from the target node is under a different configuration from what the source assumes.

Proposal 1: Introduce option 1 to enable the target node to terminate data collection if the UE Configuration at the target node after the handover and during the data collection changes.

Nokia will be happy to work on detailed stage 3 impact (XnAP) according to the choice made by RAN3.

# 3	CCO corrections

## 3.1	Stage 2

For non-split architecture, legacy CCO is described in TS 38.300 clause 15.5.5 while support of AI/ML for NG-RAN is described in clause 16.20. At last meeting, RAN3 agreed to to include the stage 2 details for AI/ML based CCO in clause 16.20.3 which describes data collection and reporting. However RAN3 also agreed to support a cancel/update mechanism for the CCO issue and this mechanism required relatively detailed stage 2 description. Therefore, because the stage 2 description for AI/ML based CCO doesn’t relate to data collection and reporting, we propose to move this description to clause 15.5.5.

Proposal 2: For non-split architecture, move the main stage 2 description for the Rel-19 AI/ML-based CCO feature in TS 38.300 from clause 16.20.3 to clause 15.5.5, while keeping a reference to this description in 16.20.3.

Similarly, we propose to move the main stage 2 description of AI/ML-based CCO for non-split architecture in TS 38.401 from clause 7.11.3 to clause 7.9, while keeping a reference to this description in clause 7.11.3.

Proposal 3: For split architecture, move the main stage 2 descripition for the Rel-19 AI/ML-based CCO feature in TS 38.401 from clause 7.11.3 to clause 7.9, while keeping a reference to this description in clause 7.11.3.

In the last RAN3 meeting the following text was added TS 38.401.

At any given point in time, for a list of predicted affected cells and beams, there is only one predicted CCO issue generated by a gNB-CU.

This is a description of the gNB-CU behaviour, i.e. on sender side, where the gNB-CU should not predict more than one CCO issue with same list of predicted affected cells and beams, or at least not make such multiple predictions visible on the F1 interface. However, because this is a sender side description the receiver side behaviour will remain unspecified in some scenarios that could realistically occur. We here consider the following scenario:

Scenario: A gNB-CU and sends a first notification containing a prediction, and then sends a second notification containing a a new prediction for the same list of predicted affected cells and beams while the first prediction is still valid.

In this case, the text leads to the following two options for handling in the gNB-DU if the gNB-CU would not behave according to the requirement above as in

- Option 1: the gNB-DU ignores the second notification

- Option 2: the gNB-DU discards the first notification and replaces it with the second notification

Current specification is therefore ambiguous or unclear, by not describing what is correct gNB-DU handling in the described scenario.

Proposal 4: RAN3 agrees that current specification is ambiguous and therefore needs correction in order to describe the case where a gNB-DU receives two consecutive notifications with same list of predicted affected cells and beams.

Handling of this scenario according to option 1 would translate to introducing an abnormal condition in stage 3. However, the gNB-CU and gNB-DU could have different views on whether there already is an ongoing (or valid) CCO issue, e.g. considering non-real time signalling and absence of time stamps. We believe it is therefore not realistic to rely on that a good gNB-CU implementation eliminates the possibility of the scenario above to happen by use of the ‘cancel’ notification in all required cases. A robust signalling solution would therefore in our view need to be based on option 2.

Proposal 5: A gNB-DU receiving two consecutive notifications with same list of predicted affected cells and beams shall take into account the latest received notification and discard the received non executed notification.

We provide stage 2 CRs in support of option 2 in annexes of this paper.

## 3.2	Remaining stage 3 details

### 3.2.1	Signalling of future coverage modification to neighbour DUs and neighbour nodes

RAN3#128 agreed that the gNB-CU directly forwards the received future CCO state of neighbor cells to gNB-DU. A corresponding stage 3 TP was also agreed, introducing a new IE carrying this information. In order to ensure that the specification remains consistent after this change, we here provide some further analysis of use of the new IE.

The current option is that the new F1AP Neighbour Future Coverage Modification Notification IE (9.3.1.369) will always be signalled together with the Predicted CCO Assistance Information IE (9.3.1.367) within the GNB-CU CONFIGURATION UPDATE message. This is shown in the following signalling flow:

Figure 1: Illustration of F1 and Xn signalling for a predicted CCO issue

As shown in Figure 1, CU1 predicts the CCO issue that involves DU1 and informs it using Predicted CCO Assistance Information IE (9.3.1.367). In response DU1 determines and provides the Future CCO state in Future Coverage Modification Notification IE. CU1 informs of this Future CCO state of DU1, i.e. cells/beams of DU1, to its other served DUs, herein DU2 using Predicted CCO Assistance Information IE (9.3.1.367) and Neighbour Future Coverage Modification Notification (9.3.1.369). Based on the information received, DU2 may determine the matching Future CCO state for its cells (as the NR CGI IE contained in the Predicted Affected Cells and Beams IE will not be served by DU2).

Further, CU1 provides the Future CCO state information of its DUs to the neighbouring nodes, herein CU2, using Future Coverage Modification List IE that also contains the Predicted Coverage Modification Cause indicating the cause of planned Future CCO state change. Upon receiving this information over Xn, CU2 informs its served DUs, herein DU1’ using Predicted CCO Assistance Information IE (9.3.1.367) and Neighbour Future Coverage Modification Notification IE (9.3.1.369). Using this information the DU1’ may determine the matching Future CCO state for their cells.

Since the Future Coverage Modification Cause IE was considered redundant in the presence of the Predicted CCO issue IE contained in the Predicted CCO Assistance Information IE (9.3.1.367), it was omitted from the new IE (9.3.1.369). However, designing a suitable procedural text that can ensure inclusion of both 9.3.1.367 and 9.3.1.369 in the GNB-CU CONFIGURATION UPDATE message is not straight forward. A better solution would be to further update the new IE (9.3.1.369) to simply reflect what is sent over Xn, which would mean to include the Predicted Coverage Modification Cause IE (or simply name it Future Coverage Modification Cause). Then, it would not be needed to send the Predicted CCO Assistance Information IE if the Neighbour Future Coverage Modification Notification IE is sent.

Proposal 6: Include the Future Coverage Modification Cause IE in the new Neighbour Future Coverage Modification Notification IE.

Current F1AP procedural text requires that the receiving gNB-DU shall use the content of Predicted Affected Cells and Beams IE to determine whether the predicted CCO issue applies to a cell served by the gNB-DU, or if the notification is a request to adopt a matching future CCO state following a future CCO decision taken by another gNB-DU. However, the introduction of the new Neighbour Future Coverage Modification Notification IE makes it possible to use the received IE to determine the nature of the CCO action to be taken by the gNB-DU. We therefore propose to update the F1AP procedural text accordingly.

Proposal 7: Update the F1AP procedural text to enable the gNB-DU to use the received IE to determine the nature of the CCO action to be taken.

A corresponding update is provided in the F1AP CR submitted to this meeting in R3-257079.

### 3.2.2	Dimensioning of the Future Coverage Modification List IE

The new Neighbour Future Coverage Modification Notification IE (F1AP clause 9.3.1.369) is intended to convey information received in the NG-RAN NODE CONFIGURATION UPDATE message. However, we notice that the XnAP Future Coverage Modification List IE in this message as per current BLCR may contain information for 16384 cells:

| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Future Coverage Modification Item |  | 1..<maxnoofCellsinNG-RAN node> |  |  | – |  |

It therefore seems needed that also the F1AP Neighbour Future Coverage Modification Notification List IE is dimensioned for 16384 cells.

Proposal 8: Update the max number of cells in F1AP Neighbour Future Coverage Modification Notification List to 16384.

### 3.3.3	ASN.1: Extendibility of the Future Coverage Modification Cause

In the current BLCR for F1AP [2] as shown below both futureCoverageModificationCause IE and predicted-CCO-issue-detection IE are using enumeration Predicted-CCO-issue-detection that has two codepoints coverage and cell-edge-capacity.

Due to this, futureCoverageModificationCause and predicted-CCO-issue-detection IEs are bound to the enumerated Predicted-CCO-issue-detection. This necessarily means that change in the code point of one IE cannot be done independently, for e.g. the possibility to add new code point to futureCoverageModificationCause IE without adding it to and predicted-CCO-issue-detection IE is not possible, i.e., Predicted CCO issue IE and Future Coverage Modification Cause IE should have distinct ASN.1 definitions, in order to enable additional codepoints in one IE without touching the other IE. For instance, NES cause might in future need to be added in the Future Coverage Modification Cause IE but not in the Predicted CCO issue IE. Therefore, we propose that futureCoverageModificationCause IE should have separate enumeration. i.e. not using Predicted-CCO-issue-detection.

Proposal 9: RAN3 to define separate ENUMERATED for futureCoverageModificationCause IE i.e. not using the Predicted-CCO-issue-detection ENUMERATED (F1AP).

# 3	Conclusion

We have made the following proposals:

For UE performance reporting:

Proposal 1: Introduce option 1 to enable the target node to terminate data collection if the UE Configuration at the target node after the handover and during the data collection changes.

For CCO:

Proposal 2: For non-split architecture, move the main stage 2 description for the Rel-19 AI/ML-based CCO feature in TS 38.300 from clause 16.20.3 to clause 15.5.5, while keeping a reference to this description in 16.20.3.

Proposal 3: For split architecture, move the main stage 2 descripition for the Rel-19 AI/ML-based CCO feature in TS 38.401 from clause 7.11.3 to clause 7.9, while keeping a reference to this description in clause 7.11.3.

Proposal 4: RAN3 agrees that current specification is ambiguous and therefore needs correction in order to describe the case where a gNB-DU receives two consecutive notifications with same list of predicted affected cells and beams.

Proposal 5: A gNB-DU receiving two consecutive notifications with same list of predicted affected cells and beams shall take into account the latest received notification and discard the received non executed notification.

Proposal 6: Include the Future Coverage Modification Cause IE in the new Neighbour Future Coverage Modification Notification IE.

Proposal 7: Update the F1AP procedural text to enable the gNB-DU to use the received IE to determine the nature of the CCO action to be taken.

Proposal 8: Update the max number of cells in F1AP Neighbour Future Coverage Modification Notification List to 16384.

Proposal 9: RAN3 to define separate ENUMERATED for futureCoverageModificationCause IE i.e. not using the Predicted-CCO-issue-detection ENUMERATED (F1AP).

The above is reflected in CRs to TS 38.300, TS 38.401, TS 38.423 and TS 37.483 provided in annexes of this paper, and in CR to TS 38.473 submitted to this meeting in R3-257079.

# Annex A – CR to TS 38.423

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | XYZ | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Some corrections on XnAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedural text for the termination conditions of UE performance collection during NR-DC is unclear. The Average Packet Loss DL IE is not defined as a loss rate but as a drop rate. The semantics description of Slice Average Packet Loss UL IE is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR clarifies the procedural text for the termination conditions of UE performance collection during NR-DC. It also removes Average Packet Loss DL IE from UE Performance IE. It updates the semantics description of Slice Average Packet Loss UL IE to refer to TS 28.558. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR is backwards compatible from protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.13.2, 9.1.3.26, 9.1.3.27, 9.2.3.179, 9.2.3.238, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and slice-specific information reporting is requested, the Slice To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and predicted information is requested, the Requested Prediction Time IE shall be included. The Requested Prediction Time IE indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the measured UE trajectory is requested, the UE Trajectory Collection Configuration IE shall be included. The NG-RAN node2 shall take the UE Trajectory Collection Configuration IE into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and one or more of the UE performance metrics are requested, the UE Performance Collection Configuration IE shall be included.The NG-RAN node2 shall take the UE Performance Collection Configuration IE into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	the time since SN addition successfully completed is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	the NG-RAN node2 is released as SN for the UE.

The result of the UE performance collection may be reported periodically during the Collection Time Duration for UE Performance via DATA COLLECTION UPDATE messages and, after collection is terminated, it shall be reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", NG-RAN node2 shall ignore this information.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Slice Available Capacity Group IE, if the tenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if

-	the Slice To Report List for Data Collection IE is included for the cell, and

-	the measurement object is admitted by NG-RAN node2.

If the Cell Capacity Class Value Uplink IE and the Cell Capacity Class Value Downlink IE are included within the Predicted Slice Available Capacity Group IE for the cell for which the Predicted Slice Available Capacity IE is reported, these IEs are used to assign weights to the available capacity indicated in the requested Predicted Slice Available Capacity Value Uplink IE and Predicted Slice Available Capacity Value Downlink IE respectively.

-	the Slice Average UE Throughput DL IE, if the eleventh bit, "Slice Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average UE Throughput UL IE, if the twelfth bit, "Slice Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average Packet Delay IE, if the thirteenth bit, "Slice Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average Packet Loss UL IE, if the fourteenth bit, "Slice Average Packet Loss UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections, Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = NULL, Eighth Bit = Energy Cost, Ninth Bit = Measured UE Trajectory, Tenth Bit = Predicted Slice Available Capacity, Eleventh Bit = Slice Average UE Throughput DL, Twelfth Bit = Slice Average UE Throughput UL, Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss UL. Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = NULL, Sixth Bit = Measured UE Trajectory, Seventh Bit = Slice Average UE Throughput DL, Eighth Bit = Slice Average UE Throughput UL, Ninth Bit = Slice Average Packet Delay, Tenth Bit = Slice Average Packet Loss UL. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >>Slice Measurement Initiation Result | O |  | 9.2.3.235 | List of measurement objects that failed to be initiated per slice. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.1. |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per-UE as specified in TS 28.558 [58] clause 6.3.1.4.2. |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | NULL | This IE is ignored in the present version of the specification. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.238	Slice Based UE Performance

This IE represents UE performance metrics per S-NSSAI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [58] clause 6.3.1.4.1. |
| Slice Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [58] clause 6.3.1.4.2. |
| Slice Average Packet Delay | O |  | Average Packet Delay 9.2.3.187 | Corresponds to the Average Packet Delay per S-NSSAI as specified in TS 28.558 [58], clause 6.3.1.1. |
| Slice Average Packet Loss UL | O |  | INTEGER (0..1000000, ...) | Corresponds to UL PDCP SDU Loss Rate per S-NSSAI defined in TS 28.558 [37] clause 6.3.1.7.1. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- S

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

SliceToReportForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceToReportForDataCollection-List-Item

SliceToReportForDataCollection-List-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions				ProtocolExtensionContainer { { SliceToReportForDataCollection-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceToReportForDataCollection-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceMeasurementInitiationResult::= SEQUENCE {

sliceMeasurementInitiationResult-List		SliceMeasurementInitiationResult-List,

iE-Extensions					ProtocolExtensionContainer { {SliceMeasurementInitiationResult-ExtIEs} } OPTIONAL,

...

}

SliceMeasurementInitiationResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceMeasurementInitiationResult-Item

SliceMeasurementInitiationResult-Item ::= SEQUENCE {

pLMNIdentity									PLMN-Identity,

sNSSAIMeasurementInitiationResult-List			SNSSAIMeasurementInitiationResult-List,

iE-Extensions									ProtocolExtensionContainer { { SliceMeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

SliceMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SNSSAIMeasurementInitiationResult-Item

SNSSAIMeasurementInitiationResult-Item ::= SEQUENCE {

sNSSAI								S-NSSAI,

sNSSAIMeasurementFailureCause-List		SNSSAIMeasurementFailureCause-List		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {SNSSAIMeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

SNSSAIMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIMeasurementFailureCause-List ::=SEQUENCE (SIZE(1..maxnoofFailedSliceMeasObjects)) OF SNSSAIMeasurementFailureCause-Item

SNSSAIMeasurementFailureCause-Item ::= SEQUENCE {

sNSSAIMeasurementFailedReportCharacteristics	BIT STRING(SIZE(32)),

cause											Cause,

iE-Extensions									ProtocolExtensionContainer { {SNSSAIMeasurementFailureCause-Item-ExtIEs} }		OPTIONAL,

...

}

SNSSAIMeasurementFailureCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceUEPerformance ::= SEQUENCE {

pLMNIdentity								PLMN-Identity,

s-NSSAIUEPerformance-List					S-NSSAIUEPerformance-List,

iE-Extensions								ProtocolExtensionContainer { { SliceUEPerformance-ExtIEs} } OPTIONAL,

...

}

SliceUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

S-NSSAIUEPerformance-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAIUEPerformance-Item

S-NSSAIUEPerformance-Item	::= SEQUENCE {

sNSSAI								S-NSSAI,

sliceBasedUEPerformance				SliceBasedUEPerformance,

iE-Extensions						ProtocolExtensionContainer { { S-NSSAIUEPerformance-Item-ExtIEs} }	OPTIONAL,

...

}

S-NSSAIUEPerformance-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceBasedUEPerformance	::= SEQUENCE {

sliceAverageUEThroughputDL			BitRate		OPTIONAL,

sliceAverageUEThroughputUL			BitRate		OPTIONAL,

sliceAverageUEPacketDelay			AveragePacketDelay					OPTIONAL,

sliceAveragePacketLossUL			INTEGER(0..1000000, ...)			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { SliceBasedUEPerformance-ExtIEs} } OPTIONAL,

...

}

SliceBasedUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- U

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						NG-RANnodeUEXnAPID,

uEPerformance								UEPerformance			OPTIONAL,

measuredUETrajectory						MeasuredUETrajectory	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceUEPerformance	CRITICALITY ignore	EXTENSION SliceUEPerformance PRESENCE optional },

...

}

UEPerformance ::= SEQUENCE {

dL-UE-AverageThroughput						BitRate					OPTIONAL,

uL-UE-AverageThroughput						BitRate					OPTIONAL,

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketLossDL						NULL			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

UEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEPerformanceCollectionConfiguration ::= SEQUENCE {

collectionTimeDurationForUEPerformance				INTEGER(1..5000, ...),

iE-Extensions								ProtocolExtensionContainer { { UEPerformanceCollectionConfiguration-ExtIEs} } OPTIONAL,

...

}

UEPerformanceCollectionConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UETrajectoryCollectionConfiguration ::= SEQUENCE {

collectionTimeDurationForUETrajectory								INTEGER (1..4096, ...),

numberOfVisitedCells								INTEGER (1..16, ...)							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UETrajectoryCollectionConfiguration-ExtIEs} }		OPTIONAL,

...

}

UETrajectoryCollectionConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ULTCIStateID  ::= OCTET STRING

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex B – CR to TS 37.483

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | XYZ | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE Performance Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a misalignment in Data Collection Reporting Initiation procedure regarding whether partial reporting is allowed. The UE Performance Collection Configuration IE is missing from DATA COLLECTION REQUEST message and the IE is undefined. The Average Packet Loss DL IE is not defined as a loss rate but as a drop rate. The semantics description of Average Packet Loss UL IE is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR aligns the Data Collection Reporting Initiation procedure to not support partial reporting. It introduces the missing UE Performance Collection Configuration IE in the DATA COLLECTION REQUEST message and defines it. Removes Average Packet Loss DL IE from UE Performance IE. Update the semantics description of Average Packet Loss UL IE to refer to TS 28.558. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR is backwards compatible from protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.11.2, 8.2.11.3, 9.3.1.149, 9.3.1.xx (new), 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.2.11	Data Collection Reporting Initiation

#### 8.2.11.1	General

This procedure is used by a gNB-CU-CP to request from a gNB-CU-UP the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.2.11.2	Successful Operation

Figure 8.4.11.2-1: Data Collection Reporting Initiation, successful operation

The gNB-CU-CP initiates the procedure by sending the DATA COLLECTION REQUEST message to gNB-CU-UP to start information reporting and stop information reporting. Upon receipt, the gNB-CU-UP:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to “start”; or

-	shall stop all information reporting and terminate the reporting in case the Registration Request for Data Collection IE is set to “stop”.

If the gNB-CU-UP is capable to provide all of the requested information, it shall initiate the information reporting as requested by the gNB-CU-CP and respond with the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic information. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and one or more of the UE performance metrics are requested, the UE Performance Collection Configuration IE shall be included. The gNB-CU-UP shall take the UE Performance Collection Configuration IE into account for the configuration of UE performance collection and reporting. The gNB-CU-UP shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since the bearer context was successfully established is equal to the value of the Collection Time Duration for UE Performance IE;

-	all the established bearers are suspended or released.

Interaction with other procedures

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average Packet Delay IE, if the first bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP;

-	the Average Packet Loss UL IE, if the second bit, "Average Packet Loss UL" of the Report Characteristics for   Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

#### 8.2.11.3	Unsuccessful Operation

Figure 8.4.11.3-1: Data Collection Reporting Initiation, unsuccessful operation

If any of the requested information reporting cannot be initiated, gNB-CU-UP shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT MODIFICATION RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	A list of PDU Session Resources which are successfully modified shall be included in the PDU Session Resource Modified List IE;

-	A list of PDU Session Resources which failed to be modified shall be included in the PDU Session Resource Failed To Modify List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Interactions with DL Data Notification procedure:

If the MT-SDT Information Request IE is included in the BEARER CONTEXT MODIFICATION REQUEST message and the value is set to 'true', the gNB-CU-UP shall, if supported, store it and report the MT-SDT Information IE in the DL DATA NOTIFICATION message as specified in TS 38.401 [2].

If the SDT Data Size Threshold IE is included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store it and act as specified in TS 38.401 [2].

Interaction with the Bearer Context Modification (gNB-CU-CP initiated)

If the BEARER CONTEXT MODIFICATION REQUEST message includes for a DRB in the DRB To Modify List IE the PDCP SN Status Request IE set to “requested” and if the gNB-CU-UP has not yet received a SDAP end marker packet for a QoS flow which has been previously re-configured to another DRB by means of a gNB-CU-CP initiated Bearer Context Modification procedure, the gNB-CU-UP shall includes the QoS Flow Identifier of that QoS flow in the Old QoS Flow List - UL End Marker expected IE in the PDU Session Resource Modified List IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, report to the gNB-CU-CP, after successful bearer context modification via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the gNB-CU-CP Measurement ID IE, allocated by the gNB-CU-CP, and the gNB-CU-UP Measurement ID IE, allocated by the gNB-CU-UP.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.23	DATA COLLECTION REQUEST

This message is sent by gNB-CU-CP to gNB-CU-UP to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average Packet Delay, Second Bit = Average Packet Loss UL. Other bits are ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.3.1.xx |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

#### 9.2.1.24	DATA COLLECTION RESPONSE

This message is sent by gNB-CU-UP to gNB-CU-CP to indicate that the requested information, for all or part of the measurement objects included in the reporting is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.25	DATA COLLECTION FAILURE

This message is sent by the gNB CU-UP to gNB-CU-CP to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.26	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the requested information.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1..<maxnoofUEReports> |  |  |  |  |
| >>UE Assistant Identifier | M |  | gNB-CU-CP UE E1AP ID 9.3.1.4 | gNB-CU-CP UE E1AP ID allocated by gNB-CU-CP. |  |  |
| >>DRB To Report List |  | 0..1 |  |  |  |  |
| >>>DRB To Report Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>>DRB ID | M |  | 9.3.1.16 |  |  |  |
| >>>>UE Performance | O |  | 9.3.1.149 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.149	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.150 |  |
| Average Packet Loss UL | O |  | INTEGER(0..1000000, …) | Corresponds to UL PDCP SDU Loss Rate defined in TS 28.558 [37] clause 6.3.1.3.1 for split gNB and clause 6.3.1.7.1 for non-split gNB. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xx	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful bearer context estabilishment within which the UE performance measurements are collected. Unit: millisecond |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

Cause,

CriticalityDiagnostics,

GNB-CU-CP-MBS-E1AP-ID,

GNB-CU-UP-MBS-E1AP-ID,

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

UEAssociatedInfoResult-List,

DataCollectionID,

Measurement-ID,

UEPerformanceCollectionConfiguration,

FROM E1AP-IEs

PrivateIE-Container{},

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- DATA COLLECTION REQUEST

--

-- **************************************************************

DataCollectionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionRequest-IEs}},

...

}

DataCollectionRequest-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID										PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID					CRITICALITY ignore	TYPE Measurement-ID										PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "stop".

{ ID id-RegistrationRequestForDataCollection		CRITICALITY reject	TYPE RegistrationRequestForDataCollection		PRESENCE mandatory}|

{ ID id-ReportCharacteristicsForDataCollection		CRITICALITY reject	TYPE ReportCharacteristicsForDataCollection	PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "start".

{ ID id-ReportingPeriodicityForDataCollection		CRITICALITY ignore	TYPE ReportingPeriodicityForDataCollection		PRESENCE optional}|

{ ID id-UEPerformanceCollectionConfiguration		CRITICALITY ignore	TYPE UEPerformanceCollectionConfiguration		PRESENCE optional},

...

}

-- **************************************************************

--

-- DATA COLLECTION RESPONSE

--

-- **************************************************************

DataCollectionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionResponse-IEs}},

...

}

DataCollectionResponse-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID										PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional},

...

}

-- **************************************************************

--

-- DATA COLLECTION FAILURE

--

-- **************************************************************

DataCollectionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionFailure-IEs}},

...

}

DataCollectionFailure-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID				CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID				CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

-- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-UEAssociatedInfoResult-List					CRITICALITY ignore	TYPE UEAssociatedInfoResult-List							PRESENCE optional },	...

}

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<<Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- U

UDC-Parameters	::= SEQUENCE {

bufferSize					BufferSize,

dictionary					Dictionary											OPTIONAL,

continueUDC					ENUMERATED {true, ...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UDC-Parameters-ExtIEs } } 		OPTIONAL

}

UDC-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-VersionID CRITICALITY ignore EXTENSION INTEGER (0..15) PRESENCE optional},

...

}

UE-Activity	::=	ENUMERATED	{

active,

not-active,

...

}

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						GNB-CU-CP-UE-E1AP-ID,

dRB-To-Report-List							DRB-To-Report-List,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-associatedLogicalE1-ConnectionItem ::= SEQUENCE {

gNB-CU-CP-UE-E1AP-ID		GNB-CU-CP-UE-E1AP-ID	 OPTIONAL,

gNB-CU-UP-UE-E1AP-ID		GNB-CU-UP-UE-E1AP-ID	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UE-associatedLogicalE1-ConnectionItemExtIEs} }	OPTIONAL,

...

}

UE-associatedLogicalE1-ConnectionItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UEPerformance ::= SEQUENCE {

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketLossUL						INTEGER (0..1000000, ...)		OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

UEPerformance-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UEPerformanceCollectionConfiguration ::= SEQUENCE {

collectionTimeDurationForUEPerformance				INTEGER(1..5000, ...),

iE-Extensions								ProtocolExtensionContainer { { UEPerformanceCollectionConfiguration-ExtIEs} } OPTIONAL,

...

}

UEPerformanceCollectionConfiguration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UESliceMaximumBitRateList ::= SEQUENCE (SIZE(1.. maxnoofSMBRValues)) OF UESliceMaximumBitRateItem

UESliceMaximumBitRateItem ::= SEQUENCE {

sNSSAI		SNSSAI,

uESliceMaximumBitRateDL		BitRate,

iE-Extensions		ProtocolExtensionContainer { { UESliceMaximumBitRateItem-ExtIEs} } OPTIONAL,

...

}

UESliceMaximumBitRateItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-Configuration	::=	ENUMERATED	{

no-data,

shared,

only,

...

}

ULUPTNLAddressToUpdateItem	::= SEQUENCE {

oldTNLAdress						TransportLayerAddress,

newTNLAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { ULUPTNLAddressToUpdateItemExtIEs } }	OPTIONAL,

...

}

ULUPTNLAddressToUpdateItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

ULDataSplitThreshold	::=	ENUMERATED {b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800, b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400, b6553600, infinity, ...}

UP-Parameters ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF UP-Parameters-Item

UP-Parameters-Item ::= SEQUENCE {

uP-TNL-Information			UP-TNL-Information,

cell-Group-ID				Cell-Group-ID,

iE-Extensions				ProtocolExtensionContainer { { UP-Parameters-Item-ExtIEs } }	OPTIONAL,

...

}

UP-Parameters-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoS-Mapping-Information	CRITICALITY reject	EXTENSION QoS-Mapping-Information	PRESENCE optional}|

{ID id-IndirectPathIndication 	CRITICALITY ignore	EXTENSION IndirectPathIndication 	PRESENCE optional},

...

}

UPSecuritykey	::= SEQUENCE {

encryptionKey				EncryptionKey,

integrityProtectionKey		IntegrityProtectionKey		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UPSecuritykey-ExtIEs } }	OPTIONAL,

...

}

UPSecuritykey-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

UP-TNL-Information		::= 	CHOICE {

gTPTunnel		GTPTunnel,

choice-extension		ProtocolIE-SingleContainer	{{UP-TNL-Information-ExtIEs}}

}

UP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

UplinkOnlyROHC ::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UplinkOnlyROHC-ExtIEs } } 		OPTIONAL

}

UplinkOnlyROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

URIaddress ::= VisibleString

UserPlaneErrorIndicator ::= ENUMERATED {

gTP-U-error-indication-received,

...

}

UEInactivityInformation ::= INTEGER (1..7200, ...)

UserPlaneFailureIndication ::= SEQUENCE {

userPlaneFailureType			UserPlaneFailureType,

nG-DL-UP-TNL-Information			UP-TNL-Information,

nG-UL-UP-TNL-Information			UP-TNL-Information,

iE-Extensions		ProtocolExtensionContainer { { UserPlaneFailureIndication-ExtIEs} }	OPTIONAL,

...

}

UserPlaneFailureIndication-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UserPlaneFailureType ::= ENUMERATED {

gtp-u-error-indication-received,

up-path-failure,

...

}

#### -- V

#### -- W

#### -- X

#### -- Y

#### -- Z

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<<Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

id-Remaining-time-based-rLC-retransmission-threshold			ProtocolIE-ID ::= 234

id-Remaining-time-based-rLC-polling-threshold					ProtocolIE-ID ::= 235

id-DL-PDU-Set-Info-Marking-Support-Ind						ProtocolIE-ID ::= 236

id-UEPerformanceCollectionConfiguration							ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex C – CR to TS 38.300

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of enhancements on AI/ML for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CCO: - Details of CCO-specific signalling is captured as mechanism for data collection and reporting for NG-RAN AI/ML, which is misleading. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CCO: - Move the main stage 2 description for the Rel-19 AI/ML-based CCO feature in TS 38.300 from clause 16.20.3 to clause 15.5.5, while keeping a reference to this description in 16.20.3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.5.5.x (new), 16.20.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 15.5.5	Support for Coverage and Capacity Optimisation

#### 15.5.5.1	General

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues, e.g. coverage and cell edge interference issues.

#### 15.5.5.2	OAM requirements

Each NG-RAN node may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

#### 15.5.5.3	Dynamic coverage configuration changes

An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator.

The coverage state indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the coverage state indicator to retrieve a previously stored Mobility Robustness Optimisation state. The coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells coverage configurations.

If the list includes indication about planned reconfiguration and possibly a list of replacing cells, the receiving NG-RAN node may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending NG-RAN node adds cells in inactive state, the receiving NG-RAN node may use this information to avoid connection or re-establishment failures. The receiving NG-RAN node may also use the notification to reduce the impact on mobility. The receiving NG-RAN node should avoid triggering handovers towards cell(s) that are indicated to be inactive.

#### 15.5.5.x	AI/ML enabled Coverage and Capacity Optimization

An NG-RAN node may derive predicted CCO issues and the corresponding future coverage states for its affected cells and beams. The NG-RAN node may notify its neighbour NG-RAN nodes, via the NG-RAN Node Configuration Update procedure, about the future coverage state changes together with modification cause and time information. The NG-RAN node may also notify its neighbour NG-RAN nodes that previously notified coverage state changes together with the corresponding modification cause have been cancelled.

At any given point in time, for a list of predicted affected cells and beams, there is only one predicted CCO issue generated by a gNB.

If a gNB receives a new notification about modified coverage as described in clause 15.5.5.3 or future coverage state before execution of a previously received notification about future coverage state for the same cell(s), the gNB takes into account the latest received notification replacing the previously received non executed notification.

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information, including predicted radio resource status per slice;

-	UE performance feedback, including per-slice UE performance;

-	Measured UE trajectory;

-	Energy Cost (EC);

-	Predicted slice available capacity.

The collection and reporting of the above information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Signalling support related to AI/ML enabled Coverage and Capacity Optimization is described in clause 15.5.5.x and in case of split gNB in TS 38.401 [4].

### 16.20.4	OAM Requirements

OAM configures the following for EC reporting:

-	The minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area; and

-	The recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

OAM further configures management-based MDT for continuous MDT towards the NG-RAN. OAM provides specific information to the NG-RAN for the NG-RAN to identify continuous MDT.

# Annex D – CR to TS 38.401

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of enhancements on AI/ML for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CCO: - Details of CCO-specific signalling is captured as mechanism for data collection and reporting for NG-RAN AI/ML, which is misleading. - Ambiguous specification due to missing description of the case where a gNB-DU receives two consecutive notifications with same list of predicted affected cells and beams. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CCO: - Move the main stage 2 description for the Rel-19 AI/ML-based CCO feature from clause 7.11.3 to clause 7.9, while keeping a reference to this description in 16.20.3 - Added description of the case where a gNB-DU receives two consecutive notifications with same list of predicted affected cells and beams |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading and ambiguous specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.x (new), 7.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.9	Support for CCO

### 7.9.1	General

The NR Capacity and Coverage Optimization (CCO) Function in non-split gNB case is specified in TS 38.300 [2]. The objective of this function is to detect and mitigate coverage and cell edge interference issues.

### 7.9.2	OAM requirements

Each gNB-DU may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

### 7.9.3	Dynamic coverage configuration changes

In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU signals to the gNB-DU the CCO issue and the affected cells and beams. The gNB-DU resolves the CCO issue concerning own served cell by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the CCO configuration. The gNB-DU informs the gNB-CU of the new coverage states adopted.

### 7.9.x	AI/ML enabled Coverage and Capacity Optimization

At any given point in time, for a list of predicted affected cells and beams, there is only one predicted CCO issue generated by a gNB-CU.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, predicted CCO assistance information, including a predicted CCO issue, the predicted affected cells and beams, and the time when the predicted CCO issue will happen.

A gNB-DU may also receive, in a GNB-CU CONFIGURATION UPDATE message, the future coverage configurations of a list of cells and beams not served by the gNB-DU with their corresponding future coverage cause and the time of application of the future coverage configuration. In response, the gNB-DU may indicate to the gNB-CU, in a GNB-DU CONFIGURATION UPDATE message, a future coverage configuration for its cells and beams, and an associated time and cause.

A gNB-DU may receive, in a GNB-CU CONFIGURATION UPDATE message, a notification that a previously received predicted CCO issue, associated to the list of affected cells and beams received with the predicted CCO issue, is cancelled.

The gNB-DU may also notify the gNB-CU, in the GNB-DU CONFIGURATION UPDATE message, that a previously notified non applied coverage state change, for a list of affected cells and beams corresponding to a cancelled predicted CCO issue, has been cancelled.

If a gNB-DU receives a new notification about predicted CCO assistance information, including a predicted CCO issue, before execution of a previously received notification for the same cell(s), the gNB-DU takes into account the latest received notification replacing the previously received non executed notification.

If a gNB-DU receives a new notification about modified coverage as described in clause 7.9.3 or future coverage state before execution of a previously received notification about future coverage state for the same cell(s), the gNB-DU takes into account the latest received notification replacing the previously received non executed notification.

### 7.11.3	Data Collection and Reporting

The following information can be configured to be reported by a gNB-DU:

-	Energy Cost (EC),

-	UE performance feedback.

The collection of EC is configured through the Resource Status Reporting Initiation procedure while the reporting is performed through the Resource Status Reporting procedure. Packet delay measurement collection for UE Performance from gNB-DU is triggered through the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages and the packet delay measurements collected by the gNB-DU are reported from gNB-DU to gNB-CU-UP through the ASSISTANCE INFORMATION DATA frame, specified in TS 38.425 [33].

The following information can be configured to be reported by a gNB-CU-UP:

-	UE performance feedback.

The collection and reporting of UE performance feedback from a gNB-CU-UP are configured through the Data Collection Reporting Initiation procedure, while the reporting is performed through the Data Collection Reporting procedure.

Signaling by a split gNB of the RLC Average UE UL/DL Throughput over the Xn interface is not supported in this version of the specifications.

Signalling support related to AI/ML enabled Coverage and Capacity Optimization is described in clause 7.9.x.