3GPP TSG-RAN WG3 Meeting #129bis	R3-257098

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	9.2.2

Source:	Nokia

Title:	Continuous MDT Discussion

Document for:	Decision

# 1	Introduction

In RAN3 #128 RAN3 sent an LS to SA5 [1] to provide SA5 the necessary information so that SA5 prepares the Continuous MDT solution. In their LS, RAN3 indicated to SA5 that they have agreed to the following principles:

- Information from OAM to the NG-RAN nodes participating to Continuous MDT, allowing for the identification of a continuous MDT process, are provided e.g. in the form of specific Trace Reference(s).

- Solutions for Continuous MDT shall not have an impact on the 5GC in Rel19 from a RAN3 perspective.

- RAN3 agrees to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem

In addition, in their LS, RAN3 provided some solutions to SA5 that could possibly solve the overlapping issue in Continuous MDT. Namely, those solutions are listed next:

- In some solutions, a dedicated TRSR range is assigned to each NG-RAN node participating in Continuous MDT. When an NG-RAN node selects a UE for Continuous MDT (not configured so far with Continuous MDT), the NG-RAN node assigns a TRSR from such range to the UE. The same TRSR will be used for the whole duration of the Continuous MDT trace of a UE for correlation purposes. Namely, the TRSR is kept for the UE even if the UE moves to a different participating node via handover (the TR and TRSR are signalled from source NG-RAN node to target NG-RAN node) or if the UE is configured with Logged MDT and it moves to RRC_Idle/RRC_Inactive. Trace Correlation is achieved by the assigned TR/TRSR which can identify that the trace records belong to the same selected UE. Overlapping is avoided because the same TRSR from the TRSR range assigned to the NG-RAN node that initiated the Continuous MDT process will be used for the whole duration of the Continuous management-based MDT trace of a UE. Or

- In some other solutions, RAN3 assumes that the problem of overlapping does not occur because OAM configures gNBs in an area with same immediate MDT configuration (TR1) and same logged MDT configuration (TR2). A gNB chooses a UE for Continuous MDT and configures the UE for Immediate MDT and Logged MDT separately (with separate TR and may be also different TRSRs, i.e. TR1/TRSR1 and TR2/TRSR2). At connected-to-idle transition the serving gNB sends the Immediate MDT report (TR1/TRSR1) to TCE by also including the TR2/TRSR2 assigned for the Logged MDT. At idle-to-connected transition the (new) serving gNB sends the Logged MDT report (TR2/TRSR2) to TCE by also including the TR1/TRSR3 assigned for the next Immediate MDT. Or,

- In some other solutions, RAN3 assumes that the problem of overlapping does not occur because OAM configures gNBs in an area with same configuration (single TR) encompassing both immediate MDT and logged MDT. A gNB chooses a UE for Continuous MDT and configures the UE for Immediate MDT and Logged MDT with the same TR and TRSR (TRSR1). If at idle-to-connected, or at handover the new serving gNB cannot re-assign TRSR1 to the UE, it will assign a new TRSR (TRSR2) and include both the previously assigned TRSR (TRSR1) and the new TRSR (TRSR2) in the Logged MDT (TR & TRSR1/TRSR2) or Immediate MDT report (TR & TRSR1/TRSR2) to TCE.

In addition, RAN3 asked SA5 to check the security aspects of the derived SA5 solution on Continuous MDT.

In this meeting, RAN3 received a reply LS from SA5 in [2] regarding Continuous MDT in which SA5 confirmed feasibility of a Continuous MDT solution based on the principles that RAN3 provided. In addition, SA5 sent to RAN3 a set of CRs, implementing the Continuous MDT solution. RAN3 had asked SA5 to check security aspects for the Continuous MDT solution but there was no reference to security aspects in the reply LS from SA5.

# 2	Discussion

In [3], SA5 introduced in clause 4.1.1.9.x two different NG-RAN activation mechanisms of Continuous Management-based MDT. In the first mechanism, the management system sends two Trace Session activation requests, one for Immediate and one for Logged MDT with the same TR to the gNB. In the second mechanism, the management system sends an activation request through a common Job Type IMMEDIATE_MDT_AND_LOGGED_MDT to the gNB. This is aligned with the RAN3 agreement (also communicated to SA5 in [1]) which stated:

- Information from OAM to the NG-RAN nodes participating to Continuous MDT, allowing for the identification of a continuous MDT process, are provided e.g. in the form of specific Trace Reference(s).

Based on the above, a gNB determines whether a Management-based MDT Configuration corresponds to Continuous MDT by checking the Trace Reference received by the OAM. Under both activation requests a given received TR value indicates to the gNB that the corresponding Trace Session is for Continuous MDT.

Observation 1: Based on SA5 specification and based on RAN3 agreements a given TR (under either mechanism introduced by SA5) identifies whether a Trace Activation corresponds to Continuous Management-based MDT.

Some important aspects of the Continuous Management-based MDT solution introduced by SA5 require in our view correction or clarification.

In particular, in 4.1.1.9.x of [3] the following is stated: “This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request.”. Based on RAN3’s requirement the gNB shall be able to identify a logged MDT session as C-MDT based on the received TR from the UE, without additional configured information relative to TRSR range. The mechanism described in clause 4.1.1.9.x of [3] therefore does not fulfill RAN3’s requirement that Continuous MDT is identified by the RAN through specific Trace Reference(s).

Observation 2: In 4.1.1.9.x of S5-254111, it is stated: “This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request.”. This does not fulfil RAN3’s requirement that Continuous MDT is identified by the RAN through specific Trace Reference(s).

We therefore think that we need to send an LS to SA5 to inform them that according to RAN3 only the TR is an indication to the RAN for continuous MDT and not the TRSR Prefix Configuration parameter.

Proposal 1: Send an LS to SA5 to inform them that according to RAN3 only the TR is an indication to the RAN for continuous MDT and not the TRSR Prefix Configuration parameter.

In addition, the SA5 definition of Continuous Management-based MDT is the following:

Continuous management-based MDT: a management-based MDT functionality that enables continuous collection of MDT data, including both Immediate MDT and Logged MDT, within a specified area.

In the following we analyse two aspects of this definition, i.e. the aspect of area for continuous MDT, and the aspect of continuity.

As can be seen from the definition, the specified area over which continuous management-based MDT is enabled is not clearly defined, namely, it is unclear whether the area is defined as a list of cells or as a list of tracking areas, following the same principles as in Management-based MDT.

In addition, the logging area for Logged MDT is an important configuration parameter to maintain measurement continuity. When it is configured, the UE will log measurements as long as it is within the logging area. When it exits the logging area or RPLMN is not part of the MDT PLMN list then even though the UE keeps the measurement configuration the actual logging is suspended. This would create discontinuity in the measurement collection.

Proposal 2: In the LS to SA5, ask them to clarify the area over which Continuous Management based MDT is configured.

One aspect that affects this area is the, newly introduced by SA5, Trace Recording Session Reference (TRSR) Prefix Configuration parameter, which reserves a TRSR range for a gNB during the TRSR assignment. This prefix creates separation between the TRSR ranges that are used by different gNBs in order to allow unique TRSR assignments by gNBs in the RAN which would enable the TCE to unambiguously correlate logged and immediate MDT reports from the same UE. Since Continuous MDT reuses the MDT framework, to make sure that data collection may continue in an uninterrupted way, Continuous MDT needs to be configured across the gNBs of the PLMN. Depending on the operator network a PLMN may span as many as 20K gNBs e.g., in crowded regions such as Japan. Note that the TRSR comprises 2 bytes. In the current MDT framework, where TRSR can be reused by different gNBs, each gNB can configure up to 216-1 UEs for management based MDT. In Continuous MDT, where a prefix is provided to each gNB and where TRSR reuse is not allowed, the number of gNBs as well as the number of Continuous MDT UEs decreases. If 15 bits are used for the prefix, then PLMNs of the size of the Japan scenario can be covered, but this would only leave 1 bit for the TRSR assignments towards a UE. This means that a single UE can be configured with continuous MDT. If we assume smaller PLMN sizes (in the order of 10K gNBs), then 14 bits are sufficient meaning that Continuous MDT can only be configured to 3 UEs within a gNB. This is a serious limitation of the introduced feature since it has limited applicability.

Observation 3: The solution introduced by SA5 has limited applicability and does not meet the RAN3 AI/ML data collection requirements where data collection is needed from a large number of UEs or where Continuous MDT needs to be configured across the PLMN.

Proposal 3: Ask SA5 to enhance their Continuous MDT solution to address scenarios where data collection is needed from a large number of UES or where Continuous MDT needs to be configured across the PLMN.

Finally, it seems that SA5 did not check security aspects of the solution with SA3, even though RAN3 had asked this in their LS to SA5.

Observation 4: Even though RAN3 asked SA5 to check security aspects for the Continuous MDT solution with SA3, there was no reference to security aspects in the reply LS from SA5.

Proposal 4: RAN3 would like to include SA3 in the reply LS to SA5 so that the Continuous MDT solution is evaluated with respect to security.

When it comes to the aspect of continuity, SA5’s definition of continuous MDT is not specific to measurements. Instead, it assumes that continuity is defined across all measurements for which logged and immediate MDT can be configured. However, there are issues with such generalization.

According to the SA5 solution, logged MDT configured for Continuous MDT may comprise event-based triggers and periodic triggers. When an event-based trigger is configured, a UE starts logging of the measurements when the triggering condition is detected. This for example can be true for Out of Coverage measurement and for RSRP reporting. Given the unpredictability when the triggering condition will be met at the UE, the log will be discontinuous until the triggering condition is met (since no data will be logged by the UE).

Observation 5: Event-triggered logging is not suitable for Continuous MDT.

Proposal 5: RAN3 to address how continuity is maintained for event-based triggers.

We provide a reply LS to SA5 in the annex of this paper.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: Based on SA5 specification and based on RAN3 agreements a given TR (under either mechanism introduced by SA5) identifies whether a Trace Activation corresponds to Continuous Management-based MDT.

Observation 2: In 4.1.1.9.x of S5-254111, it is stated: “This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request.”.  This does not fulfil RAN3’s requirement that Continuous MDT is identified by the RAN through specific Trace Reference(s).

Proposal 1: Send an LS to SA5 to inform them that according to RAN3 only the TR is an indication to the RAN for continuous MDT and not the TRSR Prefix Configuration parameter.

Proposal 2: In the LS to SA5, ask them to clarify the area over which Continuous Management based MDT is configured.

Observation 3: The solution introduced by SA5 has limited applicability and does not meet the RAN3 AI/ML data collection requirements where data collection is needed from a large number of UEs or where Continuous MDT needs to be configured across the PLMN.

Proposal 3: Ask SA5 to enhance their Continuous MDT solution to address scenarios where data collection is needed from a large number of UES or where Continuous MDT needs to be configured across the PLMN.

Observation 4: Even though RAN3 asked SA5 to check security aspects for the Continuous MDT solution with SA3, there was no reference to security aspects in the reply LS from SA5.

Proposal 4: RAN3 would like to include SA3 in the reply LS to SA5 so that the Continuous MDT solution is evaluated with respect to security.

Observation 5: Event-triggered logging is not suitable for Continuous MDT.

Proposal 5: RAN3 to address how continuity is maintained for event-based triggers.

# References

[1]	R3-253958, Reply LS on Continuous MDT, RAN3

[2]		R3-256523- S5-254110, Reply to LS on Continuous MDT, SA5

[3] 		S5-254111, Rel-19 CR TS 32.422 Continuous MDT, Ericsson, Deutsche Telekom, CATT, ZTE, Huawei

# Annex			  Draft LS to SA5

3GPP TSG-RAN WG3 Meeting #129bis	R3-25xxxx

Prague, Czech Republic, 13 – 17 October, 2025

Title:	[Draft] Reply LS on Continuous MDT

Response to:	R3-256523 - S5-254110

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	Nokia [to be RAN3]

To:	SA5, SA3

CC:

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 would like to thank SA5 on their LS and the introduced solutions for Continuous MDT.

RAN3 carefully analyzed the Continuous MDT solutions provided by SA5. RAN3 would like to note that in 4.1.1.9.x of S5-254111, it is stated: “This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request.”. However, this does not fulfil RAN3’s requirement that Continuous MDT is identified by the RAN through specific Trace Reference(s) and not by the TRSR Prefix Configuration parameter.

In addition, based on the incoming LS from SA5, RAN3 would like clarifications about the area over which Continuous Management based MDT is configured. Also, RAN3 would like to ask SA5 to enhance their Continuous MDT solution to address scenarios where data collection is needed from a large number of UEs or where Continuous MDT needs to be configured across the PLMN.

Finally, RAN3 would also like to include SA3 in the Continuous MDT discussions so that SA3 can evaluate whether there exist security aspects involved due to the Continuous MDT solution.

2. Actions:

To SA5 : 	RAN3 would like to ask SA5 to address the above issues relevant to SA5.

To SA3: 	RAN3 would like to ask SA3 to evaluate the CRs attached in R3-256523- S5-254110 corresponding to Continuous MDT with respect to security aspects.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #130				  	17 - 21 	November 2025						Dallas, US

TSG-RAN WG3 Meeting #131				  	09 -13 	February 2026					Göteborg, Sweden