3GPP TSG-RAN WG3 Meeting #129bis	R3-257102

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	10.5.1

Source:	Nokia

Title:	AI/ML Use Cases for 6G

Document for:	Decision

# 1	Introduction

We here provide initial discussion relative to RAN3 aspects of the following objective for Study on 6G Radio [RP-252912]:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

5G paved the path towards an AI/ML enabled architecture, with AI/ML introduced in the RAN, OAM, Core Network and UE for different use cases. AI/ML is a key enabler to enhance the RAN and Core functions to support various applications and services through efficient handling and processing of big amount of data and by taking actions in a proactive manner. In 5G the AI/ML functionality (including the data collection framework) has been fragmented to different domains and we believe that for a successful 6G architecture the AI/ML functionality should be an inbuilt functionality, spanning across the different domains. This would give rise to a truly AI native architecture providing a unified framework which will simplify support of new use cases and that may capitalize on the benefits in performance that can be achieved by AI/ML through better coordination and interoperability across the different domains.

# 2	Discussion

## 2.1 	6G AI/ML RAN baseline use cases

AI/ML discussions started in 3GPP in Rel-17 as a RAN3 led study item which standardized support for AI/ML in Rel-18 over three legacy SON use cases, namely AI/ML Mobility Optimization, AI/ML Load Balancing and AI/ML Energy Saving. In Rel-19, two additional use cases were standardized, namely AI/ML Slicing and AI/ML Coverage and Capacity Optimization (CCO). Two key enablers for those use cases have been UE Trajectory Prediction and UE Performance Feedback, as a feedback mechanism to enable the entity that makes AI/ML Inference determine whether the inference action was a good one or not. Analyzing and standardizing these AI/ML use cases took a lot of effort and was based on a common understanding of benefit of introducing AI/ML in those use cases. The study and work provided good insights on the potential of introducing AI/ML in NG-RAN. We therefore believe that for a native 6G system, the standardized AI/ML use cases in 5G need to also be supported in 6G. Based on this, the 6G RAN may benefit from AI/ML-based optimisations from the first 6G release.

Proposal 1: For an AI/ML native 6G system, all the use cases we considered in NG-RAN AI/ML in 5G need to be supported from the first 6G release.

For 5G, the listed use cases were studied and standardized one by one, and interactions between use cases could not be analysed in general. Creation of a unified 6G framework for the RAN use cases will also be the opportunity to revisit each of these use cases and analyse combinations of use cases and interactions between them, which could provide additional insights towards their standardization.

Such combinations and interactions could be:

- Energy Efficiency of a slice: An MNO may deploy a slice to provide a specific service, but may also need to monitor and optimise the energy consumption and energy efficiency of the slice. We believe 6G native AI/ML support should enable such optimization.

- Interaction between mobility optimization and CCO: Trajectory prediction has been a key enabler for the use case of AI/ML mobility Optimization. A source RAN node can request from a target node measured trajectory and may additionally report to the target node in a Handover Request message a predicted trajectory, measured over the cells of the target RAN node. However, CCO actions at the target RAN node may affect the statistics of the observed trajectories by the source. Subsequently, it may impact the predicted trajectory that a source node sends to the target node.

Proposal 2: Study a unified RAN AI/ML framework for 6G enabling combinations of NG-RAN use cases and the needed interactions between them.

## 2.2 Use Case Enablers

The MDT mechanism was introduced in 3GPP in Rel-10 and served as a mechanism to enable operators to troubleshoot network issues such as network coverage holes or UE signal strength issues in different areas. Over the releases, it was optimized and developed into a powerful mechanism to collect at the management plane data from the UEs and RAN, hence serving the use case of centralized SON. Since its introduction aimed to troubleshoot network issues at the operator’s network, an important principle was to allow the selection of as many UEs as possible. For this reason, logged and immediate MDT were configured separately to different UEs, since statistically this would maximize the possible number of UEs that could provide measurements. At the same time a key principle was to respect user privacy and confidentiality and to only collect data from UEs that have provided consent. Another major principle of MDT was to reuse available RRC Configurations in the RAN and to minimize possible impacts to the UEs.

Even though the original use case of MDT introduction is still valid, additional use cases emerged in 5G related to data collection for AI/ML. The fundamental principles of MDT make it a suitable data collection technique also for AI/ML data. Therefore, MDT framework has been considered as a baseline for the data collection in AI/ML for NG-RAN in 5G era. In our view, MDT framework can serve as a baseline for AI/ML data collection also in 6G, so that legacy measurements can be made available to the management system.

Proposal 3: MDT framework serves as a baseline for AI/ML data collection also in 6G, so that legacy measurements can be made available to the management system.

One entity that may train an AI/ML Model related to the RAN3 AI/ML use cases is OAM. Reusing MDT for AI/ML data collection in OAM complies with the legacy framework that aims to gather the data in the TCE. However, as part of the emerging 6G AI/ML use cases it may be necessary to collect data in different entities in the RAN e.g., for AI/ML Model Training at the 6G Node B. The current MDT framework doesn’t support such use cases since it assumes that the ultimate owner of the data is the OAM/TCE. Thus, the MDT framework would need enhancements to allow termination of the data not only in the OAM but also in the RAN.

Proposal 4: Investigate enhancements of the MDT framework to also allow data collection with different termination points in the RAN and OAM.

## 2.3 	New Use Cases

AI/ML for NG-RAN in 5G considered data collection from a local node (training the AI/ML Model), from its neighbours and from the UE. Even though, the NG-RAN AI/ML work did not have any UE impacts, existing UE measurements were in scope of the data collection. Under a unified 6G framework we think that it is important to additionally enable use of information provided by the application layer, and therefore the QoE measurement collection framework should be reused for data collection in 6G.

Proposal 5: The QoE measurement collection framework is reused for application layer feedback.

In 5G discussions, a key mechanism to evaluate the performance of an AI/ML action has been the configuration of collection and reporting by a source node of UE Performance feedback from a target node. UE Performance comprises a number of UE Performance metrics such as UL/DL UE Average Packet Delay, UL/DL UE Average Throughput and UL/DL Average Packet Loss. The source node, by taking into account UE Performance feedback, can determine the behavior of the QoS performance of the UE at the target node and through this determine if a handover decision was a good one (if the UE Performance improved or remained the same) or was a poor one (if the UE Performance reduced). Quality of experience (QoE) of a user is an important metric which is often replaced by QoS, simply because, for most practical scenarios, the latter is easier to measure.

In our view, the RAN visible QoE metrics can be exploited as a complement to (or instead of) the UE Performance feedback introduced in 5G to give a different dimension with respect to the experience of a user after a handover. However, the available RAN visible QoE metrics are, at the moment, limited to application layer buffer level and playout delay for media startup. We propose to analyse in the present study item additional metrics, e.g. QoE related, that can be collected at the application layer and made visible at the RAN. Figure 1 shows an example of UE Handover to a target node where feedback could be based on RVQoE metrics instead of (or together with) the QoS-level metrics introduced in 5G.

Figure 1 UE Performance feedback with RVQoE metrics.

As discussed above, in order to build a unified 6G system it is important to build a framework where different RAN use cases interoperate. As QoE related feedback has a role similar to UE Performance feedback, the impact of AI/ML-based network actions related to e.g., Energy Saving, Mobility Optimization, to name a few examples, on QoE can be evaluated. This could imply:

- Improving mobility decisions based on enhanced UE performance feedback, that may include application layer feedback.

- Combined performance optimization and energy saving actions in the 6G RAN.

- Combined energy saving actions in the UE and in the 6G RAN, based on multi-domain coordination potentially involving the UE, the RAN and the CN.

Proposal 6: RAN3 to study enhancements to QoE measurements, e.g., through additional metrics that can be collected at the application layer and made visible at the RAN, and evaluate the impact of already analysed AI/ML use cases e.g., Energy Saving, Mobility Optimization, on QoE.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Proposal 1: For an AI/ML native 6G system, all the use cases we considered in NG-RAN AI/ML in 5G need to be supported from the first 6G release.

Proposal 2: Study a unified RAN AI/ML framework for 6G enabling combinations of NG-RAN use cases and the needed interactions between them.

Proposal 3: MDT framework serves as a baseline for AI/ML data collection also in 6G, so that legacy measurements can be made available to the management system.

Proposal 4: Investigate enhancements of the MDT framework to also allow data collection with different termination points in the RAN and OAM.

Proposal 5: The QoE measurement collection framework is reused for application layer feedback.

Proposal 6: RAN3 to study enhancements to QoE measurements, e.g., through additional metrics that can be collected at the application layer and made visible at the RAN, and evaluate the impact of already analysed AI/ML use cases e.g., Energy Saving, Mobility Optimization, on QoE.