3GPP TSG-RAN WG3 Meeting #129bis	R3-257119

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	13.3

Source:	InterDigital, Inc.

Title:	NR ISAC RAN-CN Procedures and Signaling

Document for:	Discussion and Decision

- Introduction

The SID on Integrated Sensing and Communication (ISAC) for NR was approved in RAN#108 [1] and revised at RAN#109 (FS_Sensing_NR_bis) [2]. The objective of the study is to evaluate the performance of intra-gNB monostatic sensing (i.e., single TRP with co-located sensing transmitter and receiver) – RAN1 leads on evaluation and RAN3 addresses network architecture and RAN-CN procedures. This contribution presents technical views for AI 13.3 on RAN–CN procedures and signaling supporting gNB-based monostatic sensing for UAV targets.

- Procedures and signalling baseline

Per the SID [2], the RAN3#129b agenda [3] for AI 13.3 covers RAN-CN procedures and signalling for gNB-based mono-static sensing targeting UAVs, with FR1 prioritised, no UE impacts, and no inter-gNB coordination is in the scope of the study. Applicability to gNB bi-static sensing may be considered only where it does not introduce additional architecture impacts. Coordination with SA2 is expected as needed. The companion agenda item AI 13.2 addresses RAN architecture [2], [3].

Per our companion tdoc in [4], sensing should be executed inside the gNB following the CU/DU split, and what leaves the gNB is processed sensing output (measurement-level and/or object/track-level) with possible associated quality indicators. Carriage of sensing control and results uses a single NG plane: either N2 (NG-c) or N3 (NG-u).

A key premise is resource control, with sensing occasions, beams, timing, and bandwidth configured and scheduled by the gNB. This approach is consistent with existing CU/DU responsibilities and scheduler ownership. The Core Sensing Functions (SFs) can provide task intent, constraints, policy, and authorization, and receive results, but the Core does not select the radio resources.

Observation-1: To preserve existing NR architecture and ease-of-deployment, sensing should be done with gNB in control of the resources.

Proposal-1: Study RAN–CN procedures in which the CN/SF requests sensing, the gNB processes requests into sensing occasions under gNB-controlled radio resource selection, and the gNB returns processed results with associated quality indicators.

- Capability, discovery and authorization

Sensing capability can be discovered and authorized primarily through OAM and core policy, which avoids unnecessary AMF impacts. Where operators desire runtime visibility, the gNB may include an optional capability summary during setup to describe supported measurement/result types, applicable cells or bands, and configured limits on reporting rate or latency. Authorization and consent remain core responsibilities while the RAN enforces the admission of tasks.

Observation-2: OAM-centric discovery provides sufficient visibility with low control-plane burden.

Proposal-2: Study a runtime capability exchange, with an optional NG indication to convey brief capability summary.

- Sensing control

With on-demand sensing control, the CN, via the SF, can trigger sensing execution at the gNB and obtain corresponding sensing result set. The request can include scope (cells/areas/beams), outputs (measurement, object/track, etc.), required elements (e.g., range/ToA, Doppler, AoA/AoD, received power/SINR, RCS, micro-Doppler, etc.), timing, quality indicator, and retention/privacy policies. The gNB can return the requested payload or indicate a failure cause.

A publish/subscribe exchange establishes an ongoing flow of results under parameters agreed at setup. The subscription indicates periodic or event-driven reporting, the cadence or trigger conditions, output level(s), quality indicators, report rate, operational scope (cells/TRPs), etc. After acceptance, the gNB issues reports until expiry, suspension, modification, or termination. Suspension pauses reporting without state loss; termination deletes the context. The procedures are realization-agnostic and work with either an indirect AMF/LMF path or a direct NG-RAN↔SF path, using NGAP-native or NGAP-container mapping. Carriage of sensing control and results uses a single NG plane: either N2 (NG-c) or N3 (NG-u).

The procedures should work with either a control-plane realization—(i) NGAP-native over N2, (ii) indirect via AMF over N2 (LPP-style), or (iii) a direct NG-RAN↔SF control-plane reference point—or, if NG-u is selected, a user-plane realization using standard N3 user-plane. For the AMF-based control-plane options, either NGAP-native or NGAP-container mapping can be used.

Observation-3: On-demand plus subscription-based sensing controls can be studied.

Event-driven subscriptions deliver results only when configured conditions are met, complementing periodic reporting and reducing control-plane load. The baseline conditions cover appearance or disappearance within a defined scope, track-state change beyond thresholds (position/velocity), quality indicator dropping below a minimum threshold, ageing when no update occurs within a set interval, and changes in object count. Conditions may be combined and evaluated in the gNB. Each event report can carry subscribed outputs (measurement-level and/or object/track-level), event identifier, sensing/decision timestamps, quality indicator, etc.

Observation-4: Event-based triggers enable timely notifications with bounded control-plane traffic.

Proposal-3: Study on-demand, periodic, and event-triggered reporting with corresponding conditions.

- NG mapping

In the direct RAN–CN connection, NG-RAN exchanges sensing control and reports with a core-hosted SF over a new NG control-plane reference point that does not involve the AMF. This reduces AMF involvement and can simplify budgeting for periodic reporting. In the indirect realization, sensing control is carried via AMF/LMF over N2 using NGAP/LCS-aligned mechanisms, maximizing reuse of control-plane principles such as logging and charging. The application-level procedures are identical in both cases; only the NG mapping and packaging differ.

Two NG mappings may be envisaged. An NGAP-native mapping introduces dedicated procedures and IEs for sensing control and reporting, while an NGAP-container mapping reuses NGAP by encapsulating requests, publish/subscribe, and report payloads in a sensing container. Control remains on NG: for the AMF-based realization this is over N2, and for the direct path it is the new NG control-plane reference point. Carriage follows a single NG plane consistent with the architecture: either N2 (NG-c) or N3 (NG-u).

Observation-5: NG mapping can be based on direct RAN-CN (via SF) or indirect AMF realization with NGAP-native or NGAP-container mapping.

Proposal-4: Study both direct NG-RAN↔SF and indirect AMF realizations with NGAP-native or NGAP-container mapping, with carriage on a single NG plane (NG-c or NG-u).

- Conclusions

This paper made the following proposals regarding RAN-CN procedures and signaling enabling NR ISAC:

Proposal-1: Study RAN–CN procedures in which the CN/SF requests sensing, the gNB processes requests into sensing occasions under gNB-controlled radio resource selection, and the gNB returns processed results with associated quality indicators.

Proposal-2: Study a runtime capability exchange, with an optional NG indication to convey brief capability summary.

Proposal-3: Study on-demand, periodic, and event-triggered reporting with corresponding conditions.

Proposal-4: Study both direct NG-RAN↔SF and indirect AMF realizations with NGAP-native or NGAP-container mapping, with carriage on a single NG plane (NG-c or NG-u).

- References

[1] RP-251861, “New SID: Study on Integrated Sensing And Communication (ISAC) for NR,” RAN#108, Prague, Czech Republic, June 2025.

[2] RP-252819, “Study on Integrated Sensing And Communication (ISAC) for NR (FS_Sensing_NR_bis),” RAN#109, Beijing, China, September 2025.

[3] R3-250921, “RAN3#129b Agenda”, Prague, Czech Republic, October 2025.

[4] R3-257118, InterDigital, Inc., “ISAC Network Architecture”, Prague, Czech Republic, October 2025.