3GPP TSG-RAN WG3 #129bis                                         R3-257140

Prague, Czech, Oct. 13~17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1589 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TS 38.423 for NES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The criticality of the following IEs are not aligned between tabular and ASN.1: - Cell A ID IE in OD-SIB1 CONFIGURATION PROVISION RESPONSE; - Cell A ID IE in OD-SIB1 CONFIGURATION PROVISION FAILURE; - NES Cell ID IE in OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE; - Cell A ID IE in OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE; - Provision Status IE in OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE; In addition, OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message is captured as ‘ODSIB1ConfigurationProvisionStatus’ in ASN.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the criticality of Cell A ID IE, NES Cell ID IE, and Provision Status IE to ‘reject’ to align the tabular and ASN.1. - Revise ‘ODSIB1ConfigurationProvisionStatus’ in ASN.1 as ‘ODSIB1ConfigurationProvisionStatusUpdate’. - Other minor changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The criticality of Cell A ID IE, NES Cell ID IE, and Provision Status IE are not aligned between tabular and ASN.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.17.2, 8.4.17.3, 9.1.3.31, 9.1.3.32, 9.1.3.33, 9.3.3, 9.3.4, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

### 8.4.17	OD-SIB1 Configuration Provision

#### 8.4.17.1	General

The purpose of the OD-SIB1 Configuration Provision procedure is to enable an NG-RAN node1 to provide OD-SIB1 configuration information to an NG-RAN node2 and to request the NG-RAN node2 to transmit the provided OD-SIB1 configuration information.

The procedure is also used to enable the NG-RAN node1 to request the NG-RAN node2 to stop transmission of the OD-SIB1 configuration.

The procedure uses non-UE -associated signaling.

#### 8.4.17.2	Successful Operation

Figure 8.4.17.2-1: OD-SIB1 Configuration Provision procedure, successful operation

The NG-RAN node1 initiates the procedure by sending the OD-SIB1 CONFIGURATION PROVISION REQUEST message to the NG-RAN node2.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "start", the NG-RAN node2 shall, if supported, transmit the provided OD-SIB1 Configuration in SIBx and reply with the OD-SIB1 CONFIGURATION PROVISION RESPONSE message. The NG-RAN node2 stores the OD-SIB1 Configuration information.

If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, transmit the OD-SIB1 Configuration in the cell indicated by the Cell A ID IE and includes the Cell A ID IE indicating the same cell in the OD-SIB1 CONFIGURATION PROVISION RESPONSE message.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "start" and the NES Cell ID IE indicates a cell for which the NG-RAN node2 is already transmitting the OD-SIB1 Configuration:

-	If the request is for a cell already transmitting the OD-SIB1 configuration for the cell identified by the NES Cell ID IE, the NG-RAN node2 shall, if supported, stop the ongoing transmission and start transmission of the newly provided OD-SIB1 Configuration in SIBx. The NG-RAN node2 replaces the stored OD-SIB1 Configuration information with the newly received OD-SIB1 Configuration information.

-	If the request is for a new cell indicated by the Cell A ID IE, the NG-RAN node2 shall, if supported, start transmitting the provided OD-SIB1 Configuration in SIBx in the cell indicated by Cell A ID IE.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, is set to "stop" the NG-RAN node2 shall stop broadcasting OD-SIB1Configuration for the cell indicated by NES Cell ID IE in SIBx. The NG-RAN node2 removes the stored OD-SIB1 Configuration.

#### 8.4.17.3	Unsuccessful Operation

Figure 8.4.17.3-1: OD-SIB1 Configuration Provision procedure, unsuccessful operation

If the NG-RAN node2 cannot transmit the provided OD-SIB1 Configuration in SIBx, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value.

If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message and the NG-RAN node2 cannot transmit the provided OD-SIB1 Configuration in SIBx of the cell indicated by the Cell A ID IE, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message. The NG-RAN node2 includes the Cell A ID IE in the OD-SIB1 CONFIGURATION PROVISION FAILURE message.

-------------------------------------------Next change-------------------------------------------

#### 9.1.3.31	OD-SIB1 CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration will be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.32	OD-SIB1 CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration cannot be provided by the NG-RAN node2 as requested by the NG-RAN node1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.33	OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted OD-SIB1 configuration provision is being stopped.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | reject |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the OD-SIB1 configuration transmission. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

-------------------------------------------Next change-------------------------------------------

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

NotificationControlIndication,

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

DataCollectionUpdate,

ODSIB1ConfigurationProvisionRequest,

ODSIB1ConfigurationProvisionResponse,

ODSIB1ConfigurationProvisionFailure,

ODSIB1ConfigurationProvisionStatusUpdate,

LTMConfigurationUpdate,

LTMConfigurationUpdateAcknowledge,

LTMConfigurationUpdateFailure,

CSIRSCoordinationRequest,

CSIRSCoordinationResponse,

CellSwitchNotification,

TAInformationTransfer,

LTMCancel,

CLI-Indication

FROM XnAP-PDU-Contents

id-handoverPreparation,

id-sNStatusTransfer,

id-handoverCancel,

id-notificationControl,

id-retrieveUEContext,

id-rANPaging,

id-xnUAddressIndication,

id-uEContextRelease,

id-secondaryRATDataUsageReport,

id-sNGRANnodeAdditionPreparation,

id-sNGRANnodeReconfigurationCompletion,

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation,

id-mNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeinitiatedSNGRANnodeRelease,

id-sNGRANnodeCounterCheck,

id-sNGRANnodeChange,

id-activityNotification,

id-rRCTransfer,

id-xnRemoval,

id-xnSetup,

id-nGRANnodeConfigurationUpdate,

id-e-UTRA-NR-CellResourceCoordination,

id-cellActivation,

id-reset,

id-errorIndication,

id-privateMessage,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-conditionalHandoverCancel,

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-scgFailureIndication,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-mobilitySettingsChange,

id-accessAndMobilityIndication,

id-cellTrafficTrace,

id-RANMulticastGroupPaging,

id-scgFailureInformationReport,

id-scgFailureTransfer,

id-f1CTrafficTransfer,

id-iABTransportMigrationManagement,

id-iABTransportMigrationModification,

id-iABResourceCoordination,

id-retrieveUEContextConfirm,

id-cPCCancel,

id-partialUEContextTransfer,

id-rachIndication,

id-dataCollectionReportingInitiation,

id-dataCollectionReporting,

id-ODSIB1ConfigurationProvision,

id-ODSIB1ConfigurationProvisionStatusUpdate,

id-lTMConfigurationUpdate,

id-cSIRSCoordination,

id-cellSwitchNotification,

id-tAInformationTransfer,

id-lTMCancel,

id-cLI-Indication

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport				|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

scgFailureIndication					|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace						|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

rachIndication							|

dataCollectionReporting					|

oDSIB1ConfigurationProvisionStatusUpdate		|

cellSwitchNotification					|

tAInformationTransfer					|

lTMCancel,

...

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

oDSIB1ConfigurationProvision		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ODSIB1ConfigurationProvisionRequest

SUCCESSFUL OUTCOME				ODSIB1ConfigurationProvisionResponse

UNSUCCESSFUL OUTCOME			ODSIB1ConfigurationProvisionFailure

PROCEDURE CODE					id-ODSIB1ConfigurationProvision

CRITICALITY						reject

}

oDSIB1ConfigurationProvisionStatusUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ODSIB1ConfigurationProvisionStatusUpdate

PROCEDURE CODE			id-ODSIB1ConfigurationProvisionStatusUpdate

CRITICALITY				ignore

}

-------------------------------------------Next change-------------------------------------------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

AerialUESubscriptionInformation,

A2XPC5QoSParameters,

BitRate,

Cause,

CellAndCapacityAssistanceInfo-EUTRA,

CellAndCapacityAssistanceInfo-NR,

CellAssistanceInfo-EUTRA,

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION RESPONSE

--

-- **************************************************************

ODSIB1ConfigurationProvisionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionResponse-IEs}},

...

}

ODSIB1ConfigurationProvisionResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION FAILURE

--

-- **************************************************************

ODSIB1ConfigurationProvisionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ ODSIB1ConfigurationProvisionFailure-IEs}},

...

}

ODSIB1ConfigurationProvisionFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause									CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE

--

-- **************************************************************

ODSIB1ConfigurationProvisionStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionStatusUpdate-IEs}},

...

}

ODSIB1ConfigurationProvisionStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE optional }|

{ ID id-ProvisionStatus							CRITICALITY reject	TYPE ProvisionStatus							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

ProvisionStatus ::= ENUMERATED {stopped, ...}

-------------------------------------------Next change-------------------------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

id-partialUEContextTransfer										ProcedureCode ::= 49

id-rachIndication												ProcedureCode ::= 50

id-dataCollectionReportingInitiation							ProcedureCode ::= 51

id-dataCollectionReporting										ProcedureCode ::= 52

id-ODSIB1ConfigurationProvision									ProcedureCode ::= 53

id-ODSIB1ConfigurationProvisionStatusUpdate							ProcedureCode ::= 54

id-lTMConfigurationUpdate										ProcedureCode ::= 55

id-cSIRSCoordination											ProcedureCode ::= 56

id-cellSwitchNotification										ProcedureCode ::= 57

id-tAInformationTransfer										ProcedureCode ::= 58

id-lTMCancel													ProcedureCode ::= 59

id-cLI-Indication												ProcedureCode ::= 60

id-scgFailureIndication											ProcedureCode ::= 61

-------------------------------------------End of changes-------------------------------------------