3GPP TSG-RAN WG3 Meeting #129-bis	R3-257156

Prague, Czech Republic, 13th ~17th Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR |  | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 corrections to inter-CU MCG LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Jio Platforms, Lenovo, Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. Rel-19 Mobility supporting inter-CU MCG LTM with SN addition, where the source gNB can be not operating under NR-DC, the Xn-U Address Indication message should be optional. 2. During the preparation of inter-CU MCG LTM, step 4a may need to be repeated multiple times, as the message may come from multipole target MNs.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Change step 4a Xn-U Address Indication message to be an optional procedure 2. A note is added after the step 4a stating that separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses from multiple MNs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specificaion will lead to misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423CR1593 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 10.7.2	MR-DC with 5GC

Inter-MN handover with/without MN initiated SN change is used to transfer UE context data from a source MN to a target MN while the UE context at the SN is kept or moved to another SN. During an Inter-Master Node handover, the target MN decides whether to keep or change the SN (or release the SN, as described in clause 10.8). Only intra-RAT Inter-Master node handover with/without SN change is supported (e.g. no transition from NGEN-DC to NR-DC).

Figure 10.7.2-1: Inter-MN handover with/without MN initiated SN change procedure

Figure 10.7.2-1 shows an example signalling flow for inter-MN handover with or without MN initiated SN change:

NOTE 1:	For an Inter-Master Node handover without Secondary Node change, the source SN and the target SN shown in Figure 10.7.2-1 are the same node.

1.	The source MN starts the handover procedure by initiating the Xn Handover Preparation procedure including both MCG and SCG configuration. The source MN includes the source SN UE XnAP ID, SN ID and the UE context in the source SN in the Handover Request message.

NOTE 2:	The source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and the QMC configuration information managed by the SN and to allow provision of data forwarding related information before step 1.

2.	If the target MN decides to keep the UE context in source SN, the target MN sends SN Addition Request to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN allowing delta configuration, the target MN sends the SN Addition Request to the target SN including the UE context in the source SN that was established by the source MN. Otherwise, the target MN may send the SN Addition Request to the target SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN.

3.	The (target) SN replies with SN Addition Request Acknowledge. The (target) SN may include the indication of the full or delta RRC configuration.

NOTE 2a0: Void.

3a.	For SN terminated bearers using MCG resources, the target MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The target MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform the handover, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The target MN indicates to the source MN that the UE context in the SN is kept if the target MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

5a/5b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The (source) SN acknowledges the release request. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. If the indication as the UE context kept in SN is included, the SN keeps the UE context.

5c.	The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

6.	The source MN triggers the UE to perform handover and apply the new configuration.

7/8.	The UE synchronizes to the target MN and replies with MN RRC reconfiguration complete message.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE 2a1:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

11a. The source SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2.

NOTE 2a2:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS is stopped.

11b. The source MN sends the Secondary RAT Report message to AMF to provide information on the used NR/E-UTRA resource.

12.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target forwards the SN Status to the target SN, if needed.

13.	If applicable, data forwarding takes place from the source side. If the SN is kept, data forwarding may be omitted for SN terminated bearers or QoS flows kept in the SN.

14-17.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

NOTE 3:	If new UL TEIDs of the UPF for SN are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

18.	The target MN initiates the UE Context Release procedure towards the source MN.

19.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE contest kept indication was included in the SN Release Request message in step 5.

Inter-MN MCG LTM with SN

Figure 10.7.2-2: Inter-MN MCG LTM with Secondary Node

Figure 10.7.2-2 shows an example signaling flow for Inter-MN MCG LTM with Secondary Node.

NOTE 1:	For an Inter-MN MCG LTM without SN change, the source SN and the target SN shown in Figure 10.7.2-2 are the same node.

NOTE 2:	For an Inter-MN MCG LTM with SN addition, the source SN and steps involving the source SN in Figure 10.7.2-2 are ignored.

NOTE 3:	Figure 10.7.2-2 is not applied for an Inter-MN MCG LTM with SN changed to another SN, which is not supported.

1.	The source MN starts LTM preparation by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the source SN UE XnAP ID, SN ID and the UE context in the source SN in the Handover Request message. The source MN may include the common CSI resource configuration.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in SCG part of the reference configuration. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. Within the SN Addition Request message, the candidate MN also includes the LTM related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to an LTM and to enable the SN to identify requests related to the same UE.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The candidate MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform LTM, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3. The candidate MN(s) may also provide the CSI report configuration.

4a.	For inter-MN MCG LTM without SN change, the source MN sends the Xn-U Address Indication message to the (source) SN. This Xn-U Address Indication message notifies LTM to the (source) SN,which may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN. This step is not applicable for inter-MN MCG LTM with SN addition.

NOTE 4a:	Separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses of different target MNs. In this case, it is up to the source MN and SN implementations to make sure that the Early Status Transfer message(s) from the SN, if any, is forwarded to the right target MN. The Xn-U Address Indication procedure may further be initiated to indicate to the SN to stop already initiated early data forwarding for some SN-terminated bearers, if they are no longer subject to data forwarding due to the cancellation of the prepared LTM cells.

5.	The source MN sends an LTM Configuration Update message to the candidate MN(s) to update the LTM configurations of candidate cell(s). The source MN may include the common CSI-RS resource configuration.

6.	The candidate MN(s) sends the LTM Configuration Update Acknowledge message to the source MN. The candidate MN(s) may also provide the CSI-RS report configuration.

7.	The source MN sends an RRC reconfiguration message to the UE, including the LTM candidate configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains an MCG configuration and possibly an SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3.

8.	The UE applies the RRC reconfiguration message received in step 7, stores the LTM candidate configuration and replies to the MN with an RRCReconfigurationComplete message.

9.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

10a. The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

10b. The candidate MN(s) sends the TA Information Transfer message to the source MN if early TA acquisition is performed to the candidate MN(s).

11.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source MN, if the L1 measurement configuration in RRCReconfiguration is received in step 7. The UE can also perform L3 measurement reporting to the source MN, including beam level measurement results on cell(s) which are configured as LTM candidate cell(s) according to the received network configuration.

12.	The source MN decides to execute cell switch to a target cell.

13.	The source MN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target cell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

14.	The source MN sends the Cell Switch Notification message to the target MN to indicate the initiation of Cell Switch command to the UE.

15.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

16. The UE sends an MN RRCReconfigurationComplete* message to the target MN, which may include an SN RRCReconfigurationComplete** message to the target SN.

17.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE x:	The order the UE performs Random Access towards the MN (step 15) and performs the Random Access procedure towards the (target) SN (step 17) is not defined.

18.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

19.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

20a/b.	The source MN may send SN Release Request message to the (source) SN including a Cause indicating MCG mobility if no candidate cells exist in source MN. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. The (source) SN acknowledges the release request.

20c.The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information.

21a/b. The (source) SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2. The source MN sends the Secondary RAT Data Usage Report message to AMF to provide information on the used NR resource.

22a/b. For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN.

23.	 If applicable, data forwarding takes place from the source (i.e. source MN or source SN). If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

24-27.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

28. The target MN (i.e. the new source MN) sends the LTM Configuration Update message to the other candidate MNs and may also include the original source MN to update the security information.

28a. The other potential candidate MN sends the SN Modification Request message to the other candidate SN to update the security information,

28b. The other potential candidate SN sends the SN Modification Request Acknowledge message to the other potential candidate MN,

29. The candidate MN(s) responds the LTM Configuration Update Acknowledge message to the source MN.

30. The target MN may send the UE Context Release message to inform the source MN to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the source MN. Any ongoing data forwarding may continue.

31.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE context kept indication was included in the SN Release Request message in step 20a.

END OF CHANGES