3GPP TSG-RAN WG3 #129bis	R3-257194

Prague - Czech, 13th – 17th Oct. 2025

Agenda Item:	10.5 AI/ML for NG-RAN use cases

Source:	NTT DOCOMO INC.

Title:	Use cases of AI/ML for NG-RAN in 6G

Document for:	Discussion

Introduction

In 5 GA, many use cases were studied and supported as AI/ML for NG-RAN functions [1][2]. As described in (8)-a of 6G SID [3], RAN3 is required to identify necessary use cases in 6G, including functions supported in 5 G, considering trade-off between e.g., performance, and complexity.

Discussion

Legacy Use cases

We believe that the following features, which were mainly introduced in 5G, should be supported in 6G as a baseline.

Energy saving (measured energy cost)

Mobility enhancement (UE trajectory prediction and feedback)

Load balancing (UE performance feedback)

Slicing (UE performance feedback, slice resource prediction and reporting)

CCO (CCO issue prediction)

This is because all functions are not limited to a specific deployment/business and are useful for operators. In addition, since these functions improve energy efficiency, performance, reliability, etc., by coordination between multiple RAN nodes, it is important to support these functions on day1 because it is considered that introducing these functions on day1 for more RAN nodes will provide a greater gain than partially introducing them in a later release.

Proposal 1:	The use cases introduced in 5G should be supported in 6G day1 for AI/ML for RAN functions including following use cases:

- Energy saving (measured energy cost)

- Mobility enhancement (UE trajectory prediction and feedback)

- Load balancing (UE performance feedback)

- Slicing (UE performance feedback, slice resource prediction and reporting)

- CCO (CCO issue prediction)

Enhancement of existing use case

As shown in 6G SID (1)-b [3] and the current version of TR 38.914 [4], energy efficiency improvement is important especially for operators. While measured energy cost between RAN nodes was supported in 5G, energy cost prediction and finer granularity of energy cost (e.g. per cell) were discussed but did not converge. We think it is worth reconsidering these points in the new generation specification. This is because assuming AI/ML native 6G, it is not desirable to support the exchange of predicted information related to many other RAN nodes, but not for energy consumption. Therefore, we propose further study on AI/ML energy saving feature enhancement.

Proposal 2:	Enhancement on AI/ML assisted energy saving function for RAN should be further studied, e.g. for energy consumption prediction, energy consumption per cell/beam.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	The use cases introduced in 5G should be supported in 6G day1 for AI/ML for RAN functions including following use cases:

- Energy saving (measured energy cost)

- Mobility enhancement (UE trajectory prediction and feedback)

- Load balancing (UE performance feedback)

- Slicing (UE performance feedback, slice resource prediction and reporting)

- CCO (CCO issue prediction)

Proposal 2:	Enhancement on AI/ML assisted energy saving function for RAN should be further studied, e.g. for energy consumption prediction, energy consumption per cell/beam.

References

- TR 37.817, “Study on enhancement for Data Collection for NR and EN-DC”

- TR 38.743, “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”

- RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo.

- RP-252882, “Study on 6G Scenarios and Requirements”