3GPP TSG-RAN WG3 Meeting #129bis	R3-256825

Prague, CZ, 13-17 Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1540 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Semi-Persistent CSI-RS activation/deactivation with TCI state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, CATT, China Telecom, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For each periodic CSI-RS resource, the candidate gNB provides each CSI-RS associated with pre-configured QCL-info (via qcl-InfoPeriodicCSI-RS, which indicates a TCI state ID). However, for SP CSI-RS, this is not possible. The reason is that SP CSI-RSs are pre-configured but only activated dynamically, based on needs determined from periodic SSB or CSI-RS measurements. Therefore, when the source gNB selects an SP CSI-RS for activation, it also needs to determine the appropriate TCI State and indicate it to the candidate gNB, this ensures that the SP CSI-RS can be properly activated with the correct TCI State. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TCI state information List IE in CSI-RS COORDINATION REQUEST message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Candidate gNB is unable to know the TCI State for SP CSI-RS activation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.1, 9.1.5.7, 9.4 (ASN.1) |  |  |  |  |  |  |  |  |
| .4 |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.473 CR 1606 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.11.1	CSI-RS Coordination

#### 8.11.1.1	General

The purpose of the CSI-RS Coordination procedure is to enable coordination of CSI-RS transmission. The procedure uses UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: CSI-RS Coordination procedure, successful operation

The NG-RAN node1 initiates the procedure by sending the CSI-RS COORDINATION REQUEST message to NG-RAN node2.

If the TCI State Information List IE is included in the CSI-RS COORDINATION REQUEST message, the NG-RAN node2 shall, if supported, use it for SP CSI-RS activation.

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### 9.1.5.7 	CSI-RS COORDINATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to coordinate the activation and deactivation of CSI-RS transmission for a UE at NG-RAN node2.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| CSI-RS Coordination Request | M |  |  |  | YES | reject |
| >L1 Measurements List |  | 0..1 |  |  | – |  |
| >>L1 Measurements Item |  | 1 .. <maxnoofCSIResourceConfigurationsPlus1> |  |  | – |  |
| >>>Request For Semi Persistent CSI-RS Resources | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the LTM-CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |
| >>>TCI State Information List |  | 0..1 |  |  | - | - |
| >>>>TCI state information Item IEs |  | 1 .. < maxnoofLTM-CSI-ResourcesPerSet> |  |  |  |  |
| >>>>Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [10]. | - |  |
| >CSI-RS Acquisition List |  | 0..1 |  |  | – |  |
| >>CSI-RS Acquisition Item |  | 1 .. < maxnoofCSIResourceConfigurationsPlus1> |  |  | – |  |
| >>>Request For Semi Persistent CSI-RS Resources | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) | Corresponds to information provided in the CSI-ResourceConfigId as defined in TS 38.331 [10]. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIResourceConfigurationsPlus1 | Maximum number of CSI Resource Configurations Plus 1. Value is 112. |
| maxnoofLTM-CSI-ResourcesPerSet | Maximum number of LTM CSI-RS resource per set. Value is 512. |

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### -- L

Local-NG-RAN-Node-Identifier ::= CHOICE {

full-I-RNTI-Profile-List					Full-I-RNTI-Profile-List,

short-I-RNTI-Profile-List					Short-I-RNTI-Profile-List,

choice-extension							ProtocolIE-Single-Container { { Local-NG-RAN-Node-Identifier-ExtIEs} }

}

Local-NG-RAN-Node-Identifier-ExtIEs XNAP-PROTOCOL-IES ::= {

{ ID id-Full-and-Short-I-RNTI-Profile-List		CRITICALITY ignore	TYPE Full-and-Short-I-RNTI-Profile-List	PRESENCE mandatory},

...

}

<< skip unchanged part >>

Layer1MeasurementRequest-List ::= SEQUENCE (SIZE(1..maxnoofCSIResourceConfigurationsPlus1)) OF Layer1MeasurementRequest-Item

Layer1MeasurementRequest-Item ::= SEQUENCE {

requestForSemiPersistentCSI-RSResources			ENUMERATED {activate, deactivate,...},

cSIResourceConfigurationID						CSIResourceConfigurationID,

tci-State-InformationList						Tci-State-InformationList 	OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { Layer1MeasurementRequest-Item-ExtIEs} }	OPTIONAL,

...

}

Layer1MeasurementRequest-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### -- T

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<< skip unchanged part >>

Tci-State-InformationList	::=  SEQUENCE (SIZE(1.. maxnoofLTM-CSI-ResourcesPerSet)) OF Tci-State-Information-Item

Tci-State-Information-Item	::= SEQUENCE {

jointorDLTCIStateID			JointorDLTCIStateID,

iE-Extensions				ProtocolExtensionContainer { { Tci-State-Information-Item-ExtIEs } }	OPTIONAL,

...

}

Tci-State-Information-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs						INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRAReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 256

maxnoofUEIDIndicesforMBSPaging				INTEGER ::= 4096

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMRBs									INTEGER ::= 32

maxnoofCellsforMBS							INTEGER ::= 8192

maxnoofMBSServiceAreaInformation			INTEGER ::= 256

maxnoofTAIforMBS							INTEGER ::= 1024

maxnoofAssociatedMBSSessions				INTEGER ::= 32

maxnoofMBSSessions							INTEGER ::= 256

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofPSCellsPerSN							INTEGER ::= 8

maxnoofNR-UChannelIDs						INTEGER ::= 16

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

maxnoofServedCellsIAB						INTEGER ::= 512

maxnoofServingCells							INTEGER ::= 32

maxnoofBHInfo								INTEGER ::= 1024

maxnoofTrafficIndexEntries					INTEGER	::=	1024

maxnoofTLAsIAB								INTEGER	::=	1024

maxnoofBAPControlPDURLCCHs					INTEGER ::= 2

maxnoofIABSTCInfo							INTEGER ::= 45

maxnoofSymbols								INTEGER ::= 14

maxnoofDUFSlots								INTEGER ::= 320

maxnoofHSNASlots							INTEGER ::= 5120

maxnoofRBsetsPerCell						INTEGER ::= 8

maxnoofRBsetsPerCell1						INTEGER ::= 7

maxnoofChildIABNodes						INTEGER ::= 1024

maxnoofPSCellCandidates						INTEGER ::= 8

maxnoofTargetSNs							INTEGER ::= 8

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofMTCItems								INTEGER ::= 16

maxnoofCSIRSconfigurations					INTEGER ::= 96

maxnoofCSIRSneighbourCells					INTEGER ::= 16

maxnoofCSIRSneighbourCellsInMTC				INTEGER ::= 16

maxnoofNeighbour-NG-RAN-Nodes				INTEGER ::= 256

maxnoofSRBs									INTEGER ::= 5

maxnoofSMBR									INTEGER ::= 8

maxnoofNSAGs								INTEGER ::= 256

maxnoofTargetSNsMinusOne					INTEGER ::= 7

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofSuccessfulPSCellChangeReports		INTEGER ::= 64

maxnoofUEsforRAReportIndications			INTEGER ::= 64

maxnoofPSCellsinCPAC						INTEGER ::= 8

maxnoofCPACexecutioncond					INTEGER ::= 2

maxnoofLBTFailureInformation				INTEGER ::= 64

maxnoofCellsTrajectoryPredict				INTEGER ::= 16

maxnoofCellsTrajectory						INTEGER	::=	16

maxFailedCellMeasObjects					INTEGER ::= 124

maxFailedMeasPerNode						INTEGER ::= 124

maxnoofUEReports							INTEGER ::= 16

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofSecurityConfigurations				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofThresholds							INTEGER ::= 8

maxnoofEarlyRACHResourcesID					INTEGER	::= 8

maxnoofLTMCells								INTEGER	::= 8

maxNoSSBs									INTEGER	::= 255

maxnoofTAList								INTEGER	::= 8

maxnoofPreambleIndexes						INTEGER ::= 64

maxnoofCSIResourceConfigurationsPlus1		INTEGER ::= 112

maxnoofNZP-CSI-RS-ResourcesPerSet			INTEGER ::= 64

maxnoofSRS-Resource							INTEGER ::= 64

maxnoofFailedSliceMeasObjects				INTEGER ::= 124

maxnoofSliceItemsforMDT						INTEGER ::= 1024

maxnoofAreaNTNforMDT						INTEGER ::= 32

maxnoofLTMCellsPlusOne						INTEGER ::= 9

maxnoofSCGSecurityConfigurations			INTEGER ::= 8

maxnoofLTM-CSI-ResourcesPerSet				INTEGER ::= 512

<<<<<<<<<<<<<<<<<<<< End of changes >>>>>>>>>>>>>>>>>>>>