3GPP TSG-RAN3 Meeting #129bis	R3-257255

# Prague, Czech Republic, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1578 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Xn support for on-demand SIB1 coordination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm, Jio Platforms , ZTE, Deutsche Telekom, Huawei, Samsung, NEC. Rakuten, Nokia, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As part of the Rel-19 WI on enhancements of NES for NR, several enhancements were introduced in XnAP to support on-demand SIB1 coordination. However, a few errors and improvements have been identified in the CR approved at RAN plenary: - Incorrect ASN.1 implementation of the criticality of several IEs Additionally, a few editorial modifications have also been identified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct ASN.1 implementation of the criticality of several IEs in OD-SIB1 configuration provision status update message. - Some editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect ASN.1 implementation of the criticality of several IEs introduced inconsistencies in the specification between the tables in clause 9.1.3.33 and the ASN.1 in clause 9.3.4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.17.1, 8.4.17.2, 8.4.17.3, 8.4.18.1, 8.4.18.2, 9.1.3.33, 9.3.3, 9.3.4, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: Submission Rev 1: Implement the R3#129bis agreed correction. |  |  |  |  |  |  |  |  |

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

*************** Next change ***************

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

OD-SIB1			On-demand SIB1

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

*************** Next change ***************

### 8.4.17	OD-SIB1 Configuration Provision

#### 8.4.17.1	General

The purpose of the OD-SIB1 Configuration Provision procedure is to enable an NG-RAN node1 to provide OD-SIB1 configuration information to an NG-RAN node2 and to request the NG-RAN node2 to transmit the provided OD-SIB1 configuration.

The procedure is also used to enable the NG-RAN node1 to request the NG-RAN node2 to stop transmission of the OD-SIB1 configuration.

The procedure uses non UE-associated signalling.

#### 8.4.17.2	Successful Operation

Figure 8.4.17.2-1: OD-SIB1 Configuration Provision procedure, successful operation

The NG-RAN node1 initiates the procedure by sending the OD-SIB1 CONFIGURATION PROVISION REQUEST message to the NG-RAN node2.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "start", the NG-RAN node2 shall, if supported, transmit the provided OD-SIB1 configuration in SIB26 and reply with the OD-SIB1 CONFIGURATION PROVISION RESPONSE message. The NG-RAN node2 stores the OD-SIB1 configuration information.

If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, transmit the OD-SIB1 configuration in SIB26 in the cell indicated by the Cell A ID IE and include the Cell A ID IE indicating the same cell in the OD-SIB1 CONFIGURATION PROVISION RESPONSE message.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "start" and the NES Cell ID IE indicates a cell for which the NG-RAN node2 is already transmitting the OD-SIB1 configuration:

-	If the request is for a cell already transmitting the OD-SIB1 configuration for the cell identified by the NES Cell ID IE, the NG-RAN node2 shall, if supported, stop the ongoing transmission and start transmission of the newly provided OD-SIB1 configuration in SIB26. The NG-RAN node2 replaces the stored OD-SIB1 configuration information with the newly received OD-SIB1 configuration information.

-	If the request is for a new cell indicated by the Cell A ID IE, the NG-RAN node2 shall, if supported, start transmitting the provided OD-SIB1 configuration in SIB26 in the cell indicated by Cell A ID IE.

If the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is set to "stop", the NG-RAN node2 shall stop broadcasting the OD-SIB1 configuration for the cell indicated by NES Cell ID IE in SIB26. The NG-RAN node2 removes the stored OD-SIB1 configuration information.

#### 8.4.17.3	Unsuccessful Operation

Figure 8.4.17.3-1: OD-SIB1 Configuration Provision procedure, unsuccessful operation

If the NG-RAN node2 cannot transmit the provided OD-SIB1 configuration in SIB26, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value.

If the Cell A ID IE is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message and the NG-RAN node2 cannot transmit the provided OD-SIB1 configuration in SIB26 of the cell indicated by the Cell A ID IE, the NG-RAN node2 shall send the OD-SIB1 CONFIGURATION PROVISION FAILURE message. The NG-RAN node2 includes the Cell A ID IE in the OD-SIB1 CONFIGURATION PROVISION FAILURE message.

#### 8.4.17.4	Abnormal Conditions

Void.

### 8.4.18	OD-SIB1 Configuration Provision Status Update

#### 8.4.18.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted OD-SIB1 configuration provision being stopped.

The procedure uses non UE-associated signalling.

#### 8.4.18.2	Successful Operation

Figure 8.4.18.2-1: OD-SIB1 Configuration Provision Status Update procedure, successful operation

The NG-RAN node2 initiates the procedure by sending the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message to the NG-RAN node1.

Upon reception of the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message with the Provision Status IE set to "stopped", the NG-RAN node1 shall consider that a previously admitted OD-SIB1 configuration for the cell identified by the NES Cell ID IE is no longer transmitted in the cell identified by the Cell A ID IE, if it is present. Else the NG-RAN node1 shall consider the NG-RAN node2 has stopped the OD-SIB1 broadcasting. The NG-RAN node2 removes the stored OD-SIB1 configuration information that is no longer valid.

#### 8.4.18.3	Unsuccessful Operation

Not applicable.

#### 8.4.18.4	Abnormal Conditions

Void.

*************** Next change ***************

#### 9.1.3.33	OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted OD-SIB1 configuration provision is being stopped.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | ignore |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | ignore |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the OD-SIB1 configuration transmission. | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** Unchanged text omitted***************

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

SNStatusTransfer,

UEContextRelease,

HandoverCancel,

*************** Unchanged text omitted***************

DataCollectionUpdate,

ODSIB1ConfigurationProvisionRequest,

ODSIB1ConfigurationProvisionResponse,

ODSIB1ConfigurationProvisionFailure,

ODSIB1ConfigurationProvisionStatusUpdate,

LTMConfigurationUpdate,

LTMConfigurationUpdateAcknowledge,

LTMConfigurationUpdateFailure,

CSIRSCoordinationRequest,

CSIRSCoordinationResponse,

CellSwitchNotification,

TAInformationTransfer,

LTMCancel,

CLI-Indication

*************** Unchanged text omitted***************

id-dataCollectionReportingInitiation,

id-dataCollectionReporting,

id-ODSIB1ConfigurationProvision,

id-ODSIB1ConfigurationProvisionStatusUpdate,

id-lTMConfigurationUpdate,

id-cSIRSCoordination,

id-cellSwitchNotification,

id-tAInformationTransfer,

*************** Unchanged text omitted***************

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport				|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

scgFailureIndication					|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace						|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

rachIndication							|

dataCollectionReporting					|

oDSIB1ConfigurationProvisionStatusUpdate		|

cellSwitchNotification					|

tAInformationTransfer					|

lTMCancel,

...

}

*************** Unchanged text omitted***************

oDSIB1ConfigurationProvision		XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ODSIB1ConfigurationProvisionRequest

SUCCESSFUL OUTCOME				ODSIB1ConfigurationProvisionResponse

UNSUCCESSFUL OUTCOME			ODSIB1ConfigurationProvisionFailure

PROCEDURE CODE					id-ODSIB1ConfigurationProvision

CRITICALITY						reject

}

oDSIB1ConfigurationProvisionStatusUpdate	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ODSIB1ConfigurationProvisionStatusUpdate

PROCEDURE CODE			id-ODSIB1ConfigurationProvisionStatusUpdate

CRITICALITY				ignore

}

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

*************** Unchanged text omitted***************

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION REQUEST

--

-- **************************************************************

ODSIB1ConfigurationProvisionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionRequest-IEs}},

...

}

ODSIB1ConfigurationProvisionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RequestType								CRITICALITY reject	TYPE RequestTypeChoice							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

RequestTypeChoice ::= CHOICE {

start							ODSIB1ConfigurationInformation,

stop							NR-CGI,

choice-extension				ProtocolIE-Single-Container { {RequestTypeChoice-ExtIEs} }

}

RequestTypeChoice-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

ODSIB1ConfigurationInformation ::= SEQUENCE {

oDSIB1Configuration					OCTET STRING,

nES-Cell-ID							NR-CGI,

cEll-A-ID							NR-CGI						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {ODSIB1ConfigurationInformation-ExtIEs} }	OPTIONAL,

...

}

ODSIB1ConfigurationInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION RESPONSE

--

-- **************************************************************

ODSIB1ConfigurationProvisionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionResponse-IEs}},

...

}

ODSIB1ConfigurationProvisionResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY ignore	TYPE NR-CGI										PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION FAILURE

--

-- **************************************************************

ODSIB1ConfigurationProvisionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ ODSIB1ConfigurationProvisionFailure-IEs}},

...

}

ODSIB1ConfigurationProvisionFailure-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-Cause									CRITICALITY ignore	TYPE Cause										PRESENCE mandatory}|

{ ID id-NES-Cell-ID								CRITICALITY ignore	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY ignore	TYPE NR-CGI										PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE

--

-- **************************************************************

ODSIB1ConfigurationProvisionStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionStatusUpdate-IEs}},

...

}

ODSIB1ConfigurationProvisionStatusUpdate-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NES-Cell-ID								CRITICALITY ignore	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY ignore	TYPE NR-CGI								PRESENCE optional }|

{ ID id-ProvisionStatus							CRITICALITY ignore	TYPE ProvisionStatus							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

ProvisionStatus ::= ENUMERATED {stopped, ...}

END

-- ASN1STOP

*************** Unchanged text omitted***************

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

*************** Unchanged text omitted***************

id-rachIndication												ProcedureCode ::= 50

id-dataCollectionReportingInitiation							ProcedureCode ::= 51

id-dataCollectionReporting										ProcedureCode ::= 52

id-ODSIB1ConfigurationProvision									ProcedureCode ::= 53

id-ODSIB1ConfigurationProvisionStatusUpdate						ProcedureCode ::= 54

id-lTMConfigurationUpdate										ProcedureCode ::= 55

id-cSIRSCoordination											ProcedureCode ::= 56