# 3GPP TSG-RAN WG3 Meeting #129	 R3-255149

# Bengaluru, India, 25th – 29th August 2025

# Title: 	(TP to 38.423, 38.473) Inter-CU LTM

# Source: 	ZTE Corporation

# Agenda item:	13.2

# Document for:	Discussion and Decision

- Introduction

In the RAN3 #128 meeting, after online and offline discussion, the issue is still FFS. The following is captured in the Chair note.

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

Add description in Stage 2 TS 38.401 for describing that CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE message.

For Inter-CU LTM, LTM CONFIGURATION UPDATE procedure is per node level basis with a list of cells, and security key is per cell.

Update the online agreement to: CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message.

Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell.

In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message.

FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.

FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message.

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

This paper further discusses this issue based on the above progress.

- Discussion

2.1 CSI-RS COORDINATION

Update the online agreement to: CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message.

FFS on whether the gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

When the candidate gNB-DU provides CSI-RS resource configuration, it may provide both periodic CSI-RS resources and SP CSI-RS resources. Which type the CSI-RS resources belong to should be indicated in both F1AP and XnAP, so the source gNB-DU can know whether it needs to send the SP CSI-RS activation signalling or not for the CSI-RS resources.

Proposal1: When a candidate gNB-DU/gNB provides CSI-RS resources, it indicates the resource type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

In current RRC spec, there is no distinction for the CSI-RS resource configuration used for L1 measurements or for CSI acquisition. We can understand that the CSI-RS resource used for L1 measurements and CSI acquisition are the same. Therefore there is no need to add a new IE for SP CSI-RS resource for CSI acquisition.

Proposal2: A new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP is not needed.

In current running CR, the Request for CSI-RS Resource Configuration IE and LTM Information Request IE are included in the Handover Request message in parallel, but the CSI-RS Resource Configuration IE is in the LTM Information Response IE, which is asymmetrical. So to make them be symmetry, it is proposed to remove the CSI-RS Resource Configuration IE out of the LTM Information Response IE.

Proposal 3: To make the request and response IE be symmetry, remove the CSI-RS Resource Configuration IE out from the LTM Information Response IE and include it in the HANDOVER REQUEST ACKNOWLEDGE message directly.

2.2 Early RACH configuration

According to RAN3#127 meeting, RAN3 had discussed that in order to avoid explicitly exposing the gNB internal architecture, a new ID of early RACH configuration resource or other new name can be used to request early RACH configuration.

However, for a UE configured with inter-CU LTM, if any of the configured candidate LTM cells belongs to other gNB, the source gNB is not aware of the RACH resource request ID(s) so that it is unable to provide the RACH resource request ID(s)  for the candidate LTM cell in the early RACH request.

Observation 1 : the source gNB-CU is unable to provide RACH resource request ID(s)  in the early RACH request since it is not aware of the RACH resource request ID(s)  of the candidate LTM cell(s) which belongs to other gNB-CU.

So during LTM preparation phase, the candidate gNB-CU shall provide the RACH resource request ID(s) of the accepted candidate LTM cell(s) to the source gNB-CU. Then the source gNB can further inform this mapping to other candidate gNB(s).

Proposal 4: during LTM preparation phase, the candidate gNB shall provide the RACH resource request ID(s) of the accepted candidate LTM cell to the source gNB.

Similarly,  for a UE configured with inter-CU LTM, if any of the configured candidate LTM cells belongs to other gNB, after candidate gNB-CU receive the Early Sync Information Request from source gNB, it cannot be aware of the gNB-DUs belongs to other gNB-CU. However, in current UE CONTEXT SETUP REQUEST message, LTM gNB-DU ID is a mandatory IE in Early Sync Information Request. So a simple way is to add a field description that RACH resource request ID(s) is used for inter-CU LTM.

Proposal 5: Add the IE field description for LTM gNB-DUs List in Early Sync Information Request in UE CONTEXT SETUP REQUEST/modification message.

2.3 TA Information Transfer

In the previous RAN3 meeting, we agreed to introduce a new non-UE associated class-2 TA information Transfer message to transfer the TA information from the candidate gNB-CU to the source gNB-CU. But left an open issue on whether to include the XnAP IDs in this new message.

As we know, during Rel-18 intra-CU LTM, non-UE associated signaling procedure is used to transfer the TA information, therefore it would be good to keep the consistency with intra-CU LTM, only to have non-UE associated signaling procedure to transfer the TA information, too.

Proposal 6: For the inter-CU TA information transfer, use non-UE associated signaling procedure to transfer the TA information, which means XnAP IDs is needed included in TA Information Transfer message.

During RAN2#129bis meeting, it is agreed that MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement. Therefore, Tag ID Pointer shall be included in the TA information transfer message just like existing F1AP DU-CU TA Information Transfer message.

Proposal 7: Tag ID Pointer is included in the XNAP TA information transfer message.

In addition, during Rel-18 intra-CU LTM, Source gNB-DU ID is included in the F1AP DU-CU TA Information Transfer message to help the CU identity the received TA Information should be forwarded to which gNB-DU. However, as analysis above, in order to avoid explicitly exposing the gNB internal architecture, a new ID of early RACH configuration resource is used to request early RACH configuration. Therefore, the candidate DU may not know the source DU id belongs to other gNB, it can only indicate the early RACH configuration resource ID in the inter-CU TA information transfer message.

.Proposal 8: Source early RACH configuration resource ID is included in the  XNAP TA information transfer message.

In addition, based on previous discussion, some companies propose to include the TA values in Cell Switch Notification message for subsequent LTM, while some companies think it is not needed:

- Include the TA values in Cell Switch Notification message - Do not include the TA values in Cell Switch Notification message

Considering the TA value is introduced in Cell Switch Notification message for subsequent LTM in F1AP, it is no harmful to adopt the same way for subsequent LTM in X2AP. Therefore it would be good to keep the consistency with intra-CU LTM. Since it is an optional IE, if the source gNB consider the TA may be invalid for subsequent LTM, it may not provide it in Cell Switch Notification message based on implementation.

Proposal 9: Include the TA values of candidate cell(s) in XNAP Cell Switch Notification message.

- Conclusion

Based on the above analysis, we provide the following proposals.

Proposal1: When a candidate gNB-DU/gNB provides CSI-RS resources, it indicates the resource type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

Proposal2: A new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP is not needed.

Proposal 3: To make the request and response IE be symmetry, remove the CSI-RS Resource Configuration IE out from the LTM Information Response IE and include it in the HANDOVER REQUEST ACKNOWLEDGE message directly.

Observation 1 : the source gNB-CU is unable to provide RACH resource request ID(s)  in the early RACH request since it is not aware of the RACH resource request ID(s)  of the candidate LTM cell(s) which belongs to other gNB-CU.

Proposal 4: during LTM preparation phase, the candidate gNB shall provide the RACH resource request ID(s) of the accepted candidate LTM cell to the source gNB.

Proposal 5: Add the IE field description for LTM gNB-DUs List in Early Sync Information Request in UE CONTEXT SETUP REQUEST/modification message.

Proposal 6: For the inter-CU TA information transfer, use non-UE associated signaling procedure to transfer the TA information, which means XnAP IDs is needed included in TA Information Transfer message.

Proposal 7: Tag ID Pointer is included in the XNAP TA information transfer message. The TP is provided.

.Proposal 8: Source early RACH configuration resource ID is included in the XNAP TA information transfer message. The TP is provided.

Proposal 9: Include the TA values of candidate cell(s) in XNAP Cell Switch Notification message.

- TP to 38.423 for inter-CU LTM (BLCR R3-253959)

=======================<Start of change>=================================

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

| LTM Handover Information Response | O |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >SSB Information | M |  | 9.2.3.x10 |  | – |  |
| >LTM Information Response | O |  | 9.2.3.xx2 |  | – |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [10]. | – |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [10]. This IE applies for SUL carrier. | – |  |
| >>Early RACH Resources Requester ID | O |  | 9.2.3.xx9 | Identifies the Early RACH resources ID of this Candidate Cell ID. |  |  |
| CSI-RS Resource Configuration | O |  | 9.2.3.xx5 |  |  |  |
| Early Sync Information Response | O |  | 9.2.3.xx4 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.x.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | <maxnoofTAList> | NR CGI 9.2.2.7 |  | EACH | ignore |
| >>Candidate Cell ID | M |  |  |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.1.x2		TA INFORMATION TRANSFER

This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information.

Editor’s note: Details on IEs need to be continued.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| TA Information List |  | 1 |  |  | YES | ignore |
| >TA Information Item IEs |  | <1… maxnoofTAList> |  |  |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [35]. | - |  |
| >>Early RACH Resources Requester ID | M |  | 9.2.1.xx9 |  | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | - |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.x.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| LTM Updates from Candidate Cell Information List | 0..1 |  |  |  | YES | ignore |
| >LTM Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>Early RACH Resources Requester ID | O |  | 9.2.3.xx9 | Identifies the Early RACH resources ID of this Candidate Cell ID. |  |  |
| >>LTM Information Response | O |  | 9.2.3.xx2 |  | - |  |
| >>Early Sync Information Response | O |  | 9.2.3.xx4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | - |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.3.xx2	LTM Information Response

This IE contains LTM related information to be provided in response to information provided in the LTM Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM Candidate Configuration | M |  | OCTET STRING | Contains ltm-CandidateConfig as defined in TS 38.331 [10]. |
| Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.3.xX5	CSI-RS Resource Configuration

This IE contains the CSI-RS resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI-RS Resource Configuration To AddMod List | O |  |  | Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8]. |
| >CSI-RS Resource Configuration To AddMod Item |  |  |  |  |
| >>CSI-RS-ResourceConfigId |  | INTEGER (0..maxNrofCSI-ResourceConfigurations-1) |  |  |
| >>CSI-RS-ResourceType |  | ENUMERATED { periodic, semi-persistent} |  |  |
| >>ltm-NZP-CSI-RS-ResourceSetLis |  |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceSetList-r19 as defined in TS 38.331 [8]. |
| CSI-RS Resource Configuration To Release List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| >CSI-RS Resource Configuration To Release Item |  |  |  |  |
| >>CSI-RS-ResourceConfigId |  | INTEGER (0..maxNrofCSI-ResourceConfigurations-1) |  |  |

======================<End of change>=================================

- TP to 38.473 for inter-CU LTM (BLCR R3-253895)

======================<Start of change>=================================

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Setup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| Request for CSI-RS Resource Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s) in case the candidate gNB-DU(s) belongs to the same gNB-CU. Otherwise, this IE contains the IDs of the RACH resource request ID(s) and candidate gNB ID. | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 | This IE is the IDs of the RACH resource request ID(s) in case LTM gNB ID IE is included. |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.x1	CSI-RS Resource Configuration

This IE contains the CSI-RS resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI-RS Resource Configuration To AddMod List | O |  |  | Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8]. |
| >CSI-RS Resource Configuration To AddMod Item |  |  |  |  |
| >>CSI-RS-ResourceConfigId |  | INTEGER (0..maxNrofCSI-ResourceConfigurations-1) |  |  |
| >>CSI-RS-ResourceType |  | ENUMERATED { periodic, semi-persistent} |  |  |
| >>ltm-NZP-CSI-RS-ResourceSetLis |  |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceSetList-r19 as defined in TS 38.331 [8]. |
| CSI-RS Resource Configuration To Release List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [8]. |
| >CSI-RS Resource Configuration To Release Item |  |  |  |  |
| >>CSI-RS-ResourceConfigId |  | INTEGER (0..maxNrofCSI-ResourceConfigurations-1) |  |  |

======================<End of change>=================================