3GPP TSG-RAN WG3 Meeting #129	R3-255155

Bengaluru, India, 25 – 29 August 2025

# Title: 	(TP to BL CR for TS 38.413) Session Release procedure

# Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

# Document for:	Discussion and Decision

- Introduction

This TP is based on the following observations and proposals.

Observation 1: In the case of Inventory only scenario and Inventory completed, the NG-RAN node can release the A-IoT session locally other than wait for AIoT CN to release.

Observation 2: In the case of Inventory with command scenario and Inventory completed, both the NG-RAN node and AIoT CN shall keep the A-IoT session due to the subsequent command procedure.

Proposal 1: In the case of Inventory with command scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session.

Proposal 2: Inventory Complete indication shall be sent via current Inventory Report message, other than a new procedure.

Proposal 3: The Reader Report List IE contained in the Inventory Report Transfer IE included in Inventory Report message shall be optional instead of mandatory.

- TP for TS38.413 (based on BLCR R3-253972)

===========================Start of the change====================================

8.xx.5	A-IoT Session Release Request

8.xx.5.1	General

The purpose of the A-IoT Session Release Request procedure is to enable the NG-RAN node to request the A-IoT CN node to release the A-IoT session. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

8.xx.5.2	Successful Operation

Figure 8.xx.5.2-1: A-IoT Session Release Request

The NG-RAN node initiates the procedure by sending an AIOT SESSION RELEASE REQUEST message to the A-IoT CN node.

Interactions with the A-IoT Session Release procedure:

Upon reception of the AIOT SESSION RELEASE REQEUST message, the A-IoT CN node should initiate the A-IoT Session Release procedure for the A-IoT session denoted by the AIOTF Identifier IE and the Correlation ID contained in the AIOT SESSION RELEASE REQUEST message.

8.xx.5.3	Abnormal Conditions

Void.

===========================Next of the change====================================

9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Global gNB ID | M |  | 9.3.1.6 |  |
| Reader Report List |  | 0..1 |  |  |
| >Reader Report Item |  | 1..< maxnoofReaders> |  |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| >>Device Report List |  | 1 |  |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  |
| >>>>RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  |
| Inventory Complete indication | O |  | ENUMERATED (true, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofDevices | Maximum no. of Devices within a reader. Value is 65535. |

===========================Next of the change====================================

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<Skip unchanged part>

InventoryReportTransfer ::= SEQUENCE {

correlationIdentifier		AIoT-CorrelationIdentifier,

globalgNB-ID				GlobalGNB-ID,

readerReportList			AIoT-ReaderReportList					OPTIONAL,

InventoryCompleteIndication ENUMERATED {true, ...} 					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { InventoryReportTransfer-ExtIEs} }		OPTIONAL,

...

}

InventoryReportTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- J

-- K

===========================End of the change====================================