3GPP TSG-RAN WG3 Meeting #129	R3-255273

Bengaluru, India, 25 – 29 August 2025

Agenda Item:		11.3

Source:		Ofinno

Title:	AI/ML enabled Coverage and Capacity Optimization

Document for:		Discussion and decision

# Introduction

In the RAN3 #128 meeting, Opton2 was agreed: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

It was discussed that the cause value in the Future Coverage Modification Notification may be reused for indicating the cause of neighbour node(s) future coverage modification in the Neighbour Future Coverage Modification Notification.

This may mean that the cause value in the Future Coverage Modification Notification corresponding to the Neighbour Future Coverage Modification Notification may be generated by gNB-CU (not just forwarded from a neighbour gNB).

If this is the case, this is a case of the gNB-CU providing some assistance information to the gNB-DU.

Another similar direction may be in the following open issue:

Timing information for predicted CCO issue over Xn is not needed?

There is Time for Neighbour Future Coverage Modification in the Neighbour Future Coverage Modification Notification, but the open issue questions the need to send this over Xn from a neighbour node. This may mean that the Time for Neighbour Future Coverage Modification my be generated by the gNB-CU.

Based on the current agreements and open issue, there are different types of assistance information sent from gNB-CU to gNB-DU.

In legacy systema, final decision on cells and beams coverage states is done by the gNB-DU. This contribution proposes additional assistance information, proposed in the previous meetings, to enhance gNB-DU capabilities to make CCO decisions.

# Discussion

## 2.1	Future CCO state as assistance information

A gNB-CU has an AI/ML model for predicting a coverage or capacity issue in cells and beams managed by a gNB-DU. We think that the AI/ML model may be enhanced to also generate proposed cells and/or beams coverage states for cells and beams of the gNB-DU. This may not require a large amount of additional inference data (if any) at the input of the AI/ML model, because the AI/ML model is already capable of predicting coverage and capacity issues.

Such approach may considerably simplify why simultaneously improve gNB-DU CCO operation. This may also allow some level of coordination of coverage state changes of neighbour gNB-DUs managed by the gNB-CU (and potentially some level of inter-CU coordination).

This may also help the gNB-DU to react fast (e.g., just select one of the proposed cells and/or beams coverage states) in case gNB-CU changes the prediction close to the predicted time of a CCO issue.

We believe that it may be beneficial if the gNB-CU provides to the gNB-DU proposed cells and/or beams coverage states for cells and beams of the gNB-DU.

Proposal 1: A gNB-CU sends to a gNB-DU, proposed coverage states for cells and beams of the gNB-DU.

## 2.2	Additional assistance information (e.g., additional UE performance measurement metrics)

We think that the gNB-CU may provide to gNB-DU some additional assistance information to help with reconfiguration of cell and/or beam coverage areas. For example, this may help the gNB-DU to understand why coverage and/or capacity issue is predicted by an AI/ML model in the gNB-CU.

For example, the gNB-CU may provide to gNB-DU prediction of number of UEs/traffic/load per cell/beam; prediction of L3 performance metrics, e.g., number of RLFs, HOFs, SHOs per cell/beam (when applicable).

Proposal 2: A gNB-CU sends to a gNB-DU, additional assistance information for CCO, for example, predicted number of UEs/traffic/load, current or predicted UE location information (e.g., geographic areas), node/cell/beam level UE performance, predicted L3 performance metrics (e.g., RLF, HOF, SHO).

# Proposals

Proposal 1: A gNB-CU sends to a gNB-DU, proposed coverage states for cells and beams of the gNB-DU.

Proposal 2: A gNB-CU sends to a gNB-DU, additional assistance information for CCO, for example, predicted number of UEs/traffic/load, current or predicted UE location information (e.g., geographic areas), node/cell/beam level UE performance, predicted L3 performance metrics (e.g., RLF, HOF, SHO).

# Reference

[1] R3-255001, RAN3#129 Meeting Agenda

[2] R3-255002, RAN3#128 Meeting Report

[3] R3-255046, (BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN

[4] R3-255044, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN