3GPP TSG-RAN WG3 Meeting #129	R3-255311

Bengaluru, India, Aug 25 - 29, 2025

Agenda Item:	15.2

Source: 		Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, ZTE, CATT, China Telecom, CMCC, LG Electronics, Xiaomi

Title: 	(TP for TS 36.300) Support for Regenerative Payload for IoT NTN

Document for:	Discussion/Decision

# Introduction

This contribution proposes changes to introduce an indication to suspend and resume S1AP, during Hard FLSO or S1 Connection suspend/resume, to align with NR NTN Regenerative Payload decision on the same.

# TP for TS 36.300

### 23.21.5	Switchover

#### 23.21.5.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure.

Both hard and soft feeder link switchover are applicable to NTN.

#### 23.21.5.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption may occur during the transition between the feeder links.

#### 23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover between two eNBs is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs' context between the two eNBs at feeder link switchover is performed by means of either S1 based or X2 based handover, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

For Regenerative payload, an indication to suspend or resume data and signaling procedures over S1 interface may be sent to MME from eNB in the ENB Configuration Update message when the Hard Feeder Link switch occurs, or for S1 Connection suspend/resume.