3GPP TSG-RAN3 Meeting #129                                                                    R3-255323

# Bangalore, India, Aug 25 – 29, 2025                                        Revision of R3-253564

Agenda Item:	21.3

Source: 	Qualcomm Inc.

Title: 	R19 XR Signaling Enhancements

Document for:	Discussion/Agreement

# Introduction

Following are R19 revised XR WID [1] objectives with RAN3 impacts.

-  Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-  Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-  Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

-  QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-  Support of exposure of available data rate [RAN3]

Note: Coordination with SA2, as needed

RAN3#127bis Agreements:

For support of DL PDU Set marking without PDU Set QoS:

From SMF to gNB direction: introduce DL PDU Set Information Marking Support Indication IE in the QoS Flow Level QoS Parameters IE. This indication is also provided to target gNB over XnAP.

From gNB to SMF direction: reuse existing PDU Set based Handling Indicator IE.

For support alternative PDU Set QoS:

Add UL PSDB, DL PSDB, UL PSER and DL PSER in Alternative QoS Parameter Set List IE in NGAP/XnAP/F1AP/E1AP.

Add new codepoints “not fulfilled DL” and “not fulfilled UL” in the Notification information/cause IE in NGAP/XnAP/F1AP.

For QoS Handling when Traffic Characteristics Change Dynamically:

Introduce BSSize and TTNB in NG-U/F1-U/XN-U.

For Support of exposure of available data rate:

Introduce thresholds and direction to trigger NG-RAN monitor available data rate in NGAP/XnAP/F1AP/E1AP.

Introduce “available data rate” in TS 38.425 and TS 38.415.

For Support and specify multi-modality awareness for QoS flows in both DL and UL RAN:

Introduce MMSID in NGAP, XnAP, F1AP and E1AP.

For Specify uplink congestion signalling:

Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

Introduce the same indication in XnAP to inform target gNB.

Continue the discussion on UL rate control.

FFS on specific container/frame to be used for Traffic Characteristics Change Dynamically. FFS on impact to TS 38.420/470.

FFS on non-homogeneous deployment for support of exposure of available data rate.

FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

FFS on whether the gNB-CU sends an indication to the gNB-DU to stup the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

Work on stage2 and stage3 details and open issues above…

RAN3#128 Agreements:

CU sends the uplink rate control indication per QoS flow over F1 to DU.

Other additional assistance information from CU to DU?

- Recommended UL bit rate infor per QoS flow

- Measured bit rate per QoS flow

- No additional information

No NGAP signalling enhancement is needed for the support of non-homogeneous deployment?

Introduce a F1AP IE, similar to NGAP Indication of Bitrate Adaptation IE.

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB.

For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

In this contribution, we propose several signalling enhancements for above XR objectives by taking RAN3#128 agreements into account.

# UL Rate Control discussion

RAN3#127bis :

FFS on whether CN may inform gNB the recommended UL rate for specific QoS flows.

In order for gNB to provide any UL recommended bit rate control information to UE via MAC-CE, whether it is for option 1 or option 2 based solution, first RAN should be aware what is XR OTT application data rate that can be used for specific XR application. Since all these XR applications are OTT based and without knowing possible XR application codec rates to be used, RAN will not be able to determine what UL recommended data rate values can be sent to UE for UL XR application rate adaptation purpose. During PDU session resource setup procedure, 5GC has to provide for a given specific QoS flow what are applicable data rates that can used. 5GC has to receive this information from XR application server.

Thus we propose:

For RAN based UL Rate Control feature, RAN3 agrees that 5GC/SMF will provide possible UL recommended data rate values to be used for specific QoS low level.

# Enhancements to support unnecessary RLC Re-transmissions avoidance and timely RLC Re-transmissions.

In RAN3#127bis, Lenovo paper R3-251955 brough the following discussion and is valid point to discuss. It was discussed in RAN3#128 made some progress. But still there are open issues noted.

RAN2#129 Agreements on RLC enhancements Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

RAN2#129bis Agreements on RLC enahncements

- When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review)

- For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

- Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers.

- Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

RAN2#130 Agreements on RLC timely retransmissions:

- (RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this

- (RLC-9) No additional conditions are needed for the polling enhancement.

- (RLC-7) RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT.

- (PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed.

- (RRC-3, RRC-4) Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance

- (RRC-8) The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

RAN2#130 Agreements for unnecessary retransmission avoidance in RLC :

- (RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission.

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

- Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

- (UE capability-2) Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

- (UE capability-2) Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

.RAN3#127bis:

FFS on whether the gNB-CU sends an indication to the gNB-DU to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

RAN3#128 Agreements:

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB.

For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

In order to support autonomous RLC based Re-transmissions from Tx RLC entity and Tx RLC enhanced polling request,  RAN2 concluded that both autonomous RLC Re-transmissions and enhanced poling triggers are based on remaining time of PDCP discard timer.

- Observation 1: Autonomous Tx side RLC Re-transmissions and enhanced polling request are based on remaining time of PDCP discard timer value.

Autonomous RLC Re-Tx and enhanced polling requests are based on remaining thresholds of PDCP discard timer per PDCP SDU. For UL RLC based autonomous Re-Tx and enhanced polling, UE PDCP will provide the triggers for RLC when will Tx RLC entity has to perform autonomous RLC Re-Tx and enhanced polling request. gNB will provide the PDCP configuration including remainingTimeBasedPollingThreshold for the PDCP SDU and  remainingTimeBasedReTxThreshold for the PDCP SDU. In CU-DU split scenario, CU is responsible for generating PDCP configuration.

RAN2 PDCP Running CR text is given below.

## 5.x	Triggering of RLC functions

For the purpose of remaining-time-based RLC retransmission, if the remaining time till discardTimer expiry becomes less than the remainingTimeBasedReTxThreshold for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers but for which successful delivery has not been confirmed by lower layers, the transmitting PDCP entity shall:

-	indicate to lower layers that the condition for remaining-time-based RLC retransmission is met for the corresponding PDCP Data PDU.

For the purpose of remaining-time-based RLC polling, if the remaining time till discardTimer expiry becomes less than the remainingTimeBasedPollingThreshold for the PDCP SDU for which the corresponding PDCP Data PDU has already been submitted to lower layers but for which successful delivery has not been confirmed by lower layers, the transmitting PDCP entity shall:

-	indicate to lower layers that the condition for remaining-time-based RLC polling is met for the corresponding PDCP Data PDU.

- Based on above discussion, we have following proposals

For UL autonomous RLC Re-Tx and enhanced polling request , CU generates the new PDCP configuration parameters remainingTimeBasedPollingThreshold and remainingTimeBasedReTxThreshold for the PDCP SDU. CU will send PDCP new configuration parameters to CU-UP via E1-AP Signaling messages.

For autonomous Re-Transmission based on remaining time of PDCP discard timer, discuss and agree one of the following 2 options. But Option 2 is better than Option 1.

- Option 1: For DL, PDCP can request RLC SDU Re-Tx if RLC SDU successful delivered confirmation is not received by PDCP before the PDCP discard timer expires.

- Option 2:  PDCP discard timer is provided from PDCP to RLC as part of the DL User Plane Data for each RLC SDU and then Tx RLC knows when to trigger RLC Re-Tx autonomously.

For enhanced Polling request based on remaining time of PDCP discard timer, either DL Tx PDCP indication or Tx RLC can trigger Polling Request automatically based on PDCP discard remaining time value.

# Summary

Based on above discussion, we have the following proposals for XR enhancements.

Proposal 1.	For RAN based UL Rate Control feature, RAN3 agrees that 5GC/SMF will provide possible UL recommended data rate values to be used for specific QoS low level.

Observation 1: Autonomous Tx side RLC Re-transmissions and enhanced polling request are based on remaining time of PDCP discard timer value.

Proposal 2.	For UL autonomous RLC Re-Tx and enhanced polling request , CU generates the new PDCP configuration parameters remainingTimeBasedPollingThreshold and remainingTimeBasedReTxThreshold for the PDCP SDU. CU will send PDCP new configuration parameters to CU-UP via E1-AP Signaling messages.

Proposal 3.	For autonomous Re-Transmission based on remaining time of PDCP discard timer, discuss and agree one of the following 2 options. But Option 2 is better than Option 1.

Option 1: For DL, PDCP can request RLC SDU Re-Tx if RLC SDU successful delivered confirmation is not received by PDCP before the PDCP discard timer expires.

Option 2:  PDCP discard timer is provided from PDCP to RLC as part of the DL User Plane Data for each RLC SDU and then Tx RLC knows when to trigger RLC Re-Tx autonomously.

Proposal 4.	For enhanced Polling request based on remaining time of PDCP discard timer, either DL Tx PDCP indication or Tx RLC can trigger Polling Request automatically based on PDCP discard remaining time value.

# References

[1] RP-250107: Revised WID on XR (eXtended Reality) for NR Phase 3

[2] TS 23.501: System architecture for the 5G System (5GS); Stage 2

[3] TR 23.700-070: Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2