3GPP TSG-RAN3 Meeting #129	R3-255336

Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

Agenda item:	20.2

Source:	Xiaomi

Title:	(TPs for TS 38.305 and TS 38.455) Support of gNB-side model (case 3a)

Document for:	Other

1	Introduction

In this contribution, we’d like to further discuss the leftover issues for case 3a according to the RAN3 progress.

From RAN3#128: Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator. FFS if UE location can be requested. The Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message. Introduce POSITIONING DATA COLLECTION REPORT message as class 2 procedure including Positioning Data Information IE and RAN/LMF Measurement IDs Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason. RAN3 confirms that Performance monitoring is done in the RAN in case 3a. RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface. Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion FFS: if LMF should specifically request for AIML inferred measurement.

The following issues are discussed

Issue #1 whether to indicate requested TRPs for the data collection request

Issue #2 Encoding of Positioning Data Unavailable IE.

Issue #3 Proactive solution for data collection

Issue #4 whether UE location can be requested as Part B information

Issue #5 Whether Los/NLoS can be inferred and reported by gNB

Issue #6 Whether the LMF can request for AI/ML measurements

2	Discussion

## Issue #1 whether to indicate requested TRPs for the data collection request

In the last meeting, the Positioning Data Collection Needed IE was added as a top-level IE in the NRPPa MEASUREMENT RESPONSE and MEASUREMENT REPORT messages. This IE indicates that the data collection request applies to all TRPs involved in the positioning measurement.

From our perspective, TRP data collection should be supported, as it serves two key purposes: model training and performance monitoring.

- For model training, even TRPs not selected for positioning can benefit through opportunistic positioning.

- For performance monitoring, it is typically sufficient to monitor only a subset of TRPs involved in positioning.

Therefore, the TRPs requiring data collection may not correspond to all currently used TRPs and may involve only a subset.

Observation 1, TRPs requiring data collection may not be limited to those currently used and could involve only a subset.

Proposal 1, Introduce a requested TRP list within the Positioning Data Collection Needed IE to support data collection for particular TRP(s).

## Issue #2 Encoding of Positioning Data Unavailable IE.

In the last meeting, it was agreed that this IE would indicate when the LMF is unable to provide any positioning information in the DATA COLLECTION REPORT message. Currently, the IE is encoded as ENUMERATED(true, ...).

However, we believe that providing more detailed information would be beneficial for the gNB to make informed decisions about whether to continue requesting data for the given measurement. There are typically two possible reasons for the unavailability:

- The requested data is temporarily unavailable, for example, due to a failure in the positioning method.

- The data is permanently unavailable for the UE, such as when user consent has not been granted.

Therefore, we propose enhancing the IE by introducing specific codepoints that indicate the reason for data unavailability from the LMF side, rather than using a generic "true" value. Suggested codepoints include: no data available and user consent not granted.

Proposal 2, Re-encode the Positioning Data Unavailable IE as ENUMERATED (no data available, user consent not granted, ...) to provide clearer context for data unavailability.

## Issue #3 Proactive solution for data collection

In previous meetings, RAN3 has discussed proactive data collection and opportunistic data collection, and agreed the solution for opportunistic data collection. The opportunistic solution may have data collection limitations, as the gNB is unaware whether the requested UE is PRU or has user consent. Currently, there’re two proactive solutions discussed.

- Proactive solution 1 (NRPPa based)

- Proactive solution 2 (NGAP based)

For NGAP-based solution, according to SA2 reply LS below, it is not feasible in current Rel-19, so RAN3 can conclude that NGAP solution is not pursued in Rel-19.

SA2 Answer: For solution 1 it may be desirable to document related LMF procedures in TS 23.273. There are SA2 impacts to support the solution but SA2 could address those impacts as alignment with RAN in Rel-19. For solution 2, SA2 concludes that this solution is not feasible in current frozen Rel-19. SA2 is currently discussing for data collection for training models to be used by the LMF for positioning (RAN case 3b) whether a user consent check and/or a LCS privacy profile check is required. SA2 is anticipating that similar checks could also be required for case 3a (both for solution 1 and solution 2).

Proposal 3, NGAP solution is not pursued in Rel-19.

RAN3 need to discuss whether to support NRPPa-based proactive data collection, the following is one candidate solution.

As the NRPPa transaction is initiated by the LMF, if the LMF supports data collection, it can initiate a data collection requirement procedure to gather the data collection needs from the gNBs within a serving area. The gNB can indicate their needs in the response message or update their needs in the data collection requirement update procedure. The subsequent call flow illustrates how this process functions.

- The LMF sends a NRPPa Data Collection Requirement Request message to a gNB to indicate the data collection is available in LMF.

- The gNB response a NRPPa Data Collection Requirement Response message to the LMF. If data collection is required in the gNB, it includes the data collection requirement information in the NRPPa Data Collection Requirement Response message.

NOTE 1: the LMF may initiates multiple Data Collection Requirement Request Procedure to multiple gNBs.

- The LMF perform UE selection as described in TS 23.273 (e.g. step 1-5 in section 6.22.3) considering to the data collection requirement information received from the gNB.

NOTE 2, The selected UE can be either PRU or UE with user consent.

NOTE 3, The selected UE may be or may be not served by the gNB.

- The LMF may initiate UE positioning procedures for the selected UE as specified in TS 23.273 section 6.11 to obtain the UE location (i.e. the ground truth label).

- The LMF initiates UL positioning procedures for the selected UE to configure UL SRS as specified in TS 23.273 section 6.11.2 and 6.11.3.

- The LMF sends a NRPPa Positioning Data Collection Report message to the gNB. LMF includes the ground truth label(s) along with the measurement ID(s) and timestamps in the NRPPa Data Collection Information message. The gNB correlates the UL SRS measurements obtained in step 5 with the ground truth label obtained in step 6, and use them as data input for AI model training and/or performance monitoring.

- If the data collection requirement in the gNB is updated (e.g. gNB decide to stop data collection), the gNB sends a Data Collection Requirement Update message to the LMF.

Proposal 4, if proactive NRPPa solution is supported, introduce one class 1 and one class 2 procedure for proactive data collection requirement exchange.

## Issue #4 whether UE location can be requested as Part B information

RAN3 has agreed that The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator. FFS if UE location can be requested.

From our perspective, the collected data serves two main purposes: model training and performance monitoring. The collected data represents the output of the AI model. In the case of scenario 3a, the output consists of measurement information, making it unnecessary to collect UE location. Moreover, UE location is considered private information and should not be known by RAN.

Proposal 5, UE location information should not be requested by gNB for case 3a.

## Issue #5 Whether Los/NLoS can be inferred and reported by gNB

We believe the inferred LOS/NLOS is needed, LMF can use this information to decide whether use the timing measurement from the corresponding TRP (as multiple measurements from multiple TRPs are received by LMF), e.g., if it’s NLOS or close to NLOS, it indicates the confidence level is low for this TRP, LMF may not use the measurements from this TRP to calculate the location.

Proposal 6, LOS/NLOS can be inferred and reported by gNB.

## Issue #6 Whether the LMF can request for AI/ML measurements

In our views,inferred measurements can be understood as a type of positioning method or measurement method. Compared to traditional positioning techniques, the inferred measurements offer higher positioning accuracy. The LMF is responsible for selecting the appropriate positioning method/measurement based on the LCS QoS requirements, making it reasonable for the LMF to determine whether inferred measurements are needed.

For the encoding, the Measurement Characteristics Request Indicator IE can be utilized. Specifically, two bits should be allocated to represent the LMF’s request:

- One bit to indicate a request for inferred timing information

- One bit to indicate a request for inferred LOS/NLOS information.

Proposal 7: The LMF should be able to request for AI/ML measurements according to the positioning requirements.

Proposal 8: Introduce two bits "Inferred timing measurement" and “inferred LOS/NLOS information” in the Measurement Characteristics Request Indicator IE.

# 3	Conclusion

In this contribution, we discussed the leftover issues for case 3a and have the following observations and proposals.

Issue #1 whether to indicate requested TRPs for the data collection request

Observation 1, TRPs requiring data collection may not be limited to those currently used and could involve only a subset.

Proposal 1, Introduce a requested TRP list within the Positioning Data Collection Needed IE to support data collection for particular TRP(s).

Issue #2 Encoding of Positioning Data Unavailable IE

Proposal 2, Re-encode the Positioning Data Unavailable IE as ENUMERATED (no data available, user consent not granted, ...) to provide clearer context for data unavailability.

Issue #3 Proactive solution for data collection

Proposal 3, NGAP solution is not pursued in Rel-19.

Proposal 4, if proactive NRPPa solution is supported, introduce one class 1 and one class 2 procedure for proactive data collection requirement exchange.

Issue #4 whether UE location can be requested as Part B information

Proposal 5, UE location information should not be requested by gNB for case 3a.

Issue #5 Whether Los/NLoS can be inferred and reported by gNB

Proposal 6, LOS/NLOS can be inferred and reported by gNB.

Issue #6 Whether the LMF can request for AI/ML measurements

Proposal 7: The LMF should be able to request for AI/ML measurements according to the positioning requirements.

Proposal 8: Introduce two bits "Inferred timing measurement" and “inferred LOS/NLOS information” in the Measurement Characteristics Request Indicator IE.

# 4	Annex A (TP for TS 38.305) proactive solution

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7.x.3	Proactive Positioning Data Collection Procedure

Figure 7.x.3-1 shows the Proactive Positioning Data Collection procedure used to retrieve positioning information from LMF.

Figure 7.x.3-1: Proactive Positioning Data Collection Procedure

- The LMF sends a NRPPa Data Collection Requirement Request message to a gNB to indicate the data collection is available in LMF.

- The gNB response a NRPPa Data Collection Requirement Response message to the LMF. If data collection is required in the gNB, it includes the data collection requirement information in the NRPPa Data Collection Requirement Response message.

NOTE 1: the LMF may initiates multiple Data Collection Requirement Request Procedure to multiple gNBs.

- The LMF perform UE selection as described in TS 23.273 considering to the data collection requirement information received from the gNB.

NOTE 2, The selected UE can be either PRU or UE with user consent.

NOTE 3, The selected UE may be or may be not served by the gNB.

- The LMF may initiate UE positioning procedures for the selected UE as specified in TS 23.273 section 6.11 to obtain the UE location (i.e. the ground truth label).

- The LMF initiates UL positioning procedures for the selected UE to configure UL SRS as specified in TS 23.273 section 6.11.2 and 6.11.3.

- The LMF sends a NRPPa Positioning Data Collection Report message to the gNB. LMF includes the information related to UE location along with the measurement ID(s) and timestamps in the NRPPa Data Collection Information message. The gNB correlates the UL SRS measurements obtained in step 5 with the information related to UE location obtained in step 6, and use them as data input for AI model training and/or performance monitoring.

- If the data collection requirement in the gNB is updated (e.g. gNB decide to stop data collection), the gNB sends a Data Collection Requirement Update message to the LMF.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# 5	Annex B (TP for TS 38.455) support of gNB-side model for AI-assisted positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >>Sample-based UL-RTOA |  |  | 9.2.X |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >Inferred measurement | O |  | ENUMERATED(true, …) | This IE is only valid for UL-RTOA, gNB Rx-Tx Time Difference and LoS/NLoS Information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: UL SRS-RSRPP in Additional Path Fourth Bit: Multiple UL AoA in Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Eleventh bit: SRS bandwidth aggregation used for joint UL positioning measurement. Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteen bit: UL SRS-RSRPP in sample-based UL-RTOA. Fourteen bit: Inferred timing measurement Fifteen bit: Inferred LOS/NLOS Information Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.Z1	Positioning Data Collection Needed

This information element contains the requested positioning information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested Positioning Data Collection | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents a requested positioning information: First bit: UL-RTOA Second bit: gNB Rx-Tx Time Difference Third bit: LoS/NLoS information Value ‘1’ indicates ‘needed’, Value ‘0’ indicates ‘not needed’. Other bits reserved for future use. |
| Requested TRP List |  | 0..1 |  | Indicates the area of interests |
| >Requested TRP Item |  | 1..<maxnoofMeasTRPs> |  |  |
| >>TRP ID | M |  | 9.2.24 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>