3GPP TSG-RAN WG3 Meeting #129	R3-255339

Bengaluru, India, 25th - 29th Aug, 2025

Title: 	Signalling for 7 MHz Channel Bandwidth

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# 1	Introduction

In RAN#106, a work item to introduce a new 7MHz channel bandwidth for band n5 and n26 was approved in [1], with the RAN3 involved objective of the WI list as following:

- Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective.

In RAN3#127bis meeting and RAN3#129 meeting, there were two LSs [2][3] from RAN4, and RAN3 in CC about the introduction of 7MHz channel bandwidth:

Excerpt from [2]: To RAN2 ACTION: RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15.

Excerpt from [3]: 1. Overall Description: RAN4 discussed the contents of LS R2-2503201/ R4-2505319, and would like to provide RAN2 further clarification on the signalling requirements for 7 MHz as the minimum or maximum channel bandwidth. The RAN2 Reply LS states “RAN2 respectfully asks whether signalling to support 7 MHz as the minimum or maximum channel bandwidth should be introduced.” It is RAN4’s view that 7 MHz is will never be the minimum channel bandwidth for a given NR band. As for the maximum channel bandwidth for a band, currently 7 MHz has only been requested for NR bands that also support 5 and 10 MHz, but it is possible that at some point in the future 7 MHz could be the maximum channel bandwidth for a given NR band. For carrier aggregation, 7 MHz could be the maximum or the minimum channel bandwidth that a UE supports for a band in a band combination. In summary, signalling to indicate both minimum and maximum channel bandwidth for a band in a band combination is needed for 7 MHz to reflect these possibilities. Also, in response to some offline requests for feedback on which release this signalling should apply to, RAN4 would like to clarify that we would like 7 MHz to be release independent from Rel-15 (up to RAN2 as to which version of the spec it appears with early implementation allowed), but it is acceptable if the signalling for 7 MHz as a minimum or maximum channel bandwidth for a band in a band combination is introduced in Rel-18 or Rel-19. 2. Actions: To RAN4 ACTION: RAN4 respectfully asks that RAN 2 take the following into consideration: It is RAN4’s view that 7 MHz will never be the minimum channel bandwidth for a given NR band, but it could be the maximum channel bandwidth for a given NR band, and for carrier aggregation 7 MHz could be the maximum or minimum channel bandwidth that a UE supports for a band in a band combination. In summary, signaling to indicate both minimum and maximum channel bandwidth for a band in a band combination is needed for 7 MHz to reflect these possibilities. Also, in response to some offline requests for feedback on which release this signalling should apply to, RAN4 would like to clarify that we would like 7 MHz to be release independent from Rel-15 (up to RAN2 as to which version of the spec it appears in with early implementation allowed), but it is acceptable if the signalling for 7 MHz as a minimum or maximum channel bandwidth for a band in a band combination is introduced in Rel-18 or Rel-19.

In this contribution, we will further analyse the RAN3 impacts to support 7 MHz channel bandwidth.

# 2	Discussion

In order to introduce 7MHz channel bandwidth, RAN4 has approved the Running Rel-19 CR to TS 38.104 [4], in which the transmission bandwidth configuration of 7MHz is defined to 35 RB for 15KHz SCS:

From RAN3’s perspective, the transmission bandwidth of the cell needs to be exchanged over XnAP and F1AP specifications as per cell information. Thus, the value of 35 RB should be introduced to the NR Transmission Bandwidth IE in XnAP and the Transmission Bandwidth IE in F1AP, to indicate the 7MHz transmission bandwidth of BS as specified in TS 38.104.

- Introduce a new value of 35 RB to the NR Transmission Bandwidth IE in XnAP and the Transmission Bandwidth IE in F1AP, according to the newly approved value in TS 38.104.

Based on the LS in [3], the 7MHz channel bandwidth could be release independent from Rel-15. We understand that this early implementation is from UE perspective. While from the network perspective, the 35RB of NR transmission bandwidth can be supported from R19.

- The introduction of 35 PRB transmission bandwidth over network interfaces starts from R19.

Besides, RAN4 also mentioned that the 7MHz can be the maximum or minimum channel bandwidth that a UE supports for a band in a band combination for carrier aggregation, and required RAN2 to design the signaling to support this. For this aspect, we believe there is no RAN3 impact since the details about UE capability of band combination for carrier aggregation is already signalled as RRC container over the network interfaces.

- Observation 1: There is no RAN3 impact to support 7 MHz as a minimum or maximum channel bandwidth for a band in a band combination.

As observed from the WID, RAN3 is requested to introduce the necessary signalling for this channel bandwidth once RAN4 has introduced the new channel bandwidth. With the above proposals, RAN3 could inform the agreements made on the 7MHz channel bandwidth. Also, the LS can add RAN plenary in CC to notify that the WID needs to be updated to add F1AP specification as the impacted TS.

- RAN3 to send reply LS to RAN4 to inform the introduced new value of transmission bandwidth over Xn interface and F1 interface, with RAN plenary in the CC.

The CRs to TS 38.423 and TS 38.473 are given in [5][6], and the draft of reply LS is given in [7].

# 3	Conclusion

Based on the above discussion we have the following proposals:

- Introduce a new value of 35 RB to the NR Transmission Bandwidth IE in XnAP and the Transmission Bandwidth IE in F1AP, according to the newly approved value in TS 38.104.

- The introduction of 35 PRB transmission bandwidth over network interfaces starts from R19.

- Observation 1: There is no RAN3 impact to support 7 MHz as a minimum or maximum channel bandwidth for a band in a band combination.

- RAN3 to send reply LS to RAN4 to inform the introduced new value of transmission bandwidth over Xn interface and F1 interface, with RAN plenary in the CC.

And the CRs to TS 38.423 and TS 38.473 are given in [5][6], and the draft of reply LS is given in [7].

# 4	Reference

- RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA

- R4-2503017, LS on Signalling for 7 MHz Channel Bandwidth, Rel-19, RAN4 (T-Mobile).

- R4-2508088, Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth, Rel-19, RAN4 (T-Mobile).

- R4-2506151, Running CR to TS 38.104 - Introduction of 7 MHz channel BW, Ericsson

- R3-255340	Introduction of 7 MHz Channel Bandwidth	Huawei

- R3-255341	Introduction of 7 MHz Channel Bandwidth	Huawei

- R3-255342	[draft] Reply LS on Signalling for 7 MHz Channel Bandwidth	Huawei

- R3-255390	Correction of system information management function	Huawei