3GPP TSG-RAN WG3 Meeting #129	R3-255354

Bengaluru, India, 25th – 29th August, 2025

Title: 	(TP for AI/ML BLCR to TS 38.300) Capturing agreements in Stage 2 RAN specifications

Source: 	Huawei, Jio Platforms, Deutsche Telekom, Ofinno, FiberCop, 	Qualcomm, InterDigital, Orange

Agenda item:	11.2

Document Type:	Discussion and decision

# 1. Introduction

RAN3 is discussing the objectives of the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

In the last meeting RAN3 agreed a Stage 2 TP for TS 38.300 in [6], which became the BLCR for that specification, covering the agreements related to the AI/ML-based Network Slicing. This contribution provides a TP for the AI/ML BLCR to TS 38.300 in [7] capturing additional RAN3 agreements which are relevant for Stage 2 description of Rel-19 AI/ML for NG-RAN.

# 2. Discussion

## 2.1 RAN3 agreements relevant for the AI/ML BLCR to TS 38.300

***** RAN3#125bis ***** [4] Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message. Introducing separate time information for future coverage state and predicted CCO issue. ***** RAN3#126 ***** [5] Introducing new IE for Future Coverage Modification List over Xn.

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 38.300 in Annex 1.

# 3. Conclusion

We make the following proposal:

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 38.300 in Annex 1.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- R3-253915, (TP for BLCR for TS38.300) Support of enhancements on AI/ML for NG-RAN, NEC, ZTE, Qualcomm, Samsung, CMCC, Ericsson, CATT, Ofinno, Huawei, Nokia

- R3-255048, (BL CR to 38.300) Support of enhancements on AI/ML for NG-RAN, NEC, ZTE, Qualcomm, Samsung, CMCC, Ericsson, CATT, Ofinno, Huawei, Nokia, Jio Platforms

# 5. Annex 1 – TP for AI/ML BLCR to TS 38.300 (based on R3-255048)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 16.20	Support of AI/ML for NG-RAN

### 16.20.1	General

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization, Network Slicing, CCO.

### 16.20.2	Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [64].

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [64].

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information, including predicted radio resource status per slice;

-	UE performance feedback, including per slice UE performance;

-	Measured UE trajectory;

-	Energy Cost (EC);

-	Predicted slice available capacity.

The collection and reporting of the above information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Predicted coverage modification cause, future coverage modification, and the time for future coverage state are transferred to the neighbour NG-RAN node via the NG-RAN Configuration Update procedure to provide information for, e.g., resolution of CCO issues predicted at the sending NG-RAN node.

<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>