3GPP TSG-RAN WG3 Meeting #129	R3-255361

Bengaluru, India, 25th – 29th August, 2025

Title: 	Discussion on feedback information for AI/ML-based Coverage and Capacity Optimization

Source: 	Huawei, Jio Platforms, Deutsche Telekom, Orange, Vodafone

Agenda item:	11.3

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based CCO (Coverage and Capacity Optimization).

This contribution analyzes further the aspect related to UE performance feedback for AI/ML-based CCO, and provides proposals to consider additional metrics, that is, UE-measured radio level metrics, on top of the legacy service-level metrics already agreed in RAN3#127-bis meeting [4].

# 2. Discussion

In the last meeting RAN3 did not discuss further the topic of feedback information for the AI/ML-based CCO use case, hence the latest status for such discussion dates back to RAN3#127-bis [4]:

Legacy UE performance measurement metrics can be reused for CCO. Whether/what additional UE performance measurement metrics is needed?

We further discuss here the FFS aspects above from the previous RAN3 meeting.

In RAN3#127, contribution [5] proposed to consider UE-measured radio-level metrics as part of feedback information. The reason is that, regardless of whether the predicted issue type is “coverage” or “cell edge capacity”, the action that is needed to cope with it translates into a change in coverage, and when a cell/SSB shrinks and another cell/SSB expands their respective coverages, UEs in such cells/SSBs will experience a change in the signal strength and/or quality of the radio channel. Whether the CCO changes at radio level are beneficial or not in terms of coverage for the UE may not be reflected by legacy UE performance metrics as specified in Rel-18 because, even though such metrics for UEs in cells/SSBs affected by the CCO issue differ, UE performance metrics may be influenced by other factors, e.g., user behavior or RRM policy of the UE.

By focusing on UEs that are handed over between cells affected by a CCO state change, a neighbor gNB (i.e., gNB(-CU)2) may be requested to collect and report to the local gNB hosting the AI/ML model which predicted the CCO issue (i.e., gNB(-CU)1) over Xn the radio-level measurements (RSRP, RSRQ, SINR) provided by those UEs which have been handed over from gNB(-CU)1 to gNB(-CU)2. In this way, gNB(-CU)1 may use such measurements to assess whether the predicted CCO issue was prevented by means of the coverage state changes previously performed by involved (split) gNBs for their corresponding cells/SSBs predicted to be affected.

We think that UE-measured radio-level metrics may be useful for the proper validation of the predicted CCO issue, so we propose to introduce them by introducing a new IE UE Measured Radio Performance in XnAP.

Proposal 1: RAN3 to agree to introduce a new IE UE Measured Radio Performance in XnAP in order to also exchange UE-measured radio-level metrics (RSRP, RSRQ, SINR) corresponding to those UEs handed over between cells affected by a coverage state change.

The existing Data Collection Report Initiation procedure can be extended to request the reporting of UE-measured radio-level metrics collected by the neighbor gNB (gNB(-CU)2) as well as for partial reporting aspects.

Proposal 2: RAN3 to agree to enhance the XnAP Data Collection Reporting (Initiation) procedures to enable the transfer over Xn of UE-measured radio-level metrics as collected by the neighbor gNB.

Moreover, we think it would be beneficial for AI/ML model’s monitoring purposes that the local node could receive from the neighbour node information related to its current coverage state just after the Time for Predicted CCO Issue, that is, after the previously predicted CCO issue was expected to occur and the corresponding future coverage state was actually applied by the local node at the Time for Future Coverage Modification to avoid such predicted CCO issue. This would allow the model in the local node (gNB(-CU)1) to have a clearer overall view of the AI/ML-based CCO mechanism and to fine-tune the model accordingly. Such information could be made available at the local node side without additional specification impact, as it is already possible to transfer the neighbour node’s current coverage state by means of the NG-RAN Node Configuration Update procedure.

Proposal 3: RAN3 to agree to transfer the current coverage state of the neighbour gNB to the local node over the Xn interface as a feedback information. No specification impact if foreseen.

Even if RAN3 already agreed in RAN3#127-bis meeting that legacy UE performance metrics can be used as feedback information for the AI/ML-based CCO use case, there is still an open issue from RAN3#125bis meeting as follows [6]:

Information to validate the prediction is needed.

The above open issue is related to the discussion in RAN3 concerning what is intended to be validated by means of the feedback information. As can be seen from the text captured as open point, such text is very generic as it refers to “prediction” without clarifying whether it is the “CCO prediction”, the “future CCO state” (if derived via AI/ML in gNB-DU, which is possible only by implementation) or a combination of the two.

In our view, we should base the discussion according to the definition of “feedback” as captured in TR 37.817 [7]:

## 4.2 Functional Framework Figure 4.2-1. Functional Framework for RAN Intelligence This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1. For the functions and data/information flows shown in the Figure 4.2-1, whether there is any standardization impact and what is the standardization impact are discussed in clause 5. **** Skip non-relevant text **** - Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself. - Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

Based on the above it is clear that the feedback information is strictly related to the AI/ML model, and for the AI/ML-based CCO the only AI/ML model which is supported by the standard is the one hosted by the gNB(-CU) which predicts the CCO issue.

Observation 1: For the AI/ML-based CCO use case, the only AI/ML model which is supported by the standard is the one hosted by the gNB(-CU) which predicts the CCO issue.

As a consequence, the feedback information should be for evaluating the CCO issue prediction. There were comments from some companies in past RAN3 meetings that it is not possible to evaluate the predicted CCO issue, and that only the future coverage state can be assessed. However, in our understanding, when deriving the future coverage states for both local and neighbour nodes, at the time the predicted CCO issue was expected to occur these future coverage state should already have been activated to prevent the issue. Therefore, if the issue does not occur – for example, by checking whether UE performance collected just after the Time for Predicted CCO Issue is still “good” – this is already a feedback information to evaluate the CCO issue prediction. This suggests that the model correctly predicted that at a certain time the issue was expected but, due to the proactive CCO mechanism, it was avoided. If UE performance is “bad”, then it means that the model provided inaccurate predictions (in terms of CCO issue type and/or Time for Predicted CCO Issue).

Proposal 4: RAN3 to agree that feedback information is needed to evaluate the CCO issue prediction.

A TP to AI/ML BLCR to TS 38.423 reflecting the above proposals can be found in [8].

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Proposal 1: RAN3 to agree to introduce a new IE UE Measured Radio Performance in XnAP in order to also exchange UE-measured radio-level metrics (RSRP, RSRQ, SINR) corresponding to those UEs handed over between cells affected by a coverage state change.

Proposal 2: RAN3 to agree to enhance the XnAP Data Collection Reporting (Initiation) procedures to enable the transfer over Xn of UE-measured radio-level metrics as collected by the neighbor gNB.

Proposal 3: RAN3 to agree to transfer the current coverage state of the neighbour gNB to the local node over the Xn interface as a feedback information. No specification impact if foreseen.

Observation 1: For the AI/ML-based CCO use case, the only AI/ML model which is supported by the standard is the one hosted by the gNB(-CU) which predicts the CCO issue.

Proposal 4: RAN3 to agree that feedback information is needed to evaluate the CCO issue prediction.

A TP to AI/ML BLCR to TS 38.423 reflecting the above proposals can be found in [8].

# 4. Reference

- RP-251245, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#108

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- R3-250465, Further discussions on AI/ML enabled CCO, Ericsson, Jio Platforms (JPL), InterDigital

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP TR 37.817 “Study on enhancement for Data Collection for NR and EN-DC”, Rel-17

- R3-255362, (TP for AIML BLCR to TS 38.423) Additional feedback information for AI/ML-based Coverage and Capacity Optimization, Huawei, Jio Platforms, Deutsche Telekom, Orange, Vodafone