3GPP TSG-RAN WG3 Meeting #129       	   				    R3-255384

Bengaluru, IN, 25th – 29th Aug, 2025

Agenda Item:		22.2

Source:			China Telecom

Title:	Further discussion on multi-hop relay

Document for:	 	Discussion, Decision

- Introduction

There are some remaining open issues to be solved for the multi-hop relay:

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

FFS whether the same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

FFS whether upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1.

FFS whether to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel is needed can be further discussed in next meeting.

In this contribution, we will further discuss above left open issues.

- Discussion

RAN2 already agreed to reuse the single-hop relay mechanism to support the Local ID allocation for multi-hop relay. Furthermore, it is assumed that the uniqueness of local ID within the cell is guaranteed by gNB. it is proposed to turn the following WA into the agreement.

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

Proposal 1: Turn WA to agreement: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

For L2 U2N Remote UE, when its RRCSetupRequest message is forwarded to gNB, gNB-DU needs to know the first relay UE in order to generate at least the PC5 Relay RLC channel configuration send it to gNB-CU for relaying of U2N Remote UE’s SRB1. If the gNB-DU cannot know that the RRCSetupRequest message from U2N Remote UE is delivered via which relay UE, the gNB DU may identify multi-hop relay operations as single-hop relay operations, thus causing misconfiguration of the PC5 Relay RLC channel. Due to the fact that the Remote UE Local ID and peer UE ID are already included in the PC5 RLC Channel to Be Modified List IE the UE CONTEXT MODIFICATION REQUEST message, and the Remote UE Local ID is unique within the last relay UE, we think that gNB-DU can already implicitly identify the first relay UE from gNB-CU via the current mechanism.

Proposal 2: Upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1.

Proposal 3: The gNB-DU can confirm whether the RRC message is delivered via the multi-hop relay, based on the Remote UE Local ID.

In addition, regarding the issue of whether the same PC5 RLC channel ID can be assigned to both the parent UE and the child UE. This involves whether the PC5 RLC channel ID is unique to a UE.

On the other hand, according to TS 38.331, the SL mapping configuration of the Relay UE includes the Remote UE local ID and the PC5 channel ID. Simultaneously, for a Relay UE, it can know the egress RLC tunnel/ingress RLC tunnel used for relay operations and corresponding next hop/previous hop UE IDs via PC5 connection. So it is technically feasible to assign the same PC5 RLC channel ID toward the parent UE and child UE.

Proposal 4: It is technically feasible to assign the same PC5 RLC channel ID toward the parent UE and child UE.

- Conclusions

Based on the above discussion, we propose the following:

Proposal 1: Turn WA to agreement: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

Proposal 2: Upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1.

Proposal 3: The gNB-DU can confirm whether the RRC message is delivered via the multi-hop relay, based on the Remote UE Local ID.

Proposal 4: It is technically feasible to assign the same PC5 RLC channel ID toward the parent UE and child UE.

- References