3GPP TSG-RAN WG3 Meeting #129															R3-255509

Bengaluru, India, 25th - 29th Aug, 2025

Agenda Item:	15.2

Source:	Huawei, Ericsson, Thales, Jio Platforms, Deutsche Telekom

Title:	(TP to BL CR for TS 36.300) Hard FLSO and MME management

Document for:	discussion

# 1	Discussion

In this paper, is NTN IoT related paper on MME Management similar to NR NTN in [2].

If TP of Hard FLSO and AMF management [2] is agreed by RAN3, we kindly ask RAN3 to review the following TP for BL CR 36.300 IoT NTN and agree it.

# 2	References

- R3-25xxxx, Downlink NG Transmission Suspend/Resume, Ericsson , Thales, Huawei et al.

- R3-25xxxx, (TP to BL CR for TS 38.300) Hard FLSO and AMF management, Huawei

# Annex A: TP for TS 38.300 against For MME Management

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

#### 23.21.5.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption and/or S1 link interruption may occur during the transition between the feeder links.

In this version of the specification, for regenerative payload, the MME is not changed for an NTN payload serving UEs in all RRC state during the feeder link switch over.

#### 23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs at feeder link switchover is performed by means of handover, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, E-RAB Modification Indication procedure) may be used.

The point in time when the hard feeder link switch over is performed at the MME may be configured or determined by the NTN control function.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>