3GPP TSG-RAN WG3 Meeting #129															R3-255515

Bengaluru, India, 25th - 29th Aug, 2025

Agenda Item:	15.2

Source:	Huawei, Jio Platforms, Ericsson

Title:	Discussion on Reply LS to SA2 on support of Full eNB as Regenerative Payload

Document for:	Discussion

# 1	Discussion on eNB only as regenerative payload without CN functions

In the RAN3#126 meeting, RAN3 received the LS from SA2 on the Regenerative-based Satellite Access for E-UTRAN/EPC, where only the eNB is on-board, without any CN components [2]. Based on it, the WID was updated accordingly, and the stage 2/3 details were agreed to support the eNB-only regenerative payload architecture at RAN3#127.

RAN3 never responded to SA2, RAN3 should send a reply LS back to SA2, indicating the support for the eNB-only regenerative payload architecture, and including the potential agreements made at this meeting for SA2’s consideration.

- RAN3 to send a reply LS to SA2, as response to R3-247016/ S2-2410918, containing all potential agreements having SA2 impacts made at this meeting.

The reply to SA2 proposed the following text also in R3-255516:

RAN3 thanks SA2 for the LS on the Support of Regenerative-based Satellite Access. For the Regenerative-based Satellite Access for E-UTRAN/EPC, RAN plenary already approved the updated WID in RP-243278 to support the eNB only as the regenerative payload. RAN3 would like the provide the following agreements for SA2 consideration. For the eNB only as the regenerative payload - The S1 removal procedure is introduced. - No new S1 suspend/resume procedures. - RAN3 assumes that during the feeder link switch, the MME is not changed for the UEs. - The existing S1 mechanisms can support the feeder link switch over. For the S&F operation mode - During the S&F mode transition (i.e., from S&F mode to normal mode or vice versa), the MME(s) is not changed for the eNB regenerative payload served UEs.

# 3	Conclusion

RAN3 is kindly requested to discuss the polite principle to reply to SA2, acknowledging the work and providing an update of the work as it is described in R3-255516.