3GPP TSG-RAN WG3 Meeting #129														       R3-255532

Bengaluru, India, 25th – 29th August 2025

Agenda Item:	13.2

Source:	Ericsson, Jio Platforms, Lenovo, NTT DoCoMo

Title:	PRACH resources for RACH-less LTM

Document for:	Discussion, Agreement

# Introduction

In this contribution, we provide further discussion on the how to handle preamble index allocation for RACH-less LTM.

# Discussion

In inter-CU LTM, during the preparation phase, the source gNB requests PRACH resources from each candidate gNB, to be used later for Early UL sync. The candidate gNB allocates a pool of PRACH resources, and as long as they are kept available for the source gNB, the source gNB can send PDCCH orders for TA value acquisition. When a certain PRACH resource is used for one UE, the source gNB cannot use the same PRACH resource for another UE, i.e., once the source gNB1 has sent a PDCCH order targeting a certain PRACH resource #1 allocated by a candidate gNB2 to UE1, it cannot send another PDCCH order targeting the same PRACH resource #1 allocated by the same gNB2 to a UE2 until the TA value for UE1 is acquired. If that happens, the source gNB will not be able to discriminate whether the TA value it receives is for UE1 or UE2.

- Once the source gNB1 has sent a PDCCH order targeting a PRACH resource #1 allocated by a candidate gNB2 to UE1, it cannot send another PDCCH order targeting the same PRACH resource #1 allocated by the same gNB2 to a UE2 until the TA value for UE1 is acquired.

When an LTM execution is triggered, the source DU will send an LTM Cell Switch Command MAC CE to the UE, and RACH-less LTM will be attempted if the MAC CE will refer to PRACH resources previously allocated by the candidate cell for early UL synchronization or leading to CFRA based cell switch. If, for any reason, the candidate gNB2 has released the PRACH resources previously allocated for LTM for the source gNB1, the LTM execution will fail.

- At LTM execution, if RACH-less LTM is attempted but the candidate gNB has released the PRACH resources, the LTM execution fails.

- If a candidate gNB releases PRACH resources initially provided to the source gNB as PRACH resources for LTM, and the source gNB is not informed, this will lead to failures in TA acquisition or LTM execution.

RACH-less LTM is only possible if PRACH resources are available, and also that the amount of such resources is quite limited (even if the same Preamble Index is reused with multiple PRACH occasions). It is therefore  important to ensure that the amount of PRACH resources for LTM is kept under control. The availability of PRACH resources for LTM can be limited due to reasons not related to LTM. For example, a candidate gNB may need to use PRACH resources for non-LTM purposes and decide to deallocate some PRACH resources previously allocated for LTM.

In general, we think that, given the scarcity of PRACH resources, and to ensure that RACH-less LTM can be used effectively, a robust solution needs to consider both the source gNB and the candidate gNB “interests”. The source gNB “interest” is to use PRACH resources for RACH-less LTM. Ideally, requested PRACH resources from a candidate gNB should always be available, or otherwise Early UL sync or LTM execution will fail. The candidate gNB “interest” is instead, to reduce the amount of time during which PRACH resources allocated for LTM are not used. Obviously, the standard cannot restrict the gNB behaviour, so the candidate gNB, if needed, can simply release PRACH resources previously reserved for LTM, and use them for other purposes.

In our current signaling baseline, the two “interests” just mentioned are only partially supported. The source gNB can request PRACH resources to a candidate gNB, and a candidate gNB can still release them if needed, but in this case the source gNB will not know about it. To make the LTM solution more robust (and avoid unnecessary failures), we propose that the candidate gNB informs the source gNB when it deallocates Random Access Preamble index that were previously allocated for LTM.

- For inter-CU LTM, a candidate gNB informs the source gNB when Preamble Indexes assigned LTM are deallocated.

The same mechanism is also good to have for intra-CU LTM, the related proposal is presented below.

- For intra-CU LTM, a candidate gNB-DU informs the source gNB-DU when Preamble Indexes assigned for LTM are deallocated.

The stage 2 updates for TS 38.300 and TS 38.401 to capture the above proposals are detailed in R3-255269.

An obvious consequence of the release of previously allocated Preamble Indexes by the candidate gNB (or candidate gNB-DU) is that the source gNB (or the source gNB-DU) cannot rely on the same amount of resources originally provided. Since the PRACH resources can be used for multiple UEs, it can happen that the remaining PRACH resources may be sufficient to handle the UEs, or maybe not. In the latter case, the use of RACH-less LTM cannot be guaranteed, and the solution will not work as initially intended. This issue can be addressed allowing the source gNB (source gNB-DU) to request for more PRACH resources. One way to achieve this is to reuse the existing procedures, (the LTM Configuration Update procedure in Xn, and the UE Context Modification procedure in F1), where, in the initiating message the source gNB (source gNB-DU) requests for new allocation of Preamble Indexes, and in the return message the candidate gNB (candidate gNB-DU) can provide new Preamble Indexes if available.

- For inter-CU LTM, a source gNB can request a candidate gNB for new allocation of Preamble Indexes.

- For intra-CU LTM, a source gNB-DU can request a candidate gNB-DU (via the gNB-CU) for new allocation of Preamble Indexes.

- Agree on the TPs for 38.423 and 38.473 in R3-255533 and R3-255534 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	Once the source gNB1 has sent a PDCCH order targeting a PRACH resource #1 allocated by a candidate gNB2 to UE1, it cannot send another PDCCH order targeting the same PRACH resource #1 allocated by the same gNB2 to a UE2 until the TA value for UE1 is acquired.

Observation 2	At LTM execution, if RACH-less LTM is attempted but the candidate gNB has released the PRACH resources, the LTM execution fails.

Observation 3	If a candidate gNB releases PRACH resources initially provided to the source gNB as PRACH resources for LTM, and the source gNB is not informed, this will lead to failures in TA acquisition or LTM execution.

Proposal 1	For inter-CU LTM, a candidate gNB informs the source gNB when Preamble Indexes assigned LTM are deallocated.

Proposal 2	For intra-CU LTM, a candidate gNB-DU informs the source gNB-DU when Preamble Indexes assigned for LTM are deallocated.

Proposal 3	For inter-CU LTM, a source gNB can request a candidate gNB for new allocation of Preamble Indexes.

Proposal 4	For intra-CU LTM, a source gNB-DU can request a candidate gNB-DU (via the gNB-CU) for new allocation of Preamble Indexes.

Proposal 5	Agree on the TPs for 38.423 and 38.473 in R3-255533 and R3-255534 that capture the above proposals.