3GPP TSG-RAN WG3 Meeting #129	R3-255541

# Bengaluru, India, 25th - 29th August 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	Open topics for Rel-19 work on A-IoT

Document for:	Discussion and Conclusion

# 1	Introduction

Several open issues are still to be handled, some other issues arise when reading the BL CRs with a fresh mind,  some of the issues are well known, others are new, others notoriously known.

# 2	Discussion

## 2.1	Inventory Request and Inventory Report procedures - association to A-IoT radio procedures missing

NGAP refers to AS procedures in the following cases:

-	Paging: "The purpose of the Paging procedure is to enable the AMF to page a UE in the specific NG-RAN node.  ... At the reception ... the NG-RAN node shall perform paging of the UE ...".

-	Initial UE Message: ... is used when the NG-RAN node has received from the radio interface the first uplink NAS message ..."

The A-IoT BL CR for NGAP currently specifies for the corresponding/equivalent A-IoT procedures the following:

-	Inventory Request: "The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventory over the A-IoT radio."

There is, however, not inventory procedure defined over the A-IoT radio, the procedure name used in draft 38.391 is A-IoT paging.

-	Inventory Report: "The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node following a successful Inventory Request procedure.

This description omits completely the interaction with A-IoT radio: Inventory Report is not in response to a successful Inventory Request procedure - this procedure can be successfully terminated w/o having started A-IoT paging. Inventory Report is the response of a device to A-IoT paging.

Proposal 1:	In the NGAP BL CR, correct the General section of the NGAP Inventory Request procedure in replacing "inventory" by "A-IoT paging", to say that "The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventoryA-IoT paging over the A-IoT radio."

Proposal 2:	In the NGAP BL CR, correct the General section of the NGAP Inventory Report procedure by clarifying the association to the A-IoT radio procedure to say that "The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node for one or several A.IoT devices successfully responding to A-IoT paging, following a successful Inventory Request procedure."

The running CR for 38.300 managed by RAN2 does not explicate an "Inventory procedure over A-IoT radio interface" nor a "command procedure over A-IoT radio interface".

-	"Inventory over the A-IoT radio interface" is rather "A-IoT paging, random access and D2R data transmission as specified in 16.x.5 A-IoT MAC Layer functions."

-	"Command procedure over A-IoT radio interface" is rather "R2D and D2R data transmission as specified in 16.x.5 A.IoT MAC Layer functions".

Proposal 3:	In the 38.300 BL CR, correct the procedural description of the "inventory/command procedure over the A-IoT radio interface" and refer to actual A-IoT radio interface (A-IoT MAC) procedure names used in the RAN2 running CR for 38.300 in section 16.x.5.

## 2.2	Inventory of "all" devices

Although dealing with security concerns is not within RAN WGs' Terms of References, we believe that the security threat stemming from inventorizing "all" devices in a certain area is quite evident, as such possibility represents a threat related to an easy to accomplish DoS (Denial of Service) attack, with an attacker replaying a "page all" message able to interrupt any ongoing A-IoT procedure, as per RAN2 agreement. We propose to remove the "page all" choice from stage 3 and await the outcome of discussions in SA3. The other alternative would be to keep the possibility and to later on remove it, but such approach appears rather irresponsible and wrong, given the obvious threat.

Proposal 4:	Remove the "page all" possibility in the Inventory Request procedure. If seen necessary, we could liaise to RAN2 and SA3 on that decision.

## 2.3	Indicating multiple individual device IDs in Inventory Request

Service operations on Nnef and Naiotf, as specified in TS 23.369, clearly indicate the possibility of indicating multiple individual A-IoT device IDs.

-	Either AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation.

-	Either AIoT Device ID(s) or AIoT Device ID filter information for the command operation.

We have currently specified only the possibility of having a single individual device ID included in NGAP Inventory (which is part of command operation).

That doesn’t seem like matching the possibilities outlined in 23.369 and there is no good reason to limit the NGAP protocol design to that respect.

There might be the possibility that filtering information is not possible due to too few devices target, in which case rather a list of individual device IDs should be provide, for not breaching privacy requirements. There could be also the possibility that neither group ID nor filer information is suited for targeting a group of devices with a single A-IoT operation.

Proposal 5:	Introduce in the NGAP and stage 2 BL CR the possibility to indicate multiple individual device IDs in the Inventory Request message.

## 2.4	Inventory Complete Indication

The BL CR for 38.300 indicates an open issue on whether an indication on inventory complete is provided in the (last) Inventory Report message or a new procedure is introduced.

Our view is that, having introduced the possibility to request the CN to release the A-IoT session by means the gNB initiated A-IoT Session Release procedure, provides already an appropriate means for the gNB to perform the requested functionality.

If the gNB is of the impression (a very implementation specific term) that the session can be terminated, e.g. due to inactivity on the A-IoT radio interface, no devices responding any more, no command procedure triggered by the CN, etc., the gNB should have all the means to request the release of the session, which is the ultimate goal for even indicating an end-of-operation to the CN, regardless whether it is an Inventory, or Inventory and Command session.

We are however open to also introduce the "Inventory Complete Indication" in NGAP BL CR, assuming that this indication is only used by the gNB if it can deduce that the inventory is completed (i.e. all devices addressed).

Proposal 6:	We therefore propose to remove the open issue in the stage-2 BL CR in section 16.xx.x3 and modify step 7 accordingly and add a complete indication in the NGAP BL CR.

## 2.5	A-IoT support indication in NG interface management procedures

We have established a working assumption to indicate - on node level - the support of A-IoT by the gNB.

This might be a pragmatic (a euphemism for "quick and dirty") approach if we would have to consider Rel-19 only.

However, in the envisioned additions to Rel-19 functionality in later releases, e.g. support of an additional topology, support of device originated traffic, support of longer ranges (aiming at re-use of cell sites for provision of A-IoT services), it might become quite an odd decision to not utilise existing protocol structures of the NG interface management procedures, NG Setup and Configuration Update, given the likely requirement to support gNBs providing an arbitrary combination of these additions and the likelihood that protocol functions (on NG and A-IoT radio) are not compatible, hence we should consider forward compatibility of the chosen approach.

The following figures shows how NG Setup typically works:

Figure: basic NG interface setup

The AMF would learn form the NG-RAN node the PLMNs and slices supported. The NG-RAN node would learn in return the PLMNs and slices supported by the AMF. This is concisely described in TS 23.501 section 5.15.8. This also implies that PLMNs and slices not supported by the AMF will not be included in the NG-Setup Response, i.e. TACs configured in the NG-RAN node for which not a single PLMN ID nor a single slice is supported by the AMF cannot be served by the AMF contacted by the NG-RAN node. If not a single TAC can be served by the contacted AMF, The AMF will respond with NG Setup Failure, utilising cause codes like " Slice(s) not supported "or " Unknown PLMN or SNPN".

Further, "RAT Information" was introduced in order to indicate TAC for very specific usage, like for unlicensed spectrum, NB-IoT and satellite access, for which stage 2 is specified in TS 23.501 §5.3.2.3. This results in requirements to have Tracking Areas serving certain access types only (see references in §5.3.2.3 for further details).

The principle of considering the PLMN ID of a device while determining whether such device would have access to radio resources has not completely vanished by the introduction of A-IoT, the device permanent identifier still exhibits the PLMN Id, as specified in TS 23.369.

So, following the basic decision to integrate support of A-IoT within 5GS, moreover, within NG-RAN, and utilising NGAP interface management protocol functions, raises the question, how the AIOTF, in case of direct connectivity and the AMF in case of indirect connectivity would become aware of the PLMN ID supported by the NG-RAN node for A-IoT, given current structure of the NGAP BL CR.

The open topic on how the NG-RAN node could become aware of whether the AMF/AIOTF provides necessary system functions for support of Rel-19 and potential future release A-IoT functions/RATs might become an interesting question, if we don’t want to rely completely on OAM for those kind of RAN-CN interworking.

It is evident, that current working assumption related to indication of A-IoT support by NG-RAN does not follow principles established in previous 5G releases and that the decision made at RAN3#128 was probably not taking into account all aspects.

It can be seen that the exchange of supported PLMNs/slices provides the ability to support deployment of AMFs serving slices and PLMNs in a non-homogenous way.

Proposal 7:	Re-consider the Working Assumption on indicating the support of A-IoT by means of a node-level support indication. One potential solution would be to replace the current approach by utilising the Supported TA List IE, modifying the RAT Information IE or introducing a new A-IoT specific indication.

## 2.6	AIOTF node ID

AIOTFs fulfil a similar role towards A-IoT devices as an AMF towards UE does. It also connects to NG-RAN in the same way as an AMF does. So, having decided to re-use the NG management functions for A-IoT would make the conclusion obvious that addressing an AIOTF could be performed in the same way as addressing an AMF, i.e. to allocate a special GUAMI for an AIOTF, probably indicating the fact that the GUAMI is not for an AMF but for an AIOTF.

Proposal 8:	Utilise the GUAMI to indicate the AIOTF node ID, introducing an additional indication to denote the GUAMI to be AIOTF specific.

# 3	Conclusion and Proposals

These are the proposals developed in this paper:

Proposal 1:	In the NGAP BL CR, correct the General section of the NGAP Inventory Request procedure in replacing "inventory" by "A-IoT paging", to say that "The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventoryA-IoT paging over the A-IoT radio."

Proposal 2:	In the NGAP BL CR, correct the General section of the NGAP Inventory Report procedure by clarifying the association to the A-IoT radio procedure to say that "The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node for one or several A.IoT devices successfully responding to A-IoT paging, following a successful Inventory Request procedure."

Proposal 3:	In the 38.300 BL CR, correct the procedural description of the "inventory/command procedure over the A-IoT radio interface" and refer to actual A-IoT radio interface (A-IoT MAC) procedure names used in the RAN2 running CR for 38.300 in section 16.x.5.

Proposal 4:	Remove the "page all" possibility in the Inventory Request procedure. If seen necessary, we could liaise to RAN2 and SA3 on that decision.

Proposal 5:	Introduce in the NGAP and stage 2 BL CR the possibility to indicate multiple individual device IDs in the Inventory Request message.

Proposal 6:	We therefore propose to remove the open issue in the stage-2 BL CR in section 16.xx.x3 and modify step 7 accordingly and add a complete indication in the NGAP BL CR.

Proposal 7:	Re-consider the Working Assumption on indicating the support of A-IoT by means of a node-level support indication. One potential solution would be to replace the current approach by utilising the Supported TA List IE, modifying the RAT Information IE or introducing a new A-IoT specific indication.

Proposal 8:	Utilise the GUAMI to indicate the AIOTF node ID, introducing an additional indication to denote the GUAMI to be AIOTF specific.

Final Proposal: Agree on Text Proposals on the BL CRs as can be found for Proposals 1-3 in R3-255542, for Proposals 4 and 5 in R3-255543, for Proposal 7 in R3-255544 and Proposals 6 and 8 in the Annexes of this paper.

# 4	References

[1]	"reference"

# Annex A Text Proposal for TS 38.300 BL CR (R3-255072) for "Inventory Complete Indication"

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g. Approximate number of Target A-IoT devices.

Editor’s Note 1:	Whether the Expected D2R message size is mandatory is FFS.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 2:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

7.	If the gNB is able to deduce that the inventory procedure Is completed it includes an Inventory Complete Indication in the final Inventory Report message.

# Annex B Text Proposal for TS 38.413 BL CR (R3-255076) for "Inventory Complete Indication"

-----------------Start of the Next Change-----------------

### 8.xx.2	Inventory Report

#### 8.xx.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node following a successful Inventory Request procedure. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.2.2	Successful Operation

Figure 8.xx.2.2-1: Inventory Report

The NG-RAN node initiates the procedure by sending an INVENTORY REPORT message for the Inventory service.

If the Inventory Complete Indication IE is included in the INVENTORY REPORT message the A-IoT CN node shall use it as specified in TS 38.300 [8].

#### 8.xx.2.3	Abnormal Conditions

Void.

-----------------Start of the Next Change-----------------

#### 9.2.x.4	INVENTORY REPORT

This message is sent by the NG-RAN node to provide the inventory report to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Report Transfer | M |  | OCTET STRING | Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4 | YES | reject |
| Inventory Complete Indication | O |  | ENUMERATED (inventory complete, ...) |  | YES | reject |

-----------------Start of the Next Change-----------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

InfoOnRecommendedCellsAndRANNodesForPaging,

IntersystemSONConfigurationTransfer,

InventoryCompleteIndication,

InventoryRequestTransfer,

InventoryResponseTransfer,

InventoryFailureTransfer,

InventoryReportTransfer,

LAI,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-IntersystemSONConfigurationTransferUL,

id-InventoryCompleteIndication,

id-InventoryRequestTransfer,

id-InventoryResponseTransfer,

id-InventoryFailureTransfer,

id-InventoryReportTransfer,

id-LocationReportingRequestType,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

##### -- Inventory Report procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- INVENTORY REPORT

--

-- **************************************************************

InventoryReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {  InventoryReportIEs} },

...

}

InventoryReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier												PRESENCE mandatory	}|

{ ID id-AIoT-CorrelationIdentifier	CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE mandatory	}|

{ ID id-InventoryReportTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING InventoryReportTransfer	)	PRESENCE mandatory	}|

{ ID id-InventoryCompleteIndication	CRITICALITY reject	TYPE InventoryCompleteIndication	)					PRESENCE optional	},

...

}

-----------------Start of the Next Change-----------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

InventoryCompleteIndication ::= ENUMERATED {inventory_complete, ...}

InventoryRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { InventoryRequestTransferIEs} },

...

}

InventoryRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier							PRESENCE	mandatory	}|

{ ID id-AIoT-DeviceIdentificationRequested		CRITICALITY reject	TYPE AIoT-DeviceIdentificationRequested				PRESENCE	mandatory	}|

{ ID id-AIoT-RequestedServiceAreaInformation	CRITICALITY reject	TYPE AIoT-RequestedServiceAreaInformation			PRESENCE mandatory	}|

{ ID id-AIoT-InventoryExpectedD2RMessageSize	CRITICALITY reject	TYPE AIoT-InventoryExpectedD2RMessageSize			PRESENCE mandatory[FFS]	}|

{ ID id-AIoT-InventoryAssistanceInformation		CRITICALITY reject	TYPE AIoT-InventoryAssistanceInformation				PRESENCE	optional		}|

{ ID id-AIoT-FollowonCommandIndication			CRITICALITY reject	TYPE AIoT-FollowonCommandIndication					PRESENCE	optional		},

...

}

-- J

-----------------Start of the Next Change-----------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ExtendedOldAMF			    						ProtocolIE-ID ::= 443

id-InventoryRequestTransfer							ProtocolIE-ID ::= 993 --to be allocated

id-InventoryResponseTransfer						ProtocolIE-ID ::= 994 --to be allocated

id-InventoryFailureTransfer							ProtocolIE-ID ::= 995 --to be allocated

id-InventoryReportTransfer							ProtocolIE-ID ::= 996 --to be allocated

id-CommandRequestTransfer							ProtocolIE-ID ::= 997 --to be allocated

id-CommandResponseTransfer							ProtocolIE-ID ::= 998 --to be allocated

id-CommandFailureTransfer							ProtocolIE-ID ::= 999 --to be allocated

id-AIOTSessionReleaseCommandTransfer					ProtocolIE-ID ::= 1000 --to be allocated

id-AIOTSessionReleaseCompleteTransfer					ProtocolIE-ID ::= 1001 --to be allocated

id-AIOTSessionReleaseRequestTransfer				ProtocolIE-ID ::= 1002 --to be allocated

id-AIoT-CorrelationIdentifier							ProtocolIE-ID ::= 880 --to be allocated

id-AIOTFIdentifier									ProtocolIE-ID ::= 881 --to be allocated

id-AIoT-DeviceIdentificationRequested					ProtocolIE-ID ::= 882 --to be allocated

id-AIoT-RequestedServiceAreaInformation					ProtocolIE-ID ::= 883 --to be allocated

id-AIoT-InventoryAssistanceInformation					ProtocolIE-ID ::= 884 --to be allocated

id-AIoT-FollowonCommandIndication						ProtocolIE-ID ::= 885 --to be allocated

id-RAN-AIOT-Device-NGAP-ID								ProtocolIE-ID ::= 886 --to be allocated

id-AIoT-CommandAssistanceInformation					ProtocolIE-ID ::= 887 --to be allocated

id-AIoT-NASPDU										ProtocolIE-ID ::= 888 --to be allocated

id-AIoT-InventoryExpectedD2RMessageSize					ProtocolIE-ID ::= 889 --to be allocated

id-AIoT-Support											ProtocolIE-ID ::= 890 --to be allocated

id-InventoryCompleteIndication							ProtocolIE-ID ::= 891 --to be allocated

# Annex C Text Proposal for TS 38.413 BL CR (R3-255076) for the AIOTF node ID

Note: 	The proposal could have also been implemented by replacing in ASN.1 the type reference "AIOTFIdentifier" by "GUAMI", whatever is more preferred.

-----------------Start of the Next Change-----------------

#### 9.3.3.xx2	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | GUAMI 9.3.3.3 |  |

-----------------Start of the Next Change-----------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

AIOTFIdentifier ::= GUAMI