3GPP TSG-RAN WG3 Meeting #129	R3-255542

# Bengaluru, India, 25th - 29th August 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for BL CR 38.300 and 38.413] Correcting references to AS related protocol elements

Document for:	Discussions & Approval

# 1	Introduction

This TP follows proposals 1-3 as discussed in R3-255541:

Proposal 1:	In the NGAP BL CR, correct the General section of the NGAP Inventory Request procedure in replacing "inventory" by "A-IoT paging", to say that "The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventoryA-IoT paging over the A-IoT radio."

Proposal 2:	In the NGAP BL CR, correct the General section of the NGAP Inventory Report procedure by clarifying the association to the A-IoT radio procedure to say that "The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node for one or several A.IoT devices successfully responding to A-IoT paging, following a successful Inventory Request procedure."

Proposal 3:	In the 38.300 BL CR, correct the procedural description of the "inventory/command procedure over the A-IoT radio interface" and refer to actual A-IoT radio interface (A-IoT MAC) procedure names used in the RAN2 running CR for 38.300 in section 16.x.5.

# 2	Text Proposal for BL CR 38.300 (R3-255072)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g. Approximate number of Target A-IoT devices.

Editor’s Note 1:	Whether the Expected D2R message size is mandatory is FFS.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 2:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The Inventory procedure over the A-IoT radio interface is performed, i.e A-IoT paging, A-IoT Access and D2R data transmission, as specified in section 16.x.5 A-IoT Mac Layer Functions.

5./6.	Upon receiving the response(s) from A-IoT paging from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

7.	Upon completion of the inventory procedure the gNB may send an Inventory Complete Indication.

Editor’s Note 3:	It is FFS whether this inventory complete indication is done via a new procedure or introducing indication IE in the current Inventory Report message.

### 16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices or in parallel to other command procedures targeting other A-IoT devices within the same session.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU corresponding to the targeted A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type (FFS).

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface, i.e. R2D and D2R data transmission, as specified in section 16.x.5 A-IoT MAC Layer Functions.

4.	The gNB replies to the Command Request with the Command Response messageincluding the AIOTF Identifier  the Correlation ID and the RAN A-IoT device NGAP ID received in the Command Request, and the A-IoT NAS PDU received from the device.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

# 3	Text Proposal for BL CR 38.413 (R3-255076)

-----------------Start of the Next Change-----------------

### 8.xx.1	Inventory Request

#### 8.xx.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform A-IoT paging over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

-----------------Start of the Next Change-----------------

### 8.xx.2	Inventory Report

#### 8.xx.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node for one or several A-IoT devices successfully responding to A-IoT paging, following a successful Inventory Request procedure. This procedure applies only if the NG-RAN node is a gNB.

-----------------Start of the Next Change-----------------

### 9.2.x		A-IoT Messages

#### 9.2.x.1	INVENTORY REQUEST

This message is sent by the A-IoT CN node to trigger the NG-RAN node to perform A-IoT paging over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node