3GPP TSG-RAN WG3 #129	R3-255595

Bengaluru, India, August 25-29, 2025

Agenda Item:	10.2

Source:	Ericsson

Title:	Ping-pong avoidance for LTM

Document for:	Discussion and Decision

# 1	Introduction

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

This contribution discusses considerations regarding MRO feature enhancements for LTM. In particular, it discusses ping-pong detection, avoidance, and the necessary signalling.

# 2	Optimization of LTM cell switch ping-pong

Ping-pong for LTM cell switches is a condition that needs to be avoided. Optimizing configuration to avoid ping-pong from an LTM perspective is done differently for the Intra-DU ping pong case and the Inter-DU ping pong case. These cases will be detailed in the following subsections. It is worth noting that during RAN3#127, this problem was broken down into two questions, namely:

- Which node is responsible for identifying LTM ping-pong (CU or DU)?

- Which node is responsible for resolving LTM ping-pong (CU or DU)?

During RAN3#127, the following agreements were taken:

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

During RAN3#127bis, the following agreement was taken:

For intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue.

During RAN3#128, the following was agreed and proposed provided for down selection and FFS:

CU informs relevant information to the target DU via CU-DU Cell Switch Notification.

During RAN3#128, there was a discussion on what information should be sent from the CU to the DU if the CU discovers an LTM ping-pong situation. The meeting suggested the following four solutions for down selection:

1a: CU provide full UHI list with L1/L3 type to the DU (maximum item number is 16).

1b: CU provide full UHI list (except PScell) with L1/L3 type to the DU (maximum item number is 16).

2a: CU provide last consecutive L1 entries to the DU.

2b: CU provide FFS number of last consecutive L1 entries to the DU.

Each of these solutions have their own characteristics.

The agreed use-case discussed is ping-pong detection and resolution. For this reason, it should be sufficient that the CU sends an IE with a name indicating that the IE is for signalling ping-pong detection. The IE should contain the list of cells that the UE connected to.

A second approach would be to send a more general “UHI” IE, with an explicit indicator that the UHI list is sent due to the CU discovering a ping-pong. However, there is so far no other use-case agreed for this IE.

For these reasons, we believe that sending an explicit ping-pong detection IE is the better way.

The CU signals to the DU that a ping-pong situation has occurred through an explicit IE for this purpose.

By having an explicit IE for this purpose, there is no need for a ping-pong indicator inside the IE.

There is no need for a ping-pong indicator within the ping-pong IE.

As the use case is only to convey, from the CU to the DU, that the CU has detected an Intra-CU inter-DU LTM ping-pong, the IE sent to the DU needs to contain only information needed by the DU to resolve the ping-pong.

According to Proposal 1, the UHI IE explicitly concerns LTM cell switches. For this reason, this IE needs only to contain information about LTM cell switches but not L3 handover or PSCell changes.

The list of UHI information sent from the CU to the DU, for resolving ping-pong should contain only L1 information, not L3 information including PSCell changes.

As the definition of a ping-pong is that a UE is sent from a first cell to a second cell, and then back to the first cell again, all within a limited time, in theory only one entry (the second cell) should be needed in the UHI list. However, we believe that the DU can do a better job at resolving ping-pong if it has slightly more information to work on. This would also allow some flexibility for implementation that needs more entries for further analysis. We therefore suggest that the UHI contains up to 8 entries.

The list of UHI information sent from the CU to the DU, for resolving ping-pong should contain up to 8 entries.

As agreed during RAN3#128 (see above), the UHI IE shall be added to the CU-DU CELL SWITCH NOTIFICATION message.

Add a new IE to CU-DU CELL SWITCH NOTIFICATION message telling the DU that a ping-pong has occurred.

If ping-pong has not stopped by sending the UHI to the first DU and a cell switch occurs one again (thus yielding a “ping-pong-ping”), then the CU can send the UHI to the second DU as well. By the same procedure, the CU can then tell the second DU that a ping-pong has occurred.

The IE in the previous proposal can be used for notifying the second DU that a ping-pong has occurred for a particular UE as well.

# 4	Conclusion

- The CU signals to the DU that a ping-pong situation has occurred through an explicit IE for this purpose.

- There is no need for a ping-pong indicator within the ping-pong IE.

- The list of UHI information sent from the CU to the DU, for resolving ping-pong should contain only L1 information, not L3 information including PSCell changes.

- The list of UHI information sent from the CU to the DU, for resolving ping-pong should contain up to 8 entries.

- Add a new IE to CU-DU CELL SWITCH NOTIFICATION message telling the DU that a ping-pong has occurred.

- The IE in the previous proposal can be used for notifying the second DU that a ping-pong has occurred for a particular UE as well.