3GPP TSG-RAN WG3 Meeting #129															R3-255628

Bengaluru, India, 25th ~29th Aug, 2025

Title:	Clarification on the single UE XnAP association in inter-CU LTM

Agenda Item:	13.2

Source:	Huawei, NEC, LG Electronics

Document for:	Discussion

1	Introduction

RAN3 has agreed to implement a single UE XnAP association during inter-CU LTM preparation towards a certain candidate CU. The following agreement was made at RAN3 #127bis meetings:

Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.

And RAN3 also agreed to support parallel LTM preparation over Xn interface.

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

However, there are still 2 points that need further clarifications.

- Where should the single UE XnAP association agreement be captured in RAN3 specification?

- The parallel transaction handling for LTM in the latest BLCR is misleading.

This contribution provides further clarifications on above two issue.

2	Discussion

If there are multiple candidate cells in a candidate gNB, the source gNB may choose to send out the multiple HANDOVER REQUEST messages to the candidate gNB in parallel or in a sequential way. The latter case is also valid in case that the source gNB decides to prepare more cells in that candidate gNB after first round LTM preparation.

Capture single UE XnAP association

For the parallel preparation case, when the source gNB sends out the multiple HANDOVER REQUEST messages to the candidate gNB, the target NG-RAN node UE XnAP ID is still not available because the candidate gNB hasn’t have time to allocate it. In this case, as agreed earlier, the source gNB will use the same source NG-RAN node UE XnAP ID in all the HANDOVER REQUEST messages to a single candidate gNB. And in response, accordingly the candidate gNB should include the same target NG-RAN node UE XnAP ID in all the HANDOVER REQUEST ACK messages.

This behaviour shall be captured in a proper place in the specification to avoid any ambiguity. It may not proper to specify it in section 8.2.1 Handover Preparation in TS 38.413 to mandate the source gNB and the target gNB to allocate the same source/target UE Xn AP ID for multiple handover request procedures. As the stage 3 specification currently only captures the receiving node behavior for a single handover request transaction. Therefore, it would be better to capture the agreement in stage 2 TS 38.300 as below.

BLCR of TS 38.300 The procedure for LTM is as follows: 1. The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells. 2. The source gNB decides to configure LTM. 3. The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-gNB LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. For both intra and inter-gNB LTM, the source gNB may request the candidate cell(s)/gNB(s) to provide the CSI resource configuration. For inter-gNB LTM, the source gNB includes the same source NG-RAN node UE XnAP ID for all HANDOVER REQUEST messages to a candidate gNB. 4. Admission Control may be performed by the candidate cells(s)/gNB(s). 5. The candidate prepares and provides the LTM configuration(s) to the source gNB. For inter-gNB LTM, the candidate gNB(s) respond(s) with HANDOVER REQUEST ACKNOWLEDGE message to the source gNB including the generated RRC configurations for the accepted candidate cell. For both intra and inter-gNB LTM, the candidate may also include additional information related to the CSI-RS resource configuration and early sync information upon request. For inter-gNB LTM, each candidate gNB includes the same target NG-RAN node UE XnAP ID for all HANDOVER REQUEST ACKNOWLEDGE messages it responds.

Proposal 1: RAN3 to capture the single UE XnAP association handling for LTM preparation in stage 2 as shown above.

Furthermore, once the UE Xn association is setup between the source gNB and the target gNB (meaning that the source gNB receives the target NG-RAN node UE XnAP ID in the HANDOVER REQUEST ACK message), the source gNB should include the target NG-RAN node UE XnAP ID in the handover requests for any follow-up preparation. The stage 3 change is shown as below.

#### 9.1.1.1 HANDOVER REQUEST This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover. Direction: source NG-RAN node target NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. Source NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the source NG-RAN node; YES; reject. <skip irrelevant part>. LTM Handover Information Request; O; YES; reject. >LTM Information Request; O; 9.2.3.xx1; –. >Request for CSI-RS Resource Configuration; O; ENUMERATED (true, …); –. >Target NG-RAN node UE XnAP ID reference; O; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the target NG-RAN node; YES; ignore. Early Sync Information Request; O; 9.2.3.xx3; YES; ignore.

Proposal 2: Once the UE XnAP association is setup, the source gNB includes the target NG-RAN node UE XnAP ID in the handover request message for any follow-up preparation..

Parallel transactions handling

The specification text for parallel transactions in section 8.2.1 Handover Preparation in TS 38.413 was introduced for CHO in Rel-16. Conditional handover supports two kinds of parallel transactions, same source UE AP IDs and different source UE AP IDs. While LTM only supports parallel transactions with same source UE AP IDs.

Therefore, it is proposed to capture the parallel transaction description for LTM separately to avoid any ambiguity as below.

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns a LTM, parallel transactions are allowed only when using the same source UE AP ID. Possible parallel requests are identified by the target cell ID.

Proposal 3: RAN3 to capture the parallel transactions handling for LTM properly.

3.	Conclusion

Based on the discussion in this contribution, it is proposed:

Proposal 1: RAN3 to capture the single UE XnAP association handling for LTM preparation in stage 2 as shown above.

Proposal 2: Once the UE XnAP association is setup, the source gNB includes the target NG-RAN node UE XnAP ID in the handover request message for any follow-up preparation..

Proposal 3: RAN3 to capture the parallel transactions handling for LTM properly.

The stage 2 TP and stage 3 TP are provided in [1] and [2].

4 	Reference

[1] R3-255629	(TP for LTM BLCR for TS38.300): Clarification on the single Xn UE association in inter-CU LTM

[2] R3-255630	(TP for LTM BLCR for TS38.423): Clarification on the single Xn UE association in inter-CU LTM