3GPP TSG-RAN WG3 #129                                       R3-255660

Bangalore, IN, 25 - 29 Aug, 2025

Agenda item:	13.2

Source:	Samsung

Title:	Additional Discussion on inter-gNB-CU LTM

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we will further discuss other potential issue on inter-gNB-CU LTM(or with DC) besides the discussion in [4] and BLCR in [1], [2] and [3], and suggest the potential enhancement on Xn/F1 interface.

# 2 Discussion

## Issue 1: Co-existence between L3 SN change and inter-CU SCG LTM

Regarding to the R19 discussion on the co-existence between L3 HO and inter-CU LTM, below agreement is reached in the RAN2#127

L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied.

Let us review the R18 discussion from RAN3, the below agreements on LTM and L3 handover collision is achieved in the RAN3#121(-bis)

In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority. In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority, In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

Looking through the above conclusions, one important enhancement is to introduce a cause indication of LTM command triggered in the UE Context Modification Failure message when LTM and L3 handover are triggered almost simultaneously (refer to below figure). And below procedure text is specified in TS 38.473.

“If the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.”

This enhancement is also applicable for the case of inter-CU LTM with SA and following scenarios on LTM with NR-DC

1. Inter-CU MCG LTM without SN release

2. Inter-CU MCG LTM with SN release

3. Inter-CU MCG LTM with SN addition

In above scenarios, when L3 HO is triggered, source gNB-CU also sends UE Context Modification Request message containing RRC Reconfiguration to source gNB-DU, in case if LTM is triggered by source gNB-DU at this moment, source gNB-DU responds with the UE Context Modification Failure message including a cause indication of LTM command triggered. It can avoid triggering simultaneous LTM and L3 HO from the view of UE.

However, regarding to R19 SN initiated inter-CU SCG LTM, LTM is triggered in source SN-DU if S-SN is a split gNB. When L3 SN change is triggered, MN sends the RRC Reconfiguration message to UE after the completion of target PSCell resource preparation regardless SN change is initiated by MN or S-SN (refer to the message flows in below figures on SN change procedure-SN/MN initiated).

Due to different node finally sends L3 SN change command (RRC Reconfiguration message) and LTM cell switch command message to UE, it is possible for the UE to almost simultaneously receive LTM Cell Switch Command from source SN-DU and RRC Reconfiguration message from MN. In order to avoid inter-CU SCG LTM and L3 SN change collision, we propose to introduce one coordination mechanism to solve this issue.

Proposal 1: RAN3 to discuss one coordination mechanism to avoid inter-CU SCG LTM and L3 SN change triggered almost simultaneously.

## Issue 2: Inter-CU MCG LTM with SN addition

Regarding LTM with DC scenario, below agreements are reached in RAN2 and RAN3, wherein, inter-CU MCG LTM with SN addition is supported.

RAN2 Agreement: RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it. RAN3 Agreement: RAN3 agree the following scenarios to support LTM with NR-DC: 1. SN initiated inter-CU SCG LTM without MCG changes (high priority) 2. Inter-CU MCG LTM without SN release 3. Inter-CU MCG LTM with SN release 4. Inter-CU MCG LTM with SN addition

In order to better meet the throughput requirement, when handover request is triggered from source gNB to a candidate gNB during LTM preparation phase, the candidate gNB may trigger a SN addition procedure for this UE. Due to no limitation, each candidate gNB can add any candidate SN based on measurement report or possibly blind add any candidate SN if not measurement report. If different candidate gNB adds different candidate SN, when LTM execution, data forwarding is performed form source SN to target SN for the SN terminated bearer. It will increase packet delay from the view of UE. Therefore, if different candidate gNB are expected to add a same candidate SN, it will save radio resource of SN since it is possible same PSCell resource in same candidate SN can be allocated for this UE, and furthermore, data forwarding (including early or late data forwarding) is not needed for SN terminated bearer.

Observation 1: Regarding the scenario of inter-CU MCG LTM with SN addition, obvious benefits is existed if different candidate gNB add a same candidate SN, since data forwarding is not needed for SN terminated bearer and therefore reduce the packet delay.

In order to ensure the same candidate SN is addition by different candidate gNB(MN), current specification already supports the inter-MN handover without SN change as in the section 10.7 of TS37.340. However, this scenario is only applicable when UE’s initial state is in the Dual-Connection. If UE’s initial state is in the Single-Connection, i.e., only a standalone gNB serves the UE. During the LTM preparation phase, source gNB requests a candidate gNB to prepare the cell resource via Handover Request, and Candidate gNB(MN) may trigger a SN addition procedure for this UE as below figure. If Candidate gNB(MN) would like to take this Candidate SN as a reference Candidate SN, and expect the Other Candidate gNB(MN) can also add this Candidate SN, Candidate gNB(MN) can inform the UE context reference information at this Candidate SN to Source gNB via Handover Request Acknowledge message. After then, when Source gNB triggers the handover request to Other Candidate gNB(MN), carrying this UE context reference information at this Candidate SN, and Other Candidate gNB(MN) may decide to add this Candidate SN based on the UE context reference information at this Candidate SN.

Through above the analysis, we suggest RAN3 to agree below proposal.

Proposal 2: During LTM preparation in the scenario of inter-CU MCG LTM with SN addition, in order to add same candidate SN by other candidate gNB for setup DC, UE Context Reference information at the candidate SN can be carried via Handover Request Acknowledge message from a candidate gNB to source gNB. And source gNB can carry this information to other candidate gNBs during handover request procedure and expect candidate gNBs can setup a DC for this UE with the indicated candidate SN.

## Issue 3: FFS TA Information List IE in Cell Switch Notification message

Regarding Cell Switch Notification from source gNB-DU to candidate gNB-DU via source gNB-CU and candidate gNB-CU, we already agree to introduce a new XnAP UE-associated procedure, namely Cell Switch Notification message.

Following the current detail IEs in F1AP DU-CU CELL SWITCH NOTIFICATION in R18 intra-CU LTM, since TA values of candidate cells may be also forwarded from source gNB-DU to candidate gNB-DU for subsequent LTM use, we also propose to include TA Information List IE in the XnAP CELL SWITCH NOTIFICATION message for subsequent LTM use in new source gNB.

Proposal 3: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 4: Enhancement for the LTM candidate cells cancel via the procedure of LTM Configuration Update/Acknowledge

Regarding the LTM candidate cells cancel or release, based on previous agreements, source gNB or candidate gNB can initiate LTM cells cancel/release by Handover Cancel message or LTM Cancel message.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

However, after each inter-CU LTM Cell Switch, new source gNB may initiate to setup the UE associated Xn signalling connection with the candidate gNBs via LTM Configuration Update/Acknowledge. If some LTM candidate cells are released due to some reasons, this procedure cannot be initiated by source gNB or candidate gNB until the setup for the UE associated Xn signalling connection between new source gNB and candidate gNB. Therefore, if LTM candidate cells cancel/release information can be included in the LTM Configuration Update/Acknowledge message, the following Handover Cancel or LTM Cancel does not need be initiated so that can save signalling procedure.

In other words, it has a benefit to introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge if there are LTM candidate cells to be released during the setup for UE associated Xn signalling connection between new source gNB and candidate gNB.

Proposal 4: Introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge from new source gNB or other candidate gNB to trigger LTM candidate cells cancel.

# 3 Conclusion

This contribution firstly analyses the potential issue on inter-CU LTM and propose some enhancements for Xn interface on below aspects. Therefore, we invite RAN3 to agree below proposals for supporting the inter-CU LTM (or with DC) procedure.

## Issue 1: Co-existence between L3 SN change and inter-CU SCG LTM

Proposal 1: RAN3 to discuss one coordination mechanism to avoid inter-CU SCG LTM and L3 SN change triggered almost simultaneously.

## Issue 2: Inter-CU MCG LTM with SN addition

Observation 1: Regarding the scenario of inter-CU MCG LTM with SN addition, obvious benefits is existed if different candidate gNB add a same candidate SN, since data forwarding is not needed for SN terminated bearer and therefore reduce the packet delay.

Proposal 2: During LTM preparation in the scenario of inter-CU MCG LTM with SN addition, in order to add same candidate SN by other candidate gNB for setup DC, UE Context Reference information at the candidate SN can be carried via Handover Request Acknowledge message from a candidate gNB to source gNB. And source gNB can carry this information to other candidate gNBs during handover request procedure and expect candidate gNBs can setup a DC for this UE with the indicated candidate SN.

## Issue 3: FFS TA Information List IE in Cell Switch Notification message

Proposal 3: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 4: Enhancement for the LTM candidate cells cancel via the procedure of LTM Configuration Update/Acknowledge

Proposal 4: Introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge from new source gNB or other candidate gNB to trigger LTM candidate cells cancel.

# 4 TP to TS 38.423

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  | YES | reject |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| LTM Handover Information Response | O |  |  |  | YES | ignore |
| >SSB Information | M |  | 9.2.3.x10 |  | – |  |
| >LTM Information Response | O |  | 9.2.3.xx2 |  | – |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [10]. | – |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [10]. This IE applies for SUL carrier. | – |  |
| Early Sync Information Response | O |  | 9.2.3.xx4 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item |  | 1 .. <maxnoofTAList> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.x.x3	LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| LTM Information Request | O |  | 9.2.3.xx1 |  | YES | ignore |
| LTM Updates to Candidate Cell Information List | 0..1 |  |  |  | YES | ignore |
| >LTM Updates to Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>Request for CSI-RS Resource Configuration (FFS) | O |  | ENUMERATED (true, …) | Indicates the request to the candidate cell to provide CSI-RS resource configuration. | - |  |
| >>Early Sync Information Request | O |  | 9.2.3.xx3 | Indicates the request to the candidate cell for early synchronization information. | - |  |
| >>Early Sync Information | O |  | 9.2.3.xx6 | Indicates the consolidated early synchronization information to be used for subsequent LTM. | - |  |
| >>Early Sync Information for SUL | O |  | 9.2.3.xx6 | Indicates the consolidated early synchronization information for SUL to be used for subsequent LTM. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell identified by the Candidate Cell ID IE. | - |  |
| >>AS Security Information | O |  | 9.2.3.50 |  | - |  |
| UE Security Information for LTM | O |  | 9.2.3.x12 |  | YES | ignore |
| Data Forwarding Information | FFS |  |  |  |  |  |
| LTM Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofLTMCells> |  |  | YES | ignore |
| >Target Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.x.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| LTM Updates from Candidate Cell Information List | 0..1 |  |  |  | YES | ignore |
| >LTM Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>LTM Information Response | O |  | 9.2.3.xx2 |  | - |  |
| >>Early Sync Information Response | O |  | 9.2.3.xx4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| LTM Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofLTMCells> |  |  | YES | ignore |
| >Target Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 5 Reference

[1] R3-254007 (BL CR to 38.300) Support for Inter-CU LTM

[2] R3-254009	(BL CR to 38.423) Xn support for inter-CU LTM

[3] R3-254011	(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM

[4] R3-255659	(TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM