3GPP TSG-RAN WG3 Meeting #129	R3-255674

Bengaluru, India, 25th ~29th Aug, 2025

Agenda Item:	23.2

Source:	Huawei, Qualcomm Incorporated, EBU

Title:	Consideration on LTE-based 5G Broadcast Phase 2

Document for: Discussion

# 1	Introduction

In RAN3#128 meeting, there were some agreements about the introduction of time and frequency interleaving for LTE-based 5G Terrestrial Broadcast:

MCE decides the TFI and the configuration of TFI, and indicates the configuration of TFI to the eNBs. Regarding parameters configuration for TFI, per PMCH configuration is adopted in RAN1.

In this contribution, we will further analyse the RAN3 impacts to support time and frequency interleaves for MBMS service based on the RAN1 final conclusions.

# 2	Discussion

  - M2AP impact for Time Interleaving

In the last meeting, RAN1 has achieved the following agreements about time interleaving:

Agreement Add the following MSI periodicities to the previous RAN1 agreement {rf28, rf108, rf212}. Agreement The residual space created by the mismatch between (M,N) and the MSI periodicity may be filled with the last MTCH service with different TFI configuration (M,N), e.g., via an additional (addressing the residual space) optional configuration via pmch-Config. - In the space before mismatch, all MTCHs within the MSI period have the same (M,N) obtained via the configuration of M,N in pmch-Config. Agreement The soft buffer size for one time-interleaved TB is calculated as NIR=βNsoftM, where Nsoft is the soft buffer size configured according to a reference UE category, indicated from among { DL Cat4, DL Cat5, … DL Cat26}, M is the number of TBs for time-interleaving and β is configured from: {1/32, 1/5, 1/3, 3/8, 5/12, 1/2, 5/8, 5/6, 2/3, 1}. Agreement For clarification to RAN2: When time-interleaving is enabled, values of N for configuration/indication do not include N=1. When N is not configured, time-interleaving is disabled i.e. N=1.

And RAN1 has provide the parameters about time interleaving, as following shown:

Based on the parameter list from RAN1, the time interleaving parameters are per PMCH configuration. Thus, the MCE should indicate the time interleaving configuration in the PMCH Configuration IE in the MBMS SCHEDULING INFORMATION message.

Proposal 1: Introduce a Time Interleaving Parameters IE in the PMCH Configuration IE of the M2AP: MBMS SCHEDULING INFORMATION message.

Besides the M and N, RAN1 has also agreed to indicate the Scaling Factor Beta and the Reference UE Category for eNB to determine the soft buffer size, and also indicate the additional value M and N for last MTCH using residual resource.

Besides these parameters, it is also noticed that RAN1 have already agreed to use cyclic shift in R1-2505059, and there is a parameter called “α”, which is provided by high layer and can be set to alphaOne and alphaOther.

From RAN3 point of view, the new Time Interleaving Parameters IE should include all these parameters.

Proposal 2: The Time Interleaving Parameters IE includes: M, N, Scaling Factor Beta, Reference UE Category, M_LastMTCH, N_LastMTCH and Cyclic Shift Alpha.

And also, to deal the with the issue of mismatch between MSI periodicity and the interleaving pattern, new MSI periodicities should be indicated from MCE to eNB, i.e., introduce a new MCH Scheduling Period Extended 3 IE (including value of rf7, rf14, rf28, rf53, rf56, rf112, rf108, rf212, rf424) in the PMCH Configuration IE in the MBMS SCHEDULING INFORMATION message.

Proposal 3: Introduce the new MSI periodicities as MCH Scheduling Period Extended 3 IE in the PMCH Configuration IE of the M2AP: MBMS SCHEDULING INFORMATION message.

  - M2AP impact for frequency Interleaving

RAN1 has provided the parameters about frequency interleaving, as following shown:

Based on this, the frequency interleaving should also be configured in per PMCH config level. Thus, RAN3 can indicate a Frequency Interleaving Indicator IE in the PMCH Configuration IE of the MBMS SCHEDULING INFORMATION message.

Proposal 4: Introduce a Frequency Interleaving Indicator IE (whether the frequency interleaving is enabled or not) in the PMCH Configuration IE in the M2AP: MBMS SCHEDULING INFORMATION message.

# 3	Conclusion

Based on the above discussion we have the following proposals:

M2AP impacts for time interleaving

Proposal 1: Introduce a Time Interleaving Parameters IE in the PMCH Configuration IE of the M2AP: MBMS SCHEDULING INFORMATION message.

Proposal 2: The Time Interleaving Parameters IE includes: M, N, Scaling Factor Beta, Reference UE Category, M_LastMTCH, N_LastMTCH and Cyclic Shift Alpha.

Proposal 2: Introduce the new MSI periodicities as MCH Scheduling Period Extended 3 IE in the PMCH Configuration IE of the M2AP: MBMS SCHEDULING INFORMATION message.

M2AP impacts for frequency interleaving

Proposal 4: Introduce a Frequency Interleaving Indicator IE (whether the frequency interleaving is enabled or not) in the PMCH Configuration IE in the M2AP: MBMS SCHEDULING INFORMATION message.

The corresponding M2AP CR is provided in [2].

# 4	Reference

[1] RP-250794, “Revised WID on LTE-based 5G Broadcast Phase 2”, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 2025

[2] R3-255675 Introduction of LTE-based 5G Broadcast Phase 2, CR to TS 36.443, Rel-19, Huawei, Qualcomm Incorporated, EBU

# Annex. RAN1 agreements

  - RAN1#120 meeting

Agreement

For TBS scaling by N, down-select one of the two options below:

- Option 1: Calculate the total number of PRBs NRBtotal as NRBtotal=NRBDL×N, wherein, NRBDL is system bandwidth;

  - If NRBtotal≤110, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBtotal, obtain the corresponding TBS;

  - Else, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBDL, obtain the corresponding TBS’; and calculate the final TBS as TBS=round{TBS'×N}

  - The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

- Option 2: Obtain the initial TBS (TBS') as per current specifications, and calculate the final TBS as TBS=round{TBS'×N}

  - The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

Agreement

At least values of N={1,4,8} are supported, where N=1 indicates time interleaving is disabled.

  - FFS: other values

Agreement

LBRM is applied for MCH (excluding MCCH and MSI) with time interleaving.

  - FFS spec impact

Agreement

For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is derived based on DL-SCH as per TS 36.306 set by UE category.

  - FFS when the UE category has multiple maximum TBS

Agreement

The maximum number of transport block bits received within a TTI defined by the UE category should include the number of bits for unicast and broadcast, where the number of bits for broadcast is the one after being scaled up by N.

Agreement

The basic transmission pattern of PMCH with time interleaving consists of a set of (M x N) consecutive MBSFN subframes, excluding MCCH and MSI, where two transmissions of the same TB are separated by (M-1) MBSFN subframes, excluding MCCH and MSI.

Agreement

Specify time interleaving per MBMS session/MTCH without supporting interleaving across different MBMS sessions/MTCH.

Agreement

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per MBSFN area/MCCH configuration, e.g., via mcch-Config

- Option 2: Per PMCH configuration, e.g., via pmch-Config

- Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

Agreement

RAN1 to further study the potential issue of mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes.

Agreement

The equation of k0 for time interleaving is modified to avoid puncturing of systematic bits.

- Further study the exact equation for k0 for the n-th subframe of a transmission belonging to a same TB k0n, n∈{0, 1, 2, …, N-1}), including

  - Option 1: for each CB of the TB, k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe.

  - Option 2: For each CB of the TB, follow the principle of NR TBoMS to determine k0n

  - Option 1 and Option 2 assumes all CBs of the TB have the same RV index for all CBs of the TB, indexed by k0n

- FFS whether different CBs of the TB mapped to each subframe may have different RV indices (e.g., indexed by different k0n per CB)

Agreement

The modulation symbol vector (prior to frequency interleaving) y=y0,y1,…,yZ-1 is interleaved, according to the principles of subclause 5.1.4.1.1 of TS 36.212 as follows

- Set the number of columns of the frequency interleaver, C=X, set the number of rows R to be the minimum integer satisfying Z≤R×C, where Z is the number of data REs in one OFDM symbol

  - FFS: How to determine X

- Append y with <NULL> elements, as required, to obtain yapp

- Write the elements of yapp into the R×C-matrix column-wise, with yapp(0) in column 0 of row 0.

- FFS whether the rows and elements in each row will be permuted, with details to be further studied, e.g., intra-row cyclic shifts, inter-row permutations

- Obtain the vector yΠ by reading the elements of the above R×C-matrix row-wise, excluding the <NULL>s.

- The vector yΠ shall then be mapped to resource elements, according to clause 6.3.5 of TS 36.211.

FFS: whether to apply a cyclic shift offset, and the details of the cyclic shift offset for the concatenated frequency interleaver outputs of all OFDM symbols before the mapping to REs of a subframe, where the cyclic shift offset is dependent on the RV index.

Agreement

The granularity of the frequency interleaver is 1 RE.

Agreement

For TBS scaling by N, the following is clarified for the rounding operation in option 1 and option 2 in previous agreement:

- In case two TBSs are at the same distance of TBS'×N, the larger TBS is chosen.

Agreement

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per MBSFN area/MCCH configuration, e.g., via mcch-Config

- Option 2: Per PMCH configuration, e.g., via pmch-Config

- Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

Agreement

Values of M = 16 and M = 8 are supported.

- FFS other values

- Note: some UE categories and depending on factors such as (value of N, bandwidth, MCS) may not be able to receive an interleaved MTCH with M=16.

Agreement

The previous agreement is modified as follows:

The modulation symbol vector (prior to frequency interleaving) y=y0,y1,…,yZ-1 is interleaved, according to the principles of subclause 5.1.4.1.1 of TS 36.212 as follows

- Set the number of columns of the frequency interleaver, C=X, set the number of rows R to be the minimum integer satisfying Z≤R×C, where Z is the number of data REs in one OFDM symbol

  - FFS: How to determine X

    - Option 1: X is equal to the number of codeblocks that are mapped to the OFDM symbol

    - Option 2: X=32

    - Other options are not precluded.

- Append y with <NULL> elements, as required, to obtain yapp

- Write the elements of yapp into the R×C-matrix column-wise, with yapp(0) in column 0 of row 0.

- FFS whether the rows and elements in each row will be permuted, with details to be further studied, e.g., intra-row cyclic shifts, inter-row permutations

- Obtain the vector yΠ by reading the elements of the above R×C-matrix row-wise, excluding the <NULL>s.

- The vector yΠ shall then be mapped to resource elements, according to clause 6.3.5 of TS 36.211.

FFS: whether to apply a cyclic shift offset, and the details of the cyclic shift offset for the concatenated frequency interleaver outputs of all

OFDM symbols before the mapping to REs of a subframe, where the cyclic shift offset is dependent on the RV index.

  - RAN1#120-bis meeting

Agreement

Support at least the following values of time interleaver depth:

- Values M={4,8,16,32} and N={1,2,4,8,16} are supported.

  - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI.

- FFS: larger values, including M=64, N={32,64}.

Agreement

In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

  - May include the case of scheduling different M/N within the same MSI periodicity

  - FFS the values of the new periodicities.

- RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity

  - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity

  - Option 2: Drop the time-interleaved TBs which could not be successfully decoded.

    - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour)

  - Other Options not precluded

Agreement

The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH,

- Working assumption:

  - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.

Agreement

The following steps are applied to the rows of the frequency interleaver:

- Perform inter-row permutation with a reorder of the row indexes Ri', which is calculated as:

Ri'=R'i-1+d########################,          R'i-1≤R-d-1 #i×dR#,                 else.

With, 1≤i≤R-1, R'0=0, and d=R/R/RC-C+1.

- Perform a cyclic shift to the elements in the j-th row with an offset S(j) defined as:

Sj=modj+12×-1j,C, j=0,1,2,…,R-1

Agreement

LBRM parameters (e.g. NIR) are consistent between the broadcast network and the (multiple) UEs that are receiving the broadcast transmission.

- These multiple UEs may belong to different UE categories, thus having different values of Nsoft, NIR.

- FFS: how the consistent NIR and NCB is calculated.

Agreement

The agreement in the previous meeting is updated as follows:

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per PMCH configuration, e.g., via pmch-Config

- Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

  - Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching.

  - Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g. that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

Agreement

Obtain the initial TBS (𝑇𝐵𝑆′) as per current specifications, and calculate the final TBS as 𝑇𝐵𝑆=𝑟𝑜𝑢𝑛𝑑{𝑇𝐵𝑆′×𝑁}.

- The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

  - RAN1#121 meeting

Agreement

For each CB of the TB, k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe, where NL=1, and, rvn values are mapped sequentially to subframes, i.e., rvn=n.

k0=2RsubblockTC+rvidxEmin

Agreement

Time interleaving parameters are indicated per PMCH configuration, e.g., via a configuration of M,N in pmch-Config. Specify the following set of new MSI periodicities: {rf7, rf14, rf53, rf56, rf112, rf424}.

- Additionally down-select among {rf27 or rf28}, {rf106 or rf108}, {rf211 or rf212}.

- FFS actions on the residual mismatch after applying the new MSI periodicities in conjunction with the configuration of M,N

  - Option 1: Fill the residual space with a service with different TFI configuration (M,N), e.g., via an additional (addressing the residual space) configuration via pmch-Config

Agreement

The soft buffer size for one time-interleaved TB is down-selected from

- Option 1:  NIR=βNsoftM, where:

  - Option 1a: Nsoft is the soft buffer size pertaining to a reference UE category, M is the number of TBs for time-interleaving and β is a scaling factor. FFS: exact value/values/configuration of β.

  - Option 1b: Nsoft is configured in PMCH-Config from {1827072, 3654144, 5481216, 7308288, 9744384, 12789504, 14616576, 17052672, 19488768, 24360960, 29233152, 34105344, 36541440, 38977536, 42631680, 47431680, 303562752}, 1β is configured in PMCH-Config from: {1, 3/2, 6/5, 8/5, 2, 12/5, 8/3, 3, 5, 32};  and M is the number of TBs for time-interleaving.

- Option 2 : The soft buffer size for the code block is denoted as Ncb=min(TBSLBRMC∙RLBRM,Kw), where C and Kw are same as defined in TS 36.212 clause 5.1. TBSLBRM is calculated based on, e.g., nPRB,LBRM, ITBS, N. RLBRM=2/3.

  - FFS: Details of determining TBSLBRM, nPRB,LBRM, ITBS (e.g., based on a notion of maximum coding rate) may need to be considered, to provide the flexibilities afforded by the LTE specifications, keeping in mind the LTE UE categories.

- FFS: impact to soft buffer size for UE receiving unicast considering the case where UE prioritizes receiving broadcast.

Agreement

For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is down-selected from the following:

- Option 1: the largest TBS among the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE category.

- Option 2: the smallest TBS among the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE category.

- Option 3: signalled by the UE from the multiple maximum TBS of a DL-SCH as per TS 36.306 set by UE, e.g. using a new UE capability

Agreement

Add the following MSI periodicities to the previous RAN1 agreement {rf28, rf108, rf212}.

Agreement

The residual space created by the mismatch between (M,N) and the MSI periodicity may be filled with the last MTCH service with different TFI configuration (M,N), e.g., via an additional (addressing the residual space) optional configuration via pmch-Config.

- In the space before mismatch, all MTCHs within the MSI period have the same (M,N) obtained via the configuration of M,N in pmch-Config.

Agreement

The soft buffer size for one time-interleaved TB is down-selected from

- Option 1:  NIR=βNsoftM, where:

  - Option 1a: Nsoft is the soft buffer size pertaining to a reference UE category, M is the number of TBs for time-interleaving and β is a scaling factor configured from network among values in the interval [0 1]. FFS: the exact values in the interval.

  - Option 1b: Nsoft is configured in PMCH-Config from {1827072, 3654144, 5481216, 7308288, 9744384, 12789504, 14616576, 17052672, 19488768, 24360960, 29233152, 34105344, 36541440, 38977536, 42631680, 47431680, 303562752}, 1β is configured in PMCH-Config from: {1, 3/2, 6/5, 8/5, 2, 12/5, 8/3, 3, 5, 32};  and M is the number of TBs for time-interleaving.

- FFS: impact to soft buffer size for UE receiving unicast considering the case where UE prioritizes receiving broadcast.

Agreement

The number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

- FFS: spec impact, if any

Agreement

Cyclically shift the bit sequence b0, b1, …bMbit-1 in Section 6.3.1 of TS 36.211 for the ith subframe of the time-interleaved TB by Xi bits, where

- i∈{0,1,…,N-1},

- N denotes the number of subframes to which the time-interleaved TB is mapped

- Xi  is

  - Alt 1: ∑#CBs-1r=#CBs-SiEr, with Si=(i×α) mod #CBs

    - Er is the number of bits in the rth codeblock within a subframe (as defined in TS 36.212)

    - α=1

  - Alt 2:  ∑#CBs-1r=#CBs-SiEr, with Si=(i ×α) mod #CBs

    - Nsymb denotes the number of OFDM symbols within a subframe

    - #CBs denotes the number of CBs in the time-interleaved (scaled) TB

    - Er is the number of bits in the rth codeblock within a subframe (as defined in TS 36.212)

    - α= #CBsN×Nsymb

A new RRC parameter configures between α=1 and α= #CBsN×Nsymb as above

    - If this new RRC parameter is not configured, the cyclic shift described in this agreement is not enabled.

    - A new optional UE capability is introduced to indicate support of this cyclic shift.

    - FFS: Extension of the bit-field corresponding to this RRC parameter by one additional bit, based on NOTE 1 below

  - NOTE 1: The following alternative for Xi may be considered, exclusively based on BLER-vs-SNR performance comparison (with respect to Alt 1 / Alt 2)

    - Xi=Ai×floor(NSC ×QmN), with Ai∈{0,…N-1}

    - Where NSC  is the number of subcarriers available in 1 OFDM symbol for PMCH, e.g.  NSC =Mbit/(QmNsym)

Agreement

The soft buffer size for one time-interleaved TB is calculated as NIR=βNsoftM, where Nsoft is the soft buffer size configured according to a reference UE category, indicated from among { DL Cat4, DL Cat5, … DL Cat26}, M is the number of TBs for time-interleaving and β is configured from: {1/32, 1/5, 1/3, 3/8, 5/12, 1/2, 5/8, 5/6, 2/3, 1}.

Agreement

Rel-19 IE in MBMS Interest Indication message carries following parameters utilized for receiving Rel-19 LTE-based 5G Broadcast service(s):

- Frequency

- New values of sub-carrier spacing = {2.5 kHz, 1.25 kHz}

- New values for bandwidth = {n30, n35, n40}

- A new UE baseband resource Scaling factor (e.g., utilized soft buffer at the UE) used for receiving the time interleaved broadcast

Agreement

In case the time interleaved PMCH transmission (as configured) exceeds a UE’s capability(/es) in terms of soft buffer size or maximum TBS supported in a TTI, the UE is not expected to receive the corresponding time interleaved PMCH transmission.

Agreement

Update of the FFS of a previous agreement:

Agreement

The number of subframes used for time-interleaving derived from ‘stop MTCH (x+1)’ is expected to be an integer multiple of MxN.

Spec impact is up to RAN2.

Agreement

RAN1 asks RAN2 whether/how to address the HARQ handling in the MAC specification for the Rel-19 time-interleaved PMCH transmission with M TBs and N RVs.

Agreement

For clarification to RAN2:

When time-interleaving is enabled, values of N for configuration/indication do not include N=1. When N is not configured, time-interleaving is disabled i.e. N=1.

Agreement

LS on RAN2 aspects on LTE-based 5G Broadcast Phase 2 is agreed. Final LS in R1-2504922.

Agreement

Confirm the working assumption in the following agreement:

Agreement

The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH,

- Working assumption:

  - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.